| Document Title: SOP-Cleaning and Sanitation-Chemical Spill | Document Control No.: [Enter Document Control No.] | Rev. No: [Enter Revision No.] |
| --- | --- | --- |
| Department: [Enter Department] | Supersedes: Original | Effective Date: [Enter Date] |
| Prepared by: [Affix Name, Signature, Position Title] | Reviewed by: [Affix Name, Signature, Position Title] | Approved by: [Affix Name, Signature, Position Title] |
| Date: [Enter Date] | Date: [Enter Date] | Date: [Enter Date] |

- Purpose:

  - To manage a chemical spill to minimize the potential for injury and damage to the environment.

- Frequency:

  - For every chemical spill

  - [ADD FREQUENCY HERE]

- Equipment:

  - Chemically resistant gloves

  - Safety Glasses

  - SDS

  - Shoe Cover

  - Chemical resistant suit (or Apron)

  - Spill Kit

  - [ENTER LIST OF EQUIPMENT HERE]

- Consumable:

  - None

  - [ENTER LIST OF CONSUMABLES HERE]

- Responsibility:

  - Supervisors, department heads, and team leaders

    - Provide personal protective equipment required during emergency cleanup

    - Responsible for developing spill response plans

    - Ensure all personnel receive appropriate chemical safety training and familiarize themselves with the spill response plan.

    - Implement measures to prevent potential spills of hazardous chemicals.

  - All Employees

    - Review of Safety Data Sheets (SDS) for all chemicals used in the facility

    - Take all necessary precautions to minimize the likelihood of spills occurring.

    - Follow established spill cleanup procedures

  - [ADD OR CUSTOMIZE RESPONSIBLE PERSON HERE]

- Procedure/Instructions:

  - Incidental Spill Cleanup Procedures

    - Notify other people in the area that a spill has occurred. Prevent others from coming in contact with the spill (i.e., walking through the spilled chemical). The priority is always to protect yourself and others.

    - Before beginning cleanup, put on the Proper Personal Protective Equipment (PPE) such as goggles, gloves, etc. Do not unnecessarily expose yourself to the chemical.

    - Stop the source of the spill if possible and if safe.

    - Try to prevent spilled chemicals from entering waterways by building a dike around access points (sink, cup sinks, floor drains inside, and storm drains outside) with absorbent material if you can safely do so.

    - Use the appropriate absorbent material for liquid spills (detailed in the following section).

    - Slowly add absorbent material on and around the spill and allow the chemical to absorb. Apply enough absorbent to cover the spilled liquid completely.

    - Sweep up the absorbed spill from the outside towards the middle.

    - Scoop up and deposit in a leak-proof container.

    - Label the container and dispose of it for absorbed hazardous chemicals through the hazardous waste management program.

    - If possible, mark the spill area on the floor with chalk.

    - Wash the contaminated surface with soapy water. If the spilled chemical is highly toxic, collect the rinse water for proper disposal.

    - Report the spill to your supervisor.

    - Restock any spill cleanup supplies that you may have used from any spill kits.

  - Major Spill Cleanup Procedures

    - When a spill occurs that you are not capable of handling:

      - Alert people in the immediate area of the spill and evacuate the room.

      - If an explosion hazard is present, do not unplug or turn electrical equipment on or off – doing so can result in a spark and ignition source.

      - Confine the hazard by closing doors as you leave the room.

      - Use eyewash or safety showers to rinse spilled chemicals off people or yourself.

      - Evacuate any nearby rooms that may be affected. If the hazard affects the entire building, then evacuate the entire building by pulling the fire alarm.

      - Notify Police by calling 911. Always call from a safe location.

  - For organic solvents

    - Ensure there are no ignition sources in the area.

    - If you feel there is a risk of fire or explosion, evacuate the lab and treat it as a major spill.

    - If the spill can be cleaned up safely, proceed with the following.

    - If there are drains in the area, use a broom, sock, or other material to prevent the hazardous material from reaching the drain.

    - Gently pour solvent vapor suppressant (activated carbon) over the spill, working from the outer edge to the middle.

    - Using the absorbent pads from the spill kit, carefully wipe up the spilled liquid, again working from the outside in.

    - Place all waste materials in a plastic bag. Once the spill has been fully cleaned, temporarily place the waste bag in the fume hood.

    - Label the bag as hazardous waste

    - Remove PPE and thoroughly wash hands.

    - Use soap and water to wash the affected area and remove any minor residues that may be left.

    - Replenish your spill kit supplies

  - For acids

    - If there are drains in the area, use a broom, sock, or other material to prevent the hazardous material from reaching the drain.

    - Gently pour acid-neutralizing agents (e.g., sodium bicarbonate, calcium carbonate, etc.) over the spill, working from the outside in.

    - Allow several minutes for acid to mix with neutralizer, then test a representative area with pH paper.

    - When the spill has been neutralized, use the available absorbent pads or paper towel to wipe up the spilled material.

    - Place all waste into a plastic bag and label it as hazardous. Place in a suitable location.

    - Remove PPE and thoroughly wash hands.

    - Use soap and water to wash the affected area and remove any minor residues that may be left.

    - Replenish your spill kit supplies

  - For bases

    - If there are drains in the area, use a broom, sock, or other material to prevent the hazardous material from reaching the drain.

    - Gently pour base neutralizing agent (e.g., citric acid, sodium bisulfate), etc.) over the spill, working outside.

    - Allow several minutes for the base to mix with the neutralizer, then test a representative area with pH paper.

    - When the spill has been neutralized, use the available absorbent pads or paper towel to wipe up the spilled material.

    - Place used materials into plastic bags and label them as hazardous waste. Label the bag as hazardous waste and place it in a suitable location.

    - Remove PPE and thoroughly wash hands.

    - Use soap and water to wash the affected area and remove any minor residues that may be left.

    - Replenish your spill kit supplies

  - For dry chemicals

    - For highly corrosive, toxic, or reactive materials, treat as a major spill. With specialized HEPA vacuums, a hazardous materials team may be needed in these circumstances.

    - The powder or crystals can be cleaned up using the scoop and dustpan for materials of limited hazard.

    - Place waste material in a suitable container or bag.

    - Remove PPE and thoroughly wash hands.

    - Use soap and water to wash the affected area and remove any minor residues that may be left.

    - Replenish your spill kit supplies

  - For Mercury

    - DO NOT attempt to clean up spills involving more than a few milliliters. Mercury is highly toxic and releases vapors that can accumulate to toxic concentrations.

    - Do not dispose of Mercury or mercury-contaminated spill debris in the regular trash or down the drain.

    - There is no absorbent material available. Physical removal processes are best for removing and collecting Mercury.

    - If you need help collecting Mercury from a spill, contact EHS spill responders by calling 911. Note: While powdered sulfur will help reduce mercury vapors, the sulfur greatly complicates the spill cleanup.

    - Treat spills larger than a few milliliters as a major spills.

    - Place waste material in a suitable container

    - Replenish your spill kit supplies

  - For hydrofluoric acid

    - Hydrofluoric acid (HF) differs from other acids because it readily penetrates the skin and dissociates into fluoride ions, destroying deep tissue layers, including bone. Pain associated with skin exposure to HF may not occur for 1-24 hours. Unless you can rapidly neutralize the HF and bind the fluoride ions, tissue destruction may continue for days, resulting in limb loss or death.

    - Use Calcium carbonate or Calcium bicarbonate to bind the fluoride ion tightly.

    - If there is any skin contact, flush the area for one minute and apply calcium gluconate gel.

    - If eye contact occurs, flush the eye for 30 minutes. Have someone call ahead to the hospital emergency ward and inform them of the incident to prepare an antidote solution.

    - Seek professional medical help following any exposure to hydrofluoric acid. Medical intervention may involve debridement of wound, injections of calcium gluconate, or observation/treatment of systemic effects.

    - If the spill is small and there is a low respiratory hazard (i.e., you have spilled a small amount inside the fume hood), you may clean the spill.

    - Carefully cover the spill with an HF-neutralizing agent (calcium carbonate, magnesium sulfate, or commercial HF spill neutralizer), working from the outside edge of the spill to the interior.

    - Use a pH strip to verify the spill has been neutralized.

    - Use absorbent material, scraper and scoop to collect neutralized material, and place all waste into a plastic bag for disposal in the regular waste stream.

    - Remove PPE and thoroughly wash hands.

    - Don fresh gloves and wash the spill site with a sodium bicarbonate solution.

    - Replenish your spill kit supplies.

  - Chemical Spills on Body

    - Wash thoroughly (15 minutes) with water using the nearest deluge shower or hand/spray unit.

    - Remove any overlying clothing that may retain the chemical and prevent thorough skin washing.

    - Depending on the chemical, additional medical treatment may be required. Consult the Safety Data Sheet and/or specific laboratory procedures.

    - Seek medical attention; dial 911.

  - Chemical in Eye

    - Immediately flush eyes with copious amounts of water for at least 15 minutes, removing contact lenses.

    - Seek medical attention immediately. Go to the nearest hospital emergency department.

    - If possible, obtain the Material Safety Data Sheet and provide it to the treating physician.

  - [ADD OR CUSTOMIZE PROCEDURES/ INSTRUCTIONS HERE]

- Corrective Action

  - In the event of a chemical spill, the individual(s) who caused the spill is responsible for prompt and proper cleanup.

  - Immediately alert area occupants and supervisor, and evacuate the area if necessary.

  - Conduct annual refresher training for supervisors and employees to develop the needed skills and knowledge to perform their work in a safe and healthful manner

  - [ADD OR CUSTOMIZE CORRECTIVE ACTIONS HERE]

- Related Forms:

  - Chemical Inventory Log

  - [ADD OR CUSTOMIZE RELATED FORMS HERE]

- History/Changes:

| Revision No.: | Revision Date: | Description of Change: | Originator / Author Name: | Title / Department: |
| --- | --- | --- | --- | --- |
| 0 | YYYYMMDD | Original | ENTER AUTHOR NAME | ENTER TITLE/ENTER DEPARTMENT |

*Should you need assistance in customizing this document please email services@consultareinc.com