Ventilation – Abrasive Blasting Operations Training Material and Comprehension Quiz

- Policy

  - Romero’s Food Products Inc. ensures proper operating ventilation standards, including abrasive blasting promoting safe working conditions, and proper environmental control.

- Procedure

  - Types of Abrasives

    - Synthetic or natural mineral grains

      - Crystalline silica

      - Garnet

    - Slag abrasive

      - Copper

      - Nickel

      - Mixed metal

    - Metallic shot or grit

      - Steel

      - Chilled cast iron

    - Organic

      - Ground corncobs

      - Ground walnut shells

    - Alternative abrasives

      - Sodium bicarbonate

      - Dry ice

      - Plastic bead media

  - Hazards

    - Dust Hazard

      - Consider the composition and toxicity of dust from sources in evaluating potential health hazards.

      - Keep the concentration of respirable dust or fume in the breathing zone of the abrasive-blasting operator or any other worker below the levels specified in 29 CFR 1910.1000.

        - An employee’s exposure to any air contaminants shall not exceed the exposure limit in the particular substance stated in Tables Z-1, Z-2, and Z-3.

          - Table Z-1: Substances such as asbestos, silicon, xylene, and zinc oxide;

          - Table Z-2: Substances such as benzene, cadmium dust, mercury, and toluene;

          - Table Z-3: Mineral dust such as quartz, talc, graphite, and coal dust.

      - Use combustible organic abrasives only in automatic systems.

        - Conform to American National Standard Installation of Blower and Exhaust Systems for Dust, Stock, and Vapor Removal or Conveying requirements, Z33.1-1961 (NFPA 91-1961), specified in 29 CFR 1910.6, and subpart S when using flammable or explosive dust mixtures.

        - Bond and ground blast nozzles to prevent the buildup of static charges.

        - Construct abrasive blasting enclosures, ducts, and dust collectors with loose panels or explosion venting areas.

    - Noise Hazard

      - High noise levels can cause substantial hearing loss; wearing ear protection is a must.

      - A hearing conservation program is administered as required by the 29 CFR 1910.95 Occupational Noise Exposure Standard.

        - Develop and implement a monitoring program if any employee's exposure may equal or exceed an 8-hour time-weighted average of 85 decibels.

        - All continuous, intermittent, and impulsive sound levels from 80 decibels to 130 decibels are integrated into the noise measurements.

        - Monitoring is repeated whenever a change in production, process, equipment, or controls increases noise exposures.

      - Measure noise exposure through a sound level meter and the dosimeter:

        - Sound level meter - a device that measures the intensity of sound at a given moment;

        - Dosimeter - a sound level meter that stores sound level measurements and integrates the measurements over time, providing an average noise exposure reading for a given period. Refer to 29 CFR 1910.95 Appendix G, Monitoring Noise Levels.

  - Blast-Cleaning Exposures

    - Ensure exhaust ventilation for blast-cleaning enclosures by maintaining continuous inward airflow in all openings during blasting operations.

      - Baffle all air inlets and access openings.

      - Ensure a sufficient exhaust rate to provide prompt clearance of dust-laden air.

      - Turn off the blast and run the exhaust system before opening the enclosures.

      - Use safety glasses protected by observation windows screening when using deep-cutting abrasives.

      - Install multiple sets of stilt abrasive-resistant baffles at openings where dust might escape.

        - Doors are flanged and tight when closed.

        - Doors on blast-cleaning rooms are operable from both inside and outside.

  - Exhaust Ventilation System

    - Conform to the American National Standard Fundamentals Governing the Design and Operation of Local Exhaust Systems, Z9.2-1960, and ANSI Z33.1-1961, in 1910.6 for the construction, and installation, inspection, and maintenance of all exhaust systems.

      - Repair dust leaks as soon as possible.

      - Check static pressure drop at exhaust ducts upon installation and periodically thereafter to ensure satisfactory operation.

    - Provide an abrasive separator to remove fines from spent abrasive.

    - Discharge air exhausted from blast-cleaning via dust-collecting equipment which can be emptied and removed without contaminating working areas.

  - Personal Protective Equipment

    - Use only respirators approved by the National Institute for Occupational Safety and Health (NIOSH) under 42 CFR 84.

      - Air Purifying Respirators

      - Powered Air Purifying Respirators

      - Self-Contained Breathing Apparatus (SCBA)

      - Gas Masks

      - Particulate Filter Respirators

      - Chemical Cartridge Respirators

      - Closed-Circuit Escape Respirators

    - Use Type CE NIOSH-certified blasting airline respirator with positive pressure blasting helmet.

    - Provide hearing protection

    - Use leather gloves protecting full forearm and aprons (or coveralls)

    - Use safety foot wares to comply with the requirements specified by 29 CFR 1910.136 (b) (1).

    - Use eye and face protection which conforms with 29 CFR 1910.133.

  - Air Supply and Air Compressors

    - Meet the requirements for supplied-air quality and use specified in 29 CFR 1910.134(i) for air for abrasive-blasting respirators.

      - Oxygen content (v/v) of 19.5-23.5%;

      - Hydrocarbon (condensed) content of 5 milligrams per cubic meter of air or less;

      - Carbon monoxide (CO) content of 10 ppm or less;

      - Carbon dioxide content of 1,000 ppm or less; and

      - Lack of noticeable odor.

  - Operational Procedures and General Safety

    - Keep aisles and walkways clear of dust, dust spills, steel shots, or similar abrasives.

- Responsibility

  - Management

    - Protect employees from hazards associated with their work environment;

    - Educate employees on safe levels of dust and noise and the effects of dust and noise exposures on their health;

    - Provide personal protective equipment at no cost to the employee;

    - Provide adequate and operational ventilation systems; and

    - Follow the suitable design of the location and equipment and construct based on the regulatory standards.

  - Supervisors

    - Assess the presence of hazards (dust and/or noise) within their work environment;

    - Use engineering or administrative controls to reduce excessive exposures to dust and noise hazards;

    - Ensure personal protective equipment is worn by all personnel;

    - Routinely inspect ventilation systems and ensure it is installed and fully operational;

    - Ensure employees perform proper housekeeping before, during, and after operations;

    - Ensure employees are trained in the use of ventilation systems and the signs of system failure; and

    - Repeatedly train employees who are exposed to continuous noise above 85 dBA.

  - Employees

    - Wear proper personal protective equipment.

    - Immediately report incidents of dust leaks to the supervisor, if any.

    - Always remove accumulated dust in the workplace.

    - Undergo training in case a system failure occurs.

- Corrective Action

  - In the event that a system failure occurs stop the operations and perform preventive maintenance. Do not resume operations until the repair is completed.

  - Implement engineering or administrative controls to reduce excessive exposures to dust and noise hazards.

  - Re-train personnel when deficiencies in the employee’s understanding of the program are detected and reflected in their performance.

- Review - Quality

  - Periodical assessment of dust and/or noise hazards;

  - Complete and efficient PPEs are provided;

  - Routinary inspection of ventilation systems;

  - Training for current and new employees is performed;

  - Engineering or administrative controls to reduce excessive exposures to dust and noise hazards are implemented; and

  - Review annually and update this program whenever operations change occurs and apply to the facility.

- History

| Revision No.: | Revision Date: | Description of Change: | Originator / Author Name: | Title / Department: |
| --- | --- | --- | --- | --- |
| 0 | 20220903 | Original | Arnel Ryan | PCQI / Compliance |
| 1 | 20241213 | - Aligned with the Program - Changed title part from Lesson to Training Material and Comprehension Quiz - Changed email from etrr@consultareinc.com to rfp@consultareinc.com - Added title to Comprehension Quiz - Added Answer Key | Arnel Ryan | PCQI / Compliance |

Requirements:

- The trainee has read or received a verbal translation of all or part of the policy, procedure, method, and or SOP for which they are being trained.

- The trainee has demonstrated the task they are to perform or the procedure for which they are being trained as required.

- The trainee has demonstrated the ability to perform the task with acceptable proficiency and with minimal supervision as required.

| TRAINEE (Print Name): | TRAINEE (Signature): | Date: | Department: |
| --- | --- | --- | --- |
| Trainer / Supervisor (Print Name): | Trainer (Signature): | Date: | Department: |

UPON COMPLETION – EMAIL TO rfp@consultareinc.com

| NAME (Print): | Date: |
| --- | --- |

# Ventilation – Abrasive Blasting Operations Comprehension Quiz

Direction: Choose the best answer and encircle the letter.

- Who is responsible for following the suitable design of the location and equipment and construction based on the regulatory standards?

  - Employee

  - Management

  - Supervisors

- Develop and implement a monitoring program if any employee's exposure may equal or exceed an ________ time-weighted average of 85 decibels.

  - 5 Hours

  - 4 Hours

  - 8 Hours

- Who will undergo training in case a system failure occurs?

  - Employee

  - Supervisors

  - Management

- Use safety glasses protected by observation window screening when using deep-cutting abrasives.

  - True

  - False

- Dust Hazard measure noise exposure through a sound level meter and the dosimeter:

  - True

  - False

UPON COMPLETION – EMAIL TO rfp@consultareinc.com