Water Quality Training Material and Comprehension Quiz

- Policy

  - Water testing procedures are reviewed during the initial desk audit. Subsequently, this element is audited as part of each site audit through observation, review of records, and interviews with operating personnel. Evidence includes:

    - A documented water testing procedure is in place, including frequency and test method.

    - Water is microbiologically tested to verify the cleanliness of the supply and the effectiveness of treatment methods.

    - Appropriate standards are used to collect and analyze water.

    - The laboratories are accredited to offer water testing services where external laboratories are used.

- Procedures

  - Water Quality Test

    - Water Sampling Collection

      - Proper sampling techniques are essential to ensure representative results.

      - Selection of sampling points should consider factors such as:

        - Location of potential contaminants

        - Intended use of the water

    - Even though the water supply comes from the town or regional water supply in which the water is treated, safety tested, and maintained by the local authority, it is required that storage and distribution facilities ensure their water at the point of use complies with regulations.

    - Any water used in the operation for product or personnel hygiene must be verified to comply with local and national standards.

    - The quality of drinking water can differ based on the source water's condition and the treatment it receives. However, it adheres to the regulations established by the US Environmental Protection Agency (EPA). The Safe Drinking Water Act (SDWA) sets the guidelines for community water systems to follow.

      - The SDWA rules include guidelines for drinking water quality, water testing schedules, and water testing methods.

      - Even though U.S. tap water supplies are considered among the safest in the world, water contamination can still occur. There are many possible sources of contamination, including:

        - Sewage releases

        - Naturally occurring chemicals and minerals (for example, arsenic, radon, uranium)

        - Local land use practices (for example, fertilizers, pesticides, livestock, concentrated feeding operations)

        - Manufacturing processes (for example, heavy metals, cyanide)

        - Malfunctioning on-site wastewater treatment systems (for example, septic systems)

    - The EPA sets standards and regulations for the presence and levels of over 90 contaminants in public drinking water, including E. coli, Salmonella, Cryptosporidium, metals such as lead, and disinfection byproducts.

    - Romero’s Food Products’ drinking water comes from Santa Fe Springs Water Utility Authority. It is tested by the State Water Resources Control Board, Division of Drinking Water (DDW) certified water system operators, and laboratories to ensure its safety. SFSWUA drinking water from wells, treatment facilities, and distribution system pipes is routinely tested for bacterial, radiological and chemical constituents.

  - Water Standards

    - The site knows the national and/or international potable water standards and any customers’ microbiological or chemical water standards.

    - Monitoring and/or analysis are conducted to ensure water meets the required standard. The monitoring involves one or a number of the following:

      - Regular testing of water (e.g., pH, turbidity)

      - Checking filtration apparatus and changing it as required (Refer to Supplier Specifications)

      - Regular water cleaning of holding tanks and reservoirs

    - The following are the parameters being tested by the Santa Fe Springs Water Utility Authority

      - Primary Standards

        - Organic Chemicals

          - Tetrachloroethylene (µg/L)

        - Inorganic Chemicals

          - Aluminum (mg/L)

          - Arsenic (µg/L)

          - Barium (mg/L)

          - Bromate (mg/L)

          - Fluoride (mg/L) Naturally occurring.

          - Fluoride (mg/L) Treatment-related

          - Nitrate (mg/L as N)

        - Disinfection By-Products

          - Trihalomethanes (µg/L), Tested quarterly

          - Haloacetic Acids (µg/L), Tested quarterly

          - Total Chlorine Residual (mg/L), Tested weekly

        - Microbiological

          - E. coli

        - Other Parameters

          - Copper (mg/L)

          - Lead (µg/L)

      - Secondary Standards

        - Inorganic Chemicals

          - Aluminum (µg/L)

          - Chloride (mg/L)

          - Iron (µg/L)

          - Sulfate (mg/L)

        - General Physical Constituents (Physico-chemical)

          - Color

          - Odor (threshold odor number)

          - Total Dissolved Solids (mg/L)

          - Turbidity (NTU)

          - Unregulated Chemicals

          - Manganese (μg/L) (f)

          - Haloacetic acids (HAA5) (μg/L)

          - Haloacetic acids (HAA6Br) (μg/L)

          - Haloacetic acids (HAA9) (μg/L)

      - Additional Unregulated Chemicals of Interest

        - Alkalinity (mg/L as CaCO3)

        - Calcium (mg/L)

        - Magnesium (mg/L)

        - Perfluorobutanesulfonic Acid (ng/L)

        - Perfluorohexanoic Acid (ng/L)

        - Perfluorooctanesulfonic Acid (ng/L)

        - Perfluorooctanoic Acid (ng/L)

        - pH (standard unit)

        - Sodium (mg/L)

        - Total Hardness (mg/L as CaCO3)

  - Water Risk Assessment

    - The rate at which water is monitored or tested is, ideally, based on risk, owing to the potential for seasonal variations in the supply, but at minimum, water quality is monitored and assessed for compliance at least annually.

    - This includes potability and any additional quality or safety attribute. Utilize recognized guidance or hire a suitably qualified external resource to collect samples.

  - Laboratory

    - Certified laboratories employ specialized equipment and techniques to perform accurate and precise measurements to complete the required analysis.

    - The water is retested whenever the water source is changed or when equipment is added to treat or convey water within the site’s water system.

    - Where the water treatment occurs on-site, either before usage or as a treatment of wastewater, the treatment needs to have applicable analysis verifying the efficiency of the treatment.

    - Water used in any processing operation is from a microbiologically potable source. Costco requires this to be confirmed through testing by a certified third-party laboratory. Costco requires any municipal water source to be tested annually and well water to be tested quarterly. Samples are drawn from various sites throughout the facility, and test records are maintained. Water used for either processing or sanitation purposes is tested for generic E. coli.

- Responsibility

  - Quality Personnel

    - Ensures water testing procedure is documented and implemented.

    - Conducts sample collection and microbiological testing to verify water cleanliness in the facility.

    - Conducts regular monitoring of water testing.

    - Monitors filtration apparatus.

    - Monitors cleaning of water holding tanks and reservoirs.

    - Monitors sanitizer levels in water.

  - Production Personnel

    - Checks and changes filtration apparatus, as required.

    - Regular cleaning of water holding tanks and reservoirs.

  - Supervisors / Department Heads / Team Leaders

    - Ensure (inspect) and verify this policy and procedure is adhered to and;

    - Take corrective action to ensure compliance with this policy.

- Corrective Action

  - If the main water source is deemed contaminated, temporarily change to an alternative water source/s.

  - Use backup disinfection plants or spot dosing may be used to correct disinfection system failure within the water supply.

  - Review and revise the water quality testing procedure.

- Review – Quality

  - There is a documented water testing procedure in place, including frequency and test method.

  - Water is microbiologically tested to verify the cleanliness of the supply and effectiveness of treatment methods.

  - Appropriate standards are used to collect and analyze water.

  - Laboratories that are accredited to offer water testing services.

  - Regular monitoring of water testing (e.g., pH, turbidity).

  - Checking and monitoring of filtration apparatus and changing it as required.

  - Regular monitoring of cleaning of water holding tanks and reservoirs.

  - Regularly monitor sanitizer levels in water (levels normally tested at various food handling and processing sites).

- History

| Revision No.: | Revision Date: | Description of Change: | Originator / Author Name: | Title / Department: |
| --- | --- | --- | --- | --- |
| 0 | 20220807 | Original | Arnel Ryan | PCQI / Compliance |
| 1 | 20231013 | - Aligned with the updated program. - Updated the Policy, Procedure, and Review - Quality | Arnel Ryan | PCQI / Compliance |
| 2 | 20240603 | - Aligned with the updated program. - Changed Lesson to Training Material and Comprehension Quiz. | Arnel Ryan | PCQI / Compliance |

Requirements:

- The trainee (employee/visitor) has read or received a verbal translation of all or part of the policy, procedure, method, and/or SOP for which they are being trained.

- The trainee has observed a demonstration of the task they are to perform or the procedure for which they are being trained as required.

- The trainee has demonstrated the ability to perform the task with acceptable proficiency and minimal supervision.

| TRAINEE (Print Name): | TRAINEE (Signature): | Date: | Department: |
| --- | --- | --- | --- |
| Trainer / Supervisor (Print Name): | Trainer (Signature): | Date: | Department: |

UPON COMPLETION – EMAIL TO rfp@interlinkiq.com

| NAME (Print): | Date: |
| --- | --- |

# Water Quality Comprehension Quiz

Direction: Choose the best answer and encircle the letter.

- Who checks and changes the filtration apparatus as required?

- Quality Personnel

- Production Personnel

- Supervisors / Department Heads / Team Leaders

- Who conducts sample collection and microbiological testing to verify water cleanliness in the facility?

- Quality Personnel

- Production Personnel

- Supervisors / Department Heads / Team Leaders

- The rate at which water is monitored or tested should, ideally, be based on risk, owing to the potential for ________________ in the supply.

  - Potential hazards

  - Seasonal variations

  - Sanitation

- The water is __________ whenever the water source is changed or when equipment is added to treat or convey water within the site’s water system.

  - Retested

  - Tested

  - Filtered

- Use backup disinfection plants or spot dosing may be used to correct disinfection system failure within the water supply.

  - True

  - False

UPON COMPLETION – EMAIL TO rfp@interlinkiq.com