Water Treatment Lesson

- Policy

  - Chemical treatment of water is required to maintain the correct pH or chemical balance in boilers, heaters, or as an ingredient.

  - Treatment is controlled and carefully monitored where the above are required.

  - Any water used in the manufacturing or sanitation process is verified as potable.

  - Water and boiler (water heater) treatment chemicals are approved and properly stored (refer to 11.6.4).

- Procedure

  - Water Treatment

    - Collection

      - Collect samples from a chosen sampling point.

    - Screening and Straining

      - Place a large metal screen in front of the sampling point intake so that large particles pass through the water.

    - Coagulation and Flocculation

      - Add chemicals to help make the floating suspended particles clump together, forming a heavier and gelatinous particle.

      - A coagulant is added to water, and its positive charge neutralizes the negative charge of suspended contaminants.

      - After the water is coagulated, it can be filtered through an ultrafiltration or microfiltration membrane, or a medium filter, to remove the settled particles.

      - The settling system uses a tank to remove suspended particles from water. These particles, which have a higher density than the water, fall to the bottom of the tank. Common contaminants removed through settling include fats and greases, hair, sand, grit, wood, bottles, and sludge.

    - Sedimentation and Clarification

      - Water passes over the weir in the flocculator and travels to the clarifier's center.

      - Suspended particles are removed.

    - Filtration

      - Clarified water enters the filters from the top.

      - Gravity pulls the water down through the filters where it is collected.

      - Types of Filtration Methods

        - Reverse osmosis- During this process, a semi-permeable membrane is used to remove ions and unwanted particles. This process can remove both chemicals and bacteria, resulting in potable water safe for food and beverage production.

        - Carbon filtration- activated carbon filters can reduce organic compounds and chemicals found in drinking water. It can dissolve radon, minimize lead quantity, and remove solvents, pesticides, and industrial wastes. It’s crucial to note that activated carbon filtration does not remove metals, nitrate, or microbial contaminants.

        - Ion exchange- during ion exchange (IE) treatment, unwanted contaminants are removed from the water and replaced with a non-harmful substance. Ions can be positive or negative, and the replacement substance must have the same type of electrical charge. Keep in mind that ion exchange is a broad treatment and can remove numerous contaminants — including beneficial ones.

        - The settling system uses a tank to remove suspended particles from water. These particles, which have a higher density than the water, fall to the bottom of the tank. Common contaminants removed through settling include fats and greases, hair, sand, grit, wood, bottles, and sludge.

    - Disinfection

      - Sufficient amount of chlorine is added to the filtered water

      - Monitor levels of chlorine concentration

      - Avoid adding excess amounts that may lead to problems with the taste and odor

      - Types of Disinfection Methods

        - Chlorination- Where in-plant chlorination of water is required for washing, rinsing, or cleaning purposes, a free residual chlorine level of 0.25 ppm after 20 minutes of contact time (or equivalent at the point of use) is recommended. In-line chlorination that provides higher free residual chlorine levels at specific points is also acceptable. Regular sampling and residual chlorine testing are implemented to ensure a safe water supply.

        - UV disinfection- UV (ultraviolet) rays come from the sun, though invisible to the naked eye, and are an effective water treatment option because of their germicidal properties. It can disinfect water from bacteria, viruses, and even protozoans.

    - Storage

      - Store treated water in an elevated secured tank

    - Distribution

      - Select areas that can be distributed with treated water

  - Water Testing

    - As a minimum, request testing for biological hazards such as coliforms and E. coli. There should be no E. coli detected in your sample submitted. A test for enteroviruses and fecal indicators may be an option. Examples, Salmonella spp., Shigella spp., enterovirulent E. coli, Vibrio cholera, Yersinia enterocolitica, Campylobacter jejuni, and protozoa.

    - Any water used in the operation that could come in contact with the product or for personnel hygiene purposes is verified to comply with local and national standards.

    - For requirements, water used in the processing operation is from a microbiologically potable source. This is confirmed through testing by a certified third-party laboratory.

    - The cleanliness of the water supply should be verified bi-annually or annually. For requirements, any municipal water source is tested annually and well water is tested quarterly. Samples are drawn from various sites throughout the facility and test records are maintained.

- Responsibility

  - Quality Control Personnel

    - Collect samples from chosen sampling points

    - Perform in-house testing of microbiological and physicochemical for water

    - Send samples to duly accredited third-party laboratories

    - Monitor status and quality of treated water running in the facility.

  - Water Treatment Personnel

    - Perform water treatment process

    - Directly reports to senior management if there lapses in a water treatment facility

  - Preventative Control Qualified Individual (PCQI)

    - Oversee overall compliance of this procedure

    - Ensure all personnel are trained on this procedure

    - Take corrective actions if necessary

    - Perform a visual inspection of the involved areas of this procedure

  - Senior Management

    - Secure that water sources for potable and non-potable water are from a known and reliable source

    - Ensure adequate supply of potable water for processing needs

    - Verify if the program is adhered to

    - Take corrective actions if necessary

- Corrective Action

  - Isolate appropriate water outlets/tanks until satisfactory microbiological test results are obtained

  - Stop production where no potable supply can be provided

  - Review sampling and testing procedures

  - Improve staff instructions and training

  - In events of contamination, a review, and validation of the water supply procedures shall be conducted to assess the sources of contamination in the water supply system of the facility.

  - Revise water supply procedures if needed.

- Review – Quality

  - Annual review of water treatment procedures.

  - Annual review of records and interviews with operating personnel. Evidence includes:

    - Water treatment is performed as per written instructions;

    - Water treatment that is carried out is appropriate;

    - Water treatment is carried out using approved chemicals;

    - Water treatment equipment is regularly monitored;

    - Treated water is regularly monitored.

- History

| Revision No.: | Revision Date: | Description of Change: | Originator / Author Name: | Title / Department: |
| --- | --- | --- | --- | --- |
| 0 | 20230208 | Original | Arnel Ryan | PCQI / Compliance |

Requirements:

- The trainee (employee/visitor) has read or received a verbal translation of all or part of the policy, procedure, method, and/or SOP for which they are being trained.

- The trainee has observed a demonstration of the task they are to perform or the procedure for which they are being trained as required.

- The trainee has demonstrated the ability to perform the task with acceptable proficiency and minimal supervision.