Step Bolts and Manhole Steps Training Material and Comprehension Quiz

- Policy

  - Step bolts are uniformly spaced at a vertical distance of not less than 12 inches (30 cm) and not more than 18 inches (46 cm) apart. The minimum perpendicular distance between the centerline of each step bolt to the nearest permanent object at the back of the step bolt is 7 inches (18 cm).

  - Each maintenance hole step installed has a corrugated, knurled, dimpled, or another surface that minimizes the possibility of an employee slipping. They are inspected at the start of the work shift and maintained.

- Procedure

  - Step Bolts

    - Each step bolt installed on/after January 17, 2017, in a corrosion-prone environment is constructed of or coated with a corrosion-resistant material.

    - Each step bolt is designed, constructed, and maintained in such a way that the employee's foot does not slip off the step bolt's end.

    - Step bolts are uniformly spaced at a vertical distance not less than 12 inches (30 cm) and not more than 18 inches (46 cm) apart, measured from center to center. The distance between the entry and exit surfaces and the first step bolt may be different than the distance between the remaining step bolts; (see 4.1.4)

    - Step Bolt Spacing

    - Each step bolt has a minimum clear width of 4.5 inches (11 cm);

    - The minimum perpendicular distance between the centerline of each step bolt to the nearest permanent object at the back of the step bolt is 7 inches (18 cm). When the obstruction cannot be avoided, the minimum distance is 4.5 inches (11 cm);

    - Each step bolt installed before January 17, 2017, can support its maximum intended load.

    - Each step bolt installed on or after January 17, 2017, can support at least four times its maximum intended load.

    - Each step bolt is inspected at the start of the work shift and maintained.

    - A step bolt that bends more than 15 degrees from the perpendicular in any direction is removed and replaced with a required standard step bolt before an employee uses it.

  - Manhole Steps

    - Capable of supporting its maximum intended load.

    - Requirement of maintenance hole step installed on or after January 17, 2017:

      - Has a corrugated, knurled, dimpled, or another surface that minimizes the possibility of an employee slipping.

      - It is constructed of, or coated with, material that protects against corrosion if the maintenance hole step is located in an environment where corrosion may occur;

      - Has a minimum clear step width of 10 inches (25 cm);

      - It is uniformly spaced at a vertical distance not more than 16 inches (41 cm) apart, measured from center to center between steps. The spacing from the entry and exit surface to the first maintenance hole step may differ from the spacing between the other steps.

      - Has a minimum perpendicular distance between the centerline of the maintenance hole step to the nearest permanent object at the back of the step of at least 4.5 inches (11 cm); and

      - It is designed, constructed, and maintained to prevent the employee's foot from slipping or sliding off the end.

    - Inspected at the start of the work shift and maintained.

- Responsibility

  - Management

    - Ensures that all the requirements of OSHA for using step bolts and manhole steps will be followed for the safety of all the employees.

  - Employees

    - Recognizes potential hazards related to step bolts and maintenance hole steps.

    - Identifies safety measures and minimizes slips and falls.

- Corrective Action

  - Inspect maintenance hole steps before use. If maintenance hole steps are found unsafe (e.g., damaged or defective), immediately mark "Danger Do Not Use" and block them from use or access.

  - Do not attempt to repair or modify maintenance hole steps. A qualified person may only complete maintenance hole step repair.

  - Replace any step bolt bent more than 15 degrees from the perpendicular in any direction with one that meets the requirements.

- Review – Quality

  - Review the general requirements for the development of rules and regulations.

  - Confirm that the general requirements comply with the standards.

  - Review the documentation of the process with the relevant departments.

  - A review of these general requirements is conducted periodically.

- History

| Revision No.: | Revision Date: | Description of Change: | Originator / Author Name: | Title / Department: |
| --- | --- | --- | --- | --- |
| 0 | 20220901 | Original | Arnel Ryan | PCQI / Compliance |
| 1 | 20231205 | - Aligned with the updated program. - Updated the Procedure and Corrective Action. - Added Comprehension Quiz: ANSWER KEY. | Arnel Ryan | PCQI / Compliance |
| 2 | 20241003 | - Change the title part from Lesson to Training Material and Comprehension Quiz - Aligned with updated program - Change email from rfp@interlinkiq.com to rfp@consultareinc.com | Arnel Ryan | PCQI / Compliance |

Requirements:

- The trainee has read or received a verbal translation of all or part of the policy, procedure, method, and or SOP for which they are being trained.

- The trainee has demonstrated the task they are to perform or the procedure for which they are being trained as required.

- The trainee has demonstrated the ability to perform the task with acceptable proficiency and with minimal supervision as required.

| TRAINEE (Print Name): | TRAINEE (Signature): | Date: | Department: |
| --- | --- | --- | --- |
| Trainer / Supervisor (Print Name): | Trainer (Signature): | Date: | Department: |

UPON COMPLETION – EMAIL TO rfp@consultareinc.com

| NAME (Print): | Date: |
| --- | --- |

# Step Bolts and Manhole Steps Comprehension Quiz

Direction: Choose the best answer and encircle the letter.

- Step bolts are evenly spaced vertically between _________ and _________ measured center to center.

  - 12 Inches (30 cm), 18 Inches (46 cm)

  - 12 Inches (30 cm), 16 Inches (46 cm)

  - 12 Inches (30 cm), 7 Inches (18 cm)

- Each step bolt is inspected at the start of the work shift and maintained.

  - True

  - False

- Who is responsible for ensuring that all the requirements of OSHA for using step bolts and manhole steps will be followed for the safety of all the employees?

  - QA Supervisor

  - Management

  - QA Personnel

- Each step bolt installed on or after January 17, 2017, can support at least ______ times its maximum intended load.

  - Seven

  - Five

  - Four

- If maintenance hole steps are found unsafe, immediately mark "Danger Do Not Use" and block them from use or access.

  - True

  - False

## UPON COMPLETION – EMAIL TO rfp@consultareinc.com