| Document Title: Cleaning and Sanitation – Elevator Conveyor | Document Control No.: [Enter Document Control No.] | Rev. No.: [Enter Revision No.] |
| --- | --- | --- |
| Department: [Enter Department] | Supersedes: Original | Effective Date: [Enter Date] |
| Prepared by: [Affix Name, Signature, Position Title] | Reviewed by: [Affix Name, Signature, Position Title] | Approved by: [Affix Name, Signature, Position Title] |
| Date: [Enter Date] | Date: [Enter Date] | Date: [Enter Date] |

- Purpose:

  - To properly clean the elevator conveyors of the production facilities.

- Frequency:

  - Quarterly

  - [ADD FREQUENCY HERE]

- Tools and Equipment:

  - Pressure washer

  - Brush

  - Scouring pad

  - Cloth wipes

  - Polyethylene plastic sheeting

  - Camera (Photos)

  - Personal Protective Equipment (PPE)

    - Rubber gloves

    - Hair cap

    - Mask

    - Waterproof boots

    - Waterproof clothes

    - Waterproof apron

  - [ENTER LIST OF EQUIPMENT HERE]

- Consumable(s):

  - Liquid Detergent

  - Stain remover

  - Lubricant

  - Sanitizer

  - pH strips (litmus paper)

  - [ENTER LIST OF CONSUMABLES HERE]

- Responsibility:

  - Housekeeping Personnel – to perform the cleaning and sanitation of the elevator conveyor.

  - Housekeeping Head – to monitor/inspect the cleanliness and sanitation of the elevator conveyor.

  - Quality Assurance Personnel – to ensure that proper cleaning procedures are applied.

  - Maintenance Personnel – to perform the lubrication of the elevator conveyor.

  - Maintenance Head – to monitor/inspect that the elevator conveyor is well-lubricated.

  - [ADD OR CUSTOMIZE RESPONSIBLE PERSON HERE]

- Procedure/Instructions:

  - Turn off the elevator conveyor

  - Cover/secure electrical and non-waterproof components with polyethylene plastic sheeting.

  - Manually remove large pieces of food or other debris from the belt, sprockets, shaft, and other areas of the elevator conveyor where dirt is seen.

  - If necessary, disassemble the elevator conveyor.

  - Using the pressure washer, pre-rinse with low-pressure water and air to wash away remaining dirt particles.

  - Apply liquid detergent to the surfaces of the elevator conveyor, use a brush/scouring pad, and stain remover if necessary.

  - Rinse the liquid detergent using heated pressurized water. Set the pressure to high.

  - Check for the presence of detergent residue using pH strips (litmus paper).

  - If step 6.4 is applicable, re-assemble the elevator conveyor.

  - Turn on the elevator conveyor and run at a low speed to dry and remove pooled water.

  - If necessary, apply lubricant to the gearings of the elevator conveyor. Wipe excess lubricant using cloth wipes.

  - Sanitize using company-approved sanitizer. Run at a low speed to ensure that all parts and surfaces of the elevator conveyor are exposed to the sanitizer.

  - Leave to dry.

  - Inspect if the elevator conveyor is properly cleaned.

  - Fill out checklist

  - [ADD OR CUSTOMIZE PROCEDURE HERE]

- Corrective Action:

  - Re-rinse with heated and pressurized water if pH strips (litmus paper) turn blue (basic) to remove detergent residue.

  - Re-lubricate gearings if needed to guarantee high productivity and decrease downtime.

  - Re-inspect after re-cleaning and re-lubrication is done.

  - Issue incidence report and take photos for documentary purposes.

  - Issue CAPA (Corrective and Preventive Action Report) to involved housekeeping and/or maintenance personnel.

  - [ADD OR CUSTOMIZE CORRECTIVE ACTIONS HERE]

- Related Forms:

  - Quarterly Cleaning and Sanitation Checklist of Conveyors

  - SSOP Program for Clean and Sanitized Equipment

  - Quarterly Preventive Maintenance Checklist of Conveyors

  - [ADD OR CUSTOMIZE RELATED FORMS HERE]

- History/Changes:

| Revision No.: | Revision Date: | Description of Change: | Originator / Author Name: | Title / Department: |
| --- | --- | --- | --- | --- |
| 0 | YYYYMMDD | Original | ENTER AUTHOR NAME | ENTER TITLE/ENTER DEPARTMENT |

*Should you need assistance in customizing this document please email services@consultareinc.com