Maintenance Staff and Contractors Lesson

- Policy

  - Maintenance staff and service contractors are engaged to complete work in food production areas, observe all personnel, and process hygiene requirements. Service contractors are provided with protective clothing, as required.

- Procedure

  - Personnel Hygiene

    - Maintenance staff and service contractors engaged to complete work in food production areas observes all personnel and process hygiene requirements.

      - Refer to Personnel Welfare

      - Refer to Clothing and Personal Effects

    - Service contractors are provided with protective clothing as required, and repairs are completed following the same requirements as maintenance staff.

    - All service contractors are aware of the personnel hygiene requirements, and they are provided with any necessary protective clothing or that protective clothing meets the same requirements as those of the supplier staff.

  - Maintenance Activity

    - Maintenance of equipment or building structures is completed in a manner that does not pose a risk to the product, materials (including packaging materials), or equipment.

    - Maintenance supervisors ensure that they are notified by all contractors engaged to complete work in any product handling areas.

    - Service contractors are to inform the maintenance supervisor if any required work poses a potential threat to product, packaging, or equipment safety (i.e., pieces of electrical wire, damaged light fittings, loose fittings overhead, etc.). When necessary, maintenance is conducted outside processing times.

    - Service contractors notify the maintenance supervisor in the event of any breakage or damage that could expose products, packaging, or equipment to contamination.

    - Service contractors notify the maintenance supervisor when work has been completed.

    - Plant supervisors and operators ensure that appropriate and effective clean-up measures are taken once all maintenance or service contractor activity is completed and before the commencement of plant operations.

    - All maintenance and other engineering contractors required to work on-site are trained in the site's food safety and hygiene procedures or escorted at all times until their work is completed.

    - All tools and debris are removed from any maintenance activity once it has been completed and informed by the area supervisor and maintenance supervisor so appropriate hygiene and sanitation can be completed before the commencement of facility operations.

  - Maintenance Activity - Preventive Maintenance Program

    - Responsible

      - The Maintenance Manager will be responsible for the planning and execution of the Maintenance plan.

    - Preventive Maintenance Procedure - The following activities are included in the Preventive Maintenance Checklist

      - At the beginning of each week, refer to the Preventative Maintenance checklist to schedule the maintenance to be performed for that week.

      - Daily

        - The Pre-operation and Operational reports (filled out by QA) will inform Maintenance of what tasks need to be performed.

      - Monthly / Bimonthly

        - Tasks listed on the Preventive Maintenance Checklist

      - Quarterly

        - Tasks listed on the Preventive Maintenance Checklist.

      - Annually

        - Tasks listed on the Preventive Maintenance Checklist

        - Refrigeration equipment checked and serviced by an outside contractor (Contractor will perform service and record results on their invoice).  All Contractors must sign the Contractor Rules and Confidentiality agreement annually.

        - Inspect rooftop fans and intake filters.

    - Employees are encouraged to report problems they notice to their supervisor or the QA department.  Notification occurs when the potential risk to the product is evident through maintenance reports in food processing areas.

    - Maintenance issues that are related to maintaining food safety will be treated as “high priority” by the Maintenance personnel.  Other requirements to ensure food safety are:

      - All tools must be maintained in a sanitary manner, cleaned daily, and removed when the repair is complete by both Maintenance employees and Contractors.

      - GMPs and food safety hygiene must be followed by both Maintenance employees and Contractors

      - Maintenance Procedures that pose no risk to product safety and integrity and are followed

      - Maintenance of equipment or building structures must be completed in a manner that does not pose a risk to the product, materials (including packaging materials), or equipment

      - Temporary fixes, such as wire, tape, string, etc., are not permitted.

      - Equipment must be cleaned, washed & inspected by Quality Assurance prior to use

      - Paint must be suitable for use and NOT used on Food contact surfaces.

    - New equipment, equipment that is not used on a regular basis, and/or equipment that is kept out of the production areas needs to be:

      - Labeled

      - Properly covered

      - Properly cleaned and sanitized BEFORE going back into service

      - The oil and grease that is used on food contact surfaces for plant maintenance and equipment purposes must be food grade.

        - Food-grade and non-food-grade grease must be labeled and stored separately in a sanitary manner.

    - Once the Preventive Maintenance task has been completed, all relevant information (unusual findings) will be recorded in the work order management system.

    - If there was nothing unusual to report, then simply update the date that the maintenance was.

    - Maintenance staff and service contractors engaged to complete work in food production areas observes all personnel and process hygiene requirements.

    - Service contractors are provided with protective clothing as required, and repairs are completed following the same requirements that maintenance staff observes, such as:

      - Maintenance of equipment or building structures must be completed in a manner that does not pose a risk to the product, materials (including packaging materials), or equipment.

      - The maintenance supervisors ensure that they are notified by all contractors engaged to complete work in any product handling areas.

      - They ensure that all service contractors are aware of the personnel hygiene requirements and that they are provided with any necessary protective clothing or that protective clothing meets the same requirements as those of the supplier staff.

      - Maintenance staff and service contractors ensure that they account for and remove all tools and debris from any maintenance activity once it has been completed in any product handling area and inform the area supervisor so appropriate sanitation can be completed.

      - Service contractors are to inform the maintenance supervisor if any required work poses a potential threat to product, packaging, or equipment safety (i.e., pieces of electrical wire, damaged light fittings, loose fittings overhead, etc.). When necessary, maintenance is conducted outside processing times.

      - Service contractors shall notify the maintenance supervisor in the event of any breakage or damage that could expose products, packaging, or equipment to contamination.

      - Service contractors must notify the maintenance supervisor when work has been completed.

      - Plant supervisors and operators ensure that appropriate and effective clean-up measures are taken once all maintenance or service contractor activity is completed and prior to the commencement of plant operations.

      - Supplier staff, maintenance personnel, and service contractors adhere to the correct procedures when completing maintenance on all equipment. As part of maintenance procedures, repaired equipment is inspected for missing parts (nuts, bolts, springs, etc.) prior to use.

  - Reporting and completing repairs and cleaning the equipment after repairs

    - Maintenance staff and service contractor will report completed repairs to the Maintenance Manager, and cleaning of completed equipment are reported to the Sanitation Supervisor.

    - Where machinery that exists over product lines or food contact surfaces requires lubrication, only food-grade lubricants are used.

    - Where paint is used on equipment, roofs, walls, or floors, it is inspected for good condition and suitable for use. Paint is not used on food contact surfaces.

  - Chemicals and Lubricants

    - All chemicals or lubricants must be securely stored during periods of non-use in a place separate from all other products in the warehouse.

      - No chemicals or lubricants can be stored on the floor. Chemicals must be sealed in a leak-proof container. Any leaking container must be removed and legally disposed of.

      - All containers of chemicals must be labeled and used for the intended purpose.

      - All chemicals must be listed in the USDA list of proprietary substances and non-food compounds, or an MSDS must be on file listing the safe uses of the chemicals stored.

      - Under this program, you must report to your supervisor any violation, unsafe or unauthorized use or storage of chemicals that could directly or indirectly cause the product to become contaminated.

      - All grease cartridges are food grade. Open food-grade grease lubricants and guns for production will be stored inside a labeled, clean container.  Only food-grade lubricants are to be used in the processing areas. All food-grade lubricant guns will be labeled (engraved) to indicate food use only. No other items are to be stored in food grade lubricant bin. Food-grade lubricant guns stored in the maintenance shop shall be labeled and stored hygienically.

  - Facility repairs shall be documented on a work order with a target date; the work order shall be signed off after completion and verification of the finished item. These work orders can come from our weekly production meetings, weekly maintenance meetings, or from facility inspections; after the work order is documented, it goes directly to our maintenance department until the item is completed and signed off.

  - Equipment Repairs & Maintenance and Servicing During Production

    - Preventative maintenance program will be documented in the “Weekly Preventative Maintenance lubrication schedule,” verified by a QA person once a week and filed in the production records cabinet.

    - Monthly preventative maintenance overview/activities will be logged and verified by a QA person once monthly and filed in the production records cabinet.

    - Temporary repairs must adhere to GMPs. A permanent solution must be given within a reasonable timeframe.

    - The Maintenance shop will be maintained in an orderly fashion, including any scrap.

    - Use the PIP program to create work orders for any facility repairs or other repairs that require a frame time to get fixed.

    - Maintenance tools must be sanitized prior to entering a production room during operation hours, and sanitation must be documented on the “Daily Operational Sanitation Record” by a Quality Assurance staff or trained employee.

- Responsibility

  - Maintenance Supervisors

    - Ensure they are notified by all contractors engaged to complete work in any product handling areas.

    - Ensure that all service contractors are aware of the supplier’s personnel hygiene requirements and that they are provided with any necessary protective clothing or that protective clothing meets the same requirements as those of the supplier staff.

    - Ensure appropriate and effective clean-up measures are taken once all maintenance or service contractor activity is completed and before the commencement of plant operations.

- Corrective Action

  - In the event of product, materials, or equipment contamination due to maintenance staff and contractors’ activities:

    - Train the key personnel involved in the implementation of the program.

    - Review the procedures for effectiveness and revise if needed.

- Review – Quality Department

  - Annual review of compliance with Maintenance Staff and Contractors’ requirements by observing the following:

    - Maintenance procedures afford no risk to product safety and integrity

    - Maintenance procedures are known by maintenance personnel and contractors

    - Maintenance procedures are being followed

    - Maintenance procedures include food safety and hygiene practices

    - Maintenance staff follow food safety and hygiene practices

    - Maintenance contractors follow food safety and hygiene practices

    - Operating staff and supervisors are notified when repairs are made/completed

    - All tools, parts, and debris are removed from repair areas

    - Sanitation activity occurs after maintenance repair in food processing areas

    - Notification occurs when the potential risk to the product is evident through maintenance activities or breakdowns

    - Maintenance records are available and complete

    - There is a planned maintenance schedule

    - The maintenance schedule includes critical equipment and areas of the site

    - There are maintenance procedures that include food safety issues

    - The planned maintenance schedule is being followed

- History

| Revision No.: | Revision Date: | Description of Change: | Originator / Author Name: | Title / Department: |
| --- | --- | --- | --- | --- |
| 0 | 20221125 | Original | Arnel Ryan | PCQI / Compliance |

Requirements:

- The trainee (employee/visitor) has read or received a verbal translation of all or part of the policy, procedure, method, and/or SOP for which they are being trained.

- The trainee has observed a demonstration of the task they are to perform or the procedure for which they are being trained as required.

- The trainee has demonstrated the ability to perform the task with acceptable proficiency and minimal supervision.