Storage of Dry Ingredients, Packaging, And Shelf Stable-Packaged Goods Training Material and Comprehensive Quiz

- Policy

  - Dry goods storage rooms must be separate from wet areas and constructed to prevent contamination and pest infestation.

  - As per Costco requirements, the facility controls the proper storage and transport of all ingredients and products.

- Procedures

  - Storage Area

    - Dry ingredients, packaging, and shelf-stable packaged goods storage areas are located away from wet processing areas. It is designed to protect products, components, and packaging materials.

    - The storage area is designed to facilitate stock rotation.

    - Storage racks and shelving are made up of non-corrosive material and easily cleanable. Racks allow access to floor/wall junctions for cleaning.

    - Using wooden racks to store packaging and packing materials is limited to dry areas only.

    - Stands and the lower shelves of stands are at least 150 mm (6 inches), or as required by applicable regulation above floor level to facilitate proper cleaning.

      - Refer to Daily Operational Sanitation Record.

    - The storage area is kept dry, cool, well-lit, ventilated, and vermin-proof.

  - Dry Ingredients, Packaging, and Shelf Stable Packaged Goods

    - Dry ingredients, packaging, and shelf-stable packaged goods are labeled with the date of receipt before storage and then rotated on a first in, first out, or the first expiry, first out basis.

      - Refer to Stock Records Log Sheet.

    - All dry ingredients, packaging, and shelf-stable packaged goods are properly labeled.

    - Dry ingredients, packaging, and shelf-stable packaged goods are stored according to the manufacturer’s instructions.

    - Dry ingredients, packaging, and shelf-stable packaged goods are stored off the floor, shelving, or racks to prevent contamination and allow adequate air circulation around the product. The materials are kept a minimum of 6 inches from the floor. Foods are not directly stored on the floor.

    - Dry ingredients, packaging, and shelf-stable packaged goods are stored well away from chemicals and cleaning equipment to prevent cross-contamination from physical, chemical, or biological contaminants.

    - Dry ingredients, packaging, and shelf-stable packaged goods are protected from exposure to moisture and direct sunlight.

    - Packaged foods that have been opened are stored in clean, non-toxic, food-grade, pest-proof containers with tight-fitting lids or protective packaging (such as foil, plastic food wrap, greaseproof paper, etc.). The container is appropriately labeled and dated.

    - Raw materials containing allergens are identified, properly labeled, and segregated.

      - Refer to Allergen Management Program and Allergen Blue Tag Area Location.

  - Pest Control

    - Food spills are cleaned up promptly to minimize pests' attraction and reduce the risk of contamination.

    - Pests are deterred and eradicated as soon as possible. Signs of pest activity include droppings, eggs, webs, feathers, and smells.

    - Staff adhere to good housekeeping practices and visually inspect products for signs of damage, contamination, or spoilage.

    - As per Costco, an integrated pest management program (IPM) manages the facility's interior and exterior, including product and packaging storage areas.

      - Refer to Pest Prevention Program

      - Refer to Pest Control Inspection Report

- Responsibility

  - All Personnel

    - Shall adhere to this policy.

  - Quality Department/Preventive Control Qualified Individual (PCQI)

    - Visually inspect areas for compliance with this policy.

    - Oversee (Verify) compliance of procedure.

    - Ensure personnel are trained on this policy and procedures.

    - Validate training and adherence to policy and procedure.

    - Monitor and document non-conformance observation, also known as discrepancy, using the Corrective Action Preventive Action-CAPA Report.

  - Shipping/Receiving Personnel

    - Ensures that products are properly labeled, stored, and segregated into different groups (allergen-containing ingredients, non-allergen ingredients).

  - Maintenance

    - Ensures that the storage area and shelving racks are maintained in good condition.

- Corrective Action

  - In the event of product, materials, or equipment contamination due to Storage of Dry Ingredients, Packaging and Shelf Stable Packaged Goods:

    - Train the key personnel involved in the implementation of the program.

    - Review the procedures for effectiveness and revise if needed.

  - In the event the storage area is infested with pests,

    - Isolate the infested material or product and dispose of it.

    - Transfer the materials or products to another area.

    - Eliminate pests using traps or approved chemicals.

    - Clean, sanitize, inspect, and verify the area to continue the operations.

- Review – Quality

  - Product storage rooms are located away from wet processing areas

  - Storage rooms are adequately designed to protect products, ingredients, materials, or packaging materials.

  - Packaging racks are made of material that is easily cleanable.

  - A Pest control program is in place to ensure no pest activity

  - Dry ingredients, packaging, and shelf-stable packed goods are properly labeled and stored based on the first expiry, first-out rule.

  - Dry ingredients, packaging, and shelf-stable packed goods are stored away from chemicals and cleaning equipment.

- History

| Revision No.: | Revision Date: | Description of Change: | Originator / Author Name: | Title / Department: |
| --- | --- | --- | --- | --- |
| 0 | 20220805 | Original | Arnel Ryan | PCQI / Compliance |
| 1 | 20240611 | - Aligned with the updated program. - Changed Lesson to Training Material and Comprehension Quiz. | Arnel Ryan | PCQI / Compliance |

Requirements:

- The trainee has read or received a verbal translation of all or part of the policy, procedure, method, and or SOP for which they are being trained.

- The trainee has demonstrated the task they are to perform or the procedure for which they are being trained as required.

- The trainee has demonstrated the ability to perform the task with acceptable proficiency and with minimal supervision as required.

| TRAINEE (Print Name): | TRAINEE (Signature): | Date: | Department: |
| --- | --- | --- | --- |
| Trainer / Supervisor (Print Name): | Trainer (Signature): | Date: | Department: |

UPON COMPLETION – EMAIL TO rfp@interlinkiq.com

| NAME (Print): | Date: |
| --- | --- |

Storage of Dry Ingredients, Packaging, And Shelf Stable-Packaged Goods Comprehension Quiz

Direction: Choose the best answer and encircle the letter.

- Dry ingredients, packaging, and shelf-stable packaged goods storage areas are located away from wet processing areas.

- Dry processing areas

- Wet processing areas

- Cold areas

- Storage _________ and shelving are made up of non-corrosive material and easily cleanable. _____ allow access to floor/wall junctions for cleaning.

  - Shelves

  - Racks

  - Cabinets

- Food spills are cleaned up promptly to minimize pests' attraction and reduce the risk of contamination.

  - True

  - False

- Dry ingredients, packaging, and shelf-stable packaged goods are stored according to the ___________ instructions.

  - FDA’s

  - Supplier’s

  - Manufacturer’s

  - All of the above

- Who is in charge of ensuring that products are properly labeled, stored, and segregated into different groups (allergen-containing ingredients, non-allergen ingredients)?

  - Shipping/Receiving Personnel

  - All Personnel

  - Maintenance

UPON COMPLETION – EMAIL TO rfp@interlinkiq.com