| Document Title: SSOP-Cleaning and Sanitation-Stuffing Room | Document Control No.: [Enter Document Control No.] | Rev. No.: [Enter Revision No.] |
| --- | --- | --- |
| Department: [Enter Department] | Supersedes: Original | Effective Date: [Enter Date] |
| Prepared by: [Affix Name, Signature, Position Title] | Reviewed by: [Affix Name, Signature, Position Title] | Approved by: [Affix Name, Signature, Position Title] |
| Date: [Enter Date] | Date: [Enter Date] | Date: [Enter Date] |

- Purpose:

  - To provide a standard operating procedure for cleaning and sanitation in the stuffing room.

- Frequency:

  - At the end of the shift

  - [ADD FREQUENCY HERE]

- Tools and Equipment:

  - Polypropylene plastic broom

  - Vacuum cleaner

  - Dustpan

  - Mop and mop squeezer

  - Microfiber cloth and duster

  - Spatula

  - Scrubbing pad

  - Glass wiper/ window squeezer

  - Pail/ bucket/ basin

  - Spray bottle

  - Plastic film

  - Personal Protective Equipment (PPE)

    - Apron

    - Rubber gloves – chemical-resistant

    - Eye goggles

    - Facemask

    - Hair net

  - Trash bin and polybag for waste collection

  - Cleaning and sanitation recording tools

    - Ballpen

    - Clipboard

    - Records/ forms

  - [ENTER LIST OF TOOLS AND EQUIPMENT HERE]

- Consumable:

  - Potable supply of water

  - All-purpose detergent

  - Sanitizer (chlorine-based)

  - Test strips for disinfectant and sanitizer concentration

  - [ENTER LIST OF CONSUMABLES HERE]

- Responsibility:

  - Production staff – conducts daily cleaning and sanitation of the stuffing room;

  - Production supervisor - ensures that the activity is carried out according to the master cleaning and sanitation plan and is compliant with the instructions specified in this document;

  - Quality assurance personnel – conducts verification of the compliance of the production group in the cleaning and sanitation program of the stuffing room;

  - Production head/ manager – reviews and approves the revised guidelines as necessary

  - [ADD OR CUSTOMIZE RESPONSIBLE PERSON HERE]

- Procedure/Instructions:

  - Pre-cleaning activity

    - Wear appropriate personal protective equipment (PPE) when preparing chemicals and cleaning and sanitation activities.

    - Prepare the cleaning solution and sanitizer per the manufacturer’s recommendation.

      - Check the concentration of the cleaning detergent and sanitizer by dipping the applicable test strip into the solution, then immediately removing and comparing it to the color chart.

      - Check the concentration of the solutions to ensure that their effectiveness is maintained over time.

    - Transfer ample amounts of the freshly prepared solutions into the hand-held spray bottles (for easy-to-reach areas) in the basin or bucket.

    - Prepare the cleaning tools and equipment like a broom, dustpan, spatula, microfiber cloth, vacuum cleaner, mop, etc.

    - Turn off the main power supply of the electrical equipment in the area where the cleaning and sanitation will be conducted.

    - Cover all equipment and other contact surfaces with plastic film before starting the cleaning of the ceiling to protect them from the dust that could fall off while cleaning.

    - Observe proper ventilation inside the stuffing room to allow air drying of all sanitized surfaces.

    - As a rule of thumb, start the cleaning and sanitation from the top, progressing downwards. (e.g., from the ceiling and then to the walls up to the floor.)

  - Cleaning and sanitation of ceilings and light fixtures

    - Use a vacuum cleaner to remove cobwebs, dirt, dust, and debris lodged in the crevices of the ceiling.

    - Scrub thickened dust, grime, and stains with the cleaning solution using a brush with a long handle or spatula.

    - Clean the tube light fixtures using a wet duster followed by a dry duster.

    - Scrub the ceiling using a microfiber cloth (with handle attachment) dipped in the cleaning solution.

    - Wipe the ceiling using a clean microfiber cloth. Repeat until the suds are removed.

    - Spray the sanitizer solution on the ceiling with at least 10-15 minutes of contact time before wiping it with a clean microfiber cloth.

  - Cleaning and sanitation of walls and floors

    - Scrub thickened dust, grime, and stains on walls with a scrubbing pad dipped in the cleaning solution. It is also advised to use a spatula to scrape away dried-out dirt.

    - Remove sweepings left on the floor using a polypropylene broom or vacuum cleaner.

    - Ensure that the coving and corners of the wall-floor junction are reached using a mop with a cleaning solution.

    - Spray the sanitizer solution on the walls. Let it stand for 10-15 minutes, and wipe it using a clean microfiber cloth.

    - For the floor, sanitize using a clean mop dipped in sanitizer in the bucket. Let it stand for at least 15 minutes, then wipe it with a clean mop.

  - Cleaning and sanitation of windows/screens

    - Remove the window screens.

    - Scrub the screen using a brush gently with the detergent solution under running water. Let it dry naturally.

    - For glass windows, remove dirt and dust using a microfiber towel.

    - Spay with sanitizer and wipe down using a window squeezer.

    - Let it dry naturally.

  - Cleaning and sanitation of trays, pallets, and trolleys

    - Remove the accumulated dirt on the surfaces using a duster or clean microfiber cloth first.

    - Spray the cleaning solution on the microfiber cloth and wipe the surfaces.

    - Rinse with a wet, clean microfiber cloth until the suds are removed.

    - Spray sanitizer on the microfiber cloth and wipe it on the surface of the trays, crates, and trolleys.

    - Let dry.

  - Cleaning and sanitation of highly touched surfaces (door knobs, handles, switchboards)

    - Wipe with a microfiber cloth to remove adhering dirt.

    - Spray the cleaning solution on the microfiber cloth and wipe the highly-touched surfaces.

    - Rinse with a wet, clean microfiber cloth until the suds are removed.

    - Spray sanitizer on the microfiber cloth and wipe it on the surface of the trays, crates, and trolleys.

    - Let dry.

  - Post-Cleaning Activity

    - Collect all the waste from the waste bin into a poly bag and send it to the scrap room. Clean the waste bin with a dry duster and replace it with a new trash bag.

    - Clean and mop the entire floor twice with the sanitizer solution. Let dry.

    - Discard any leftover diluted liquid detergent and disinfectant solution down the sink with running water.

    - Return all cleaned cleaning aids or tools to their proper storage location.

    - Ensure clean and sanitized surfaces are dry before returning to working operations.

    - Record the activities in the designated forms provided. Ensure that it is checked and verified timely by the assigned personnel.

- Corrective Action:

  - If the solution does not reach the appropriate reading, discard it according to its proper waste disposal for

chemicals and prepare a new solution as per the manufacturer’s recommendation. If the concentration is still not obtained:

    - Check the expiration date of the chemicals.

    - Prepare a new solution using unexpired chemicals.

    - Source out an alternative supplier or brand.

  - If the stain or dirt cannot be removed immediately, repeat the rinsing with detergent twice or as needed to soften the soiled portion. Call the attention of the production aide responsible and repeat the cleaning and sanitation procedures with strict observance.

  - If there is negligence on the part of employees, e. g., improper cleaning and sanitation, failure to follow the pre-cleaning procedure, failure to follow the frequency of cleaning and sanitation, or failure to monitor and verify the activity, the following must be administered:

    - Conduct coaching to the concerned personnel to demonstrate proper solution preparation and cleaning procedure.

    - Conduct training to reiterate the importance of the procedure

  - Issue a non-conformance and corrective action report for historical tracking and monitoring of the corrective action.

  - In the event of chemical contact with personnel, wash the affected area under running clean water. Refer to the chemical SDS for first aid treatment. Report immediately to the supervisor and medical personnel for further inspection.

  - [ADD OR CUSTOMIZE CORRECTIVE ACTION HERE]

- Reference Document:

  - Test Strip Color Chart

  - Chemical SDS and Recommended Preparation

  - Disinfection and Sanitation Chemical Preparation Form

  - Stuffing Room Cleaning and Sanitation Checklist

  - Cleaning of Cleaning tools checklist

  - [ADD OR CUSTOMIZE REFERENCE DOCUMENT HERE]

- History/Changes:

| Revision No.: | Revision Date: | Description of Change: | Originator / Author Name: | Title / Department: |
| --- | --- | --- | --- | --- |
| 0 | YYYYMMDD | Original | ENTER AUTHOR NAME | ENTER TITLE/ENTER DEPARTMENT |

*Should you need assistance in customizing this document, please email services@consultareinc.com