Product Rework Training Material and Comprehension Quiz

- Policy

  - Romero's Food Product Inc. does not rework sealed products, per company policy. Below, three definitions are provided to elucidate this statement. If the product analysis so dictates, we will recycle or reprocess the product.

  - Rework: treatment of product by a process different than what was originally used to produce the product.

  - Recycle: treatment of a product through a predetermined redirection of the product to a previous step in the process.

  - Reprocess: retreatment of the product by the same process to provide acceptable quality and/or food safety.

    - Romero’s Food Products concludes that remixing a product that is out-of-spec due to moisture or diameter is reprocessing as opposed to reworking.

  - Product sent back to some point in the process to bring it within specs. An exception to this policy shall be export and preservative-free products that are not to be reworked, reprocessed, or recycled.

  - The company documents all the products that have been reworked or recouped under qualified supervision and an approved procedure.

  - Traceability is retained, and the product is identified. Evidence is provided that each lot is released only after inspection.

    - Refer to Product Trace Program

    - Refer to Product Identification Program

    - Refer to Product Release Program

  - An important element of the rework procedure is the criteria for determining when a product will be reworked, how much can be reworked, under what conditions it can be reworked, and how it is identified and traced (Rework Protocol).

  - After being reworked, the product is reviewed per company-designated food safety checks to ensure that it meets all applicable specifications. Care is taken to ensure that allergens are not accidentally introduced into the product through rework.

  - Reworking of perishable products considers the shelf life of the reworked product (i.e., the oldest product).

  - Recouping operations ensure that recouped product is not dented or damaged outside company specifications. Records of all reworking/recouping operations are maintained.

  - When reworking or repackaging ingredients, whether in a warehouse or production setting, the area where the work is being conducted is checked for potential allergen residue, or the product itself is contained if it is an allergen to protect other surrounding products from contamination, otherwise those products are now labeled appropriately.

  - Ensure all of the details of the original product label remain with the damaged product. Use only the assigned utensils for repacking/reworking. Ensure that any new container or repacked size contains the same ingredient information, allergen alerts, and lot numbers as the original label.

  - Affix additional allergen alert label to finished reworked product.

- Procedure

  - Non-conforming products are segregated by the responsible employees in the following containers:

    - White: Waste; the product is not reworked, reprocessed, or recycled but disposed of per Non-Conforming Product & Equipment Program.

    - Yellow: Reprocess; Corn Product is reprocessed at a rate of 50lbs (10” of product) using 90˚ Fahrenheit water and mixed for 10 minutes per batch of the same product and/or lot # to ensure traceability.

    - Blue: Reprocess; Flour Product is ground into a fine powder and mixed into the same product (25lbs per burrito and/or Casero formula) and/or lot # to ensure traceability.

  - Corn operators record reprocessing on Corn Preparation Control Sheet.

  - Flour operators record reprocessing on Flour Preparation Control Sheet.

  - Waste disposals are stored electronically by R.T.C Waste at the time of disposal by inputting the unique access code assigned to each department foreman. And physically on the Total Waste Form.

  - Damaged products are isolated in the designated area in the warehouse or taken to the Repack area immediately. Fill up a “Product Non-conformity” Form detailing the problem with the batch production, clearly identifying the part of the batch that has been affected.

  - Damaged product reworked/repacked within a specified period; if any product has been exposed to the elements, corrected/secured immediately to prevent further contamination.

  - Product for rework is transferred to the process area (except for recouping) as requested in the “Product for Rework” form; a copy will be kept in the warehouse while the original will be sent with the goods.

  - Finished goods for recouping are to be transferred in a way separated from the other raw materials and components.

  - When the goods are transferred, the production operator checks the pallet for the correct product code and batch number labeled in the “Product to be Reworked” Form.

  - Rework Protocol proceeds.

  - The responsibility and methods outlining how raw materials (including in-process materials or food packaging products) are documented, implemented, and processed in a manner that does not contaminate raw materials or food packaging.

  - The methods applied ensure:

    - Reworking operations are supervised by qualified personnel.

    - Reworked products are identified and traceable.

    - Each lot of reworked products is inspected or analyzed as required before release.

    - Inspections and analyses conform to the requirements outlined in SQF element 2.4.4 (Product Sampling, Inspection, and Analysis)

    - Release of the reworked product conforms to SQF element 2.4.7 (Product Release).

  - Food packaging manufactured with post-consumer recycled materials conform to the applicable regulatory requirements.

  - Food Packaging that contains printed information is handled in a manner that prevents mixed-up or intermingled products.

  - Records of all reworking operations are maintained.

  - Rework Protocol (Product Specific)

    - The protocol specifies the manner of reworking a product. The Protocol replicates the usual production process as closely as possible. Checks and tests performed in the usual production process are included in the Protocol. For example, if check-weighing of packed cartons is part of the usual production process, then it is included in the Rework Protocol.

    - An Operations team member or a Quality Personnel prepared the Rework Protocol. The Protocol was checked and approved before execution by the Production and Quality Heads, respectively.

- Responsibility

  - Designated Staff - Quality Personnel, Operations Team

    - Responsible for performing reprocessing and reworking of non-compliant products.

    - Implement process procedure according to SOP.

    - Issue and review monitoring forms and control sheets.

  - All Personnel

    - Shall adhere to this policy.

  - Preventative Control Qualified Individual (PCQI)

    - Visually inspect areas for compliance with this policy.

    - Oversee (Verify) compliance of procedure.

    - Ensure personnel are trained in this policy and procedures.

    - Monitor and take corrective action to ensure compliance with this policy.

  - Supervisors or department heads and team leaders are responsible to:

    - Ensure (inspect) and verify this policy and procedure is adhered to.

    - Take corrective action to ensure compliance with this policy.

  - Quality Department

    - Validate training and adherence to policy and procedure.

    - Monitor and document non-conformance observation, also known as discrepancy, using Corrective Action Preventive Action-CAPA Report.

  - Quality / Compliance Department.

    - Document control, records, and archive.

- Corrective Action

  - In the event of non-compliance or deviation, the following is conducted:

    - Train the key personnel involved in the implementation of the program,

    - Review the procedures for effectiveness and revise if needed.

- Review - Quality

  - Review of the Reprocessing procedure; and update as needed using customer complaints, Total waste totals, and organoleptic testing of the finished product.

  - Warehouse staff isolate damaged products when it is recognized and advise the supervisor or designated employee.

  - Reworking staff ensure all of the appropriate labels are attached to the product when repacking or reworking occurs. Reworking staff maintains records of the damaged product that is reworked on their production report.

- History

| Revision No.: | Revision Date: | Description of Change: | Originator / Author Name: | Title / Department: |
| --- | --- | --- | --- | --- |
| 0 | 20220805 | Original | Arnel Ryan | PCQI / Compliance |
| 1 | 20230929 | - Aligned with the updated program. - Updated the Policy, Procedure, Responsibility, and Comprehension Quiz | Arnel Ryan | PCQI / Compliance |
| 2 | 20240625 | Changed Lesson to Training Material and Comprehension Quiz | Arnel Ryan | PCQI / Compliance |

Requirements:

- The trainee (employee/visitor) has read or received a verbal translation of all or part of the policy, procedure, method, and/or SOP for which they are being trained.

- The trainee has observed a demonstration of the task they are to perform or the procedure for which they are being trained as required.

- The trainee has demonstrated the ability to perform the task with acceptable proficiency and minimal supervision.

| TRAINEE (Print Name): | TRAINEE (Signature): | Date: | Department: |
| --- | --- | --- | --- |
| Trainer / Supervisor (Print Name): | Trainer (Signature): | Date: | Department: |

UPON COMPLETION – EMAIL TO services@consultareinc.com

| NAME (Print): | Date: |
| --- | --- |

Product Rework Comprehension Quiz

Direction: Choose the best answer and encircle the letter.

- What are the colors of containers does non-conforming products segregated by the responsible employees?

- White, Green, Blue

- White, Yellow, Blue

- White, Yellow, Purple

- ______: treatment of product by a process different than what was originally used to produce the product.

  - Recycle

  - Rework

  - Reprocess

- ______: retreatment of the product by the same process to provide acceptable quality and/or food safety.

  - Recycle

  - Rework

  - Reprocess

- ______: treatment of a product through a predetermined redirection of product to a previous step in the process.

    - Recycle

    - Rework

    - Reprocess

- In the event of non-compliance or deviation, what should be conducted?

    - Train the key personnel involved in the implementation of the program.

    - Review the procedures for effectiveness and revise if needed.

    - Both A and B

    - None of the above

UPON COMPLETION – EMAIL TO services@consultareinc.com