Manlifts Training Material and Comprehension Quiz

- Policy

  - Romero’s Food Products Inc., ensures that this program is strictly implemented by the authorized personnel.

  - Romero’s Food Products Inc., is committed to maintaining its employees’ safety by providing proper training and durable PPEs for manlifts.

- Procedure

  - General Requirements

    - Design Requirements

      - Equipment used for manlifts comply with “American National Standard for Manlifts ANSI A90.1-1969.

    - Floor Openings

      - Allowable size – Floor openings for “up” and “down” run within specific width ranges based on belt size; openings extend 24 to 28 inches from the face of the belt.

        - 12-inch belt: 28 to 36 inches

        - 14-inch belt: 34 to 38 inches

        - 16-inch belt: 36 to 40 inches

      - Uniformity – Floor openings on a specific manlift have consistency in size, are approximately circular, and are positioned directly above the opening below.

    - Landing

      - Ensure vertical clearance between the floor or mounting platform and the lower edge of the conical guard has a minimum of 7 feet 6 inches.

      - Make sure landing space adjacent to floor openings is unobstructed, maintained clear at all times, and has a minimum width of 2 feet, which is measured from the edge of the floor opening used for mounting and dismounting.

      - Adequate lighting measures are not less than 5-foot candles.

      - Construct and maintain landing surfaces at the entrances and exits of the manlift to ensure safe footing at all times.

      - Manlifts with a travel distance of 50 feet or more are required to have one or more emergency landings to ensure a landing for every 25 feet or less of manlift travel.

      - Emergency landings are accessible from both the "up" and "down" runs of the manlift.

      - Ensure that emergency landings are fully enclosed.

      - Platforms built to facilitate access to equipment like bucket elevators for inspection, lubrication, and repair can also function as emergency landings.

    - Guards on the underside of floor openings

      - Fixed-type manlifts, on the ascending side, floor openings have a bevel guard or cone that meets the following requirements:

        - A cone that makes an angle of not less than 45° with the horizontal and an angle of 60° or greater for ceiling heights permit.

        - The lower edge extends at least 42 inches outward from any handhold on the belt and does not extend beyond the upper surface of the floor above.

        - Cone made of not less than No. 18 U.S. gauge sheet steel or material of equivalent strength or stiffness, lower edge rolled to a minimum diameter of one-half inch, and the interior is smooth with no rivets, bolts, or screws protruding.

      - Floating-type cones hinged at least 6 inches below the underside of the floor and constructed to activate a limit switch if a force of 2 pounds is applied to the edge of the cone closest to the hinge. The depth of this floating cone is not required to exceed 12 inches

    - Protection of entrances and exits

      - Protect the entrances and exits at all floor landings providing access to the manlift by either a maze (staggered railing) or a handrail equipped with self-closing gates.

      - Rails are standard guardrails with toe boards.

      - Gates used outward and self-closing and the corners of gates are rounded.

      - Ensure maze or staggered openings offer no direct passage between the enclosure and outer floor space.’

      - Entrances at all landings are in the same relative position unless the building layout prevents this arrangement

    - Guards for Opening

      - Safeguard floor opening at each landing on the sides not designated for entrance or exit. This protection can be achieved through the use of a wall, a railing with a toe board, or panels of wire mesh of sufficient strength.

      - Rails or guards have a minimum height of 42 inches on the up-running side and 66 inches on the down-running side.

    - Bottom arrangement

      - Bottom landing, the clear area is no smaller than the area enclosed by the guardrails on the floors above. Additionally, any wall in front of the down-running side of the belt is not less than 48 inches from the face of the belt. This space cannot be encroached upon by stairs or ladders.

      - Install the lower (boot) pulley in a manner that is supported by the lowest landing served. The sides of the pulley support are guarded to prevent contact with the pulley or the steps.

      - The mounting platform supplied in front or to one side of the uprun is at the lowest landing unless the floor level permits meeting the following requirement:

        - The floor or platform is at or above the point where the upper surface of the ascending step completes its turn and assumes a horizontal position.

      - Guard the area on the downside of the manlift to prevent individuals from walking under a descending step

    - Top arrangements

      - Require and maintain the top clearance of at least 11 feet above the top terminal landing from a plane through each face of the belt to a vertical cylindrical plane with a diameter 2 feet greater than the diameter of the floor opening.

      - Pulley Clearance

        - Clearance of at least 5 feet between the center of the head pulley shaft and any ceiling obstruction.

        - Center of the head pulley shaft not less than 6 feet above the top terminal landing.

    - Emergency exit ladder

      - Provide a fixed metal ladder accessible from both the "up" and "down" run of the manlift for the entire travel of the manlift.

    - Superstructure bracing

      - Secure the manlift rails in such a manner as to avoid spreading, vibration, and misalignment.

    - Illumination

      - Ensure both runs of the manlift are illuminated continuously when the lift is in operation. A minimum intensity of 1 foot-candle should be maintained at all points.

      - Illumination of manlift runways is achieved through circuits permanently connected to the building circuits without switches, or by using switches located at each landing.

      - Shield the complete manlift and its driving mechanism from the weather at all times.

  - Mechanical Requirements:

    - Machines

      - Brakes is inherently self-engaging, requiring external power or force for disengagement. These electrically released brakes are applied to the motor shaft for direct-connected units or to the input shaft for belt-driven units and have the capability to stop and hold the manlift, even under a load of 250 pounds on each step on the descending side.

      - Manlift belts are made of hard-woven canvas, rubber-coated canvas, leather, or a material meeting specified strength requirements. The belts also exhibit a coefficient of friction suitable for passing a brake test when used with an adequate tension device.

      - The width of the manlift belt has a minimum of 12 inches for travel up to 100 feet, 14 inches for travel between 100 and 150 feet, and 16 inches for travel exceeding 150 feet.

      - A belt that has become torn while in use on a manlift does not be spliced and put back in service.

    - Platforms or steps

      - Depth ranging from a minimum of 12 inches to a maximum of 14 inches, measured from the belt to the edge of the step or platform.

      - The width of the step or platform is not less than the width of the belt to which it is attached.

      - Distance between steps is equally spaced and not less than 16 feet measured from the upper surface of one step to the upper surface of the next step above it.

      - The surface of the step makes approximately a right angle with the "up" and "down" run of the belt and travels in the approximate horizontal position with the "up" and "down" run of the belt.

      - The upper surfaces of manlift steps either be made of a material with inherent nonslip characteristics or be entirely covered by a securely fastened nonslip tread.

      - Step frames, supports, and guides have sufficient strength to withstand a load of 400 pounds applied at the approximate center of the step.

        - Prevent the disengagement of any step roller.

        - Prevent any appreciable misalignment.

        - Prevent any visible deformation of the steps or its support.

      - Steps on a manlift are always accompanied by a corresponding handhold above or below it.

    - Handholds

      - Provide and install handholds attached to the belt so that they are not less than 4 feet nor more than 4 feet 8 inches above the step tread and located as to be available on the both "up" and "down" runs of the belt.

      - Handhold on a manlift has a grab surface of at least 4½ inches in width and 3 inches in depth, providing 2 inches of clearance from the belt. Fastenings for handholds situated no less than 1 inch from the edge of the belt.

      - Handhold on a manlift is capable of withstanding a load of 300 pounds applied parallel to the run of the belt without sustaining damage.

      - Ensure handholds on a manlift are always accompanied by a corresponding step. If a handhold is removed, whether permanently or temporarily, the corresponding step and handhold for the opposite direction of travel must also be removed before the lift is put back into service.

      - All handholds are of the closed type.

    - Up limit stops

      - Two automatic stop devices are installed on a manlift to cut off power and apply the brake when a loaded step passes the upper terminal landing.

        - One device is a split-rail switch mechanically operated by the step roller, positioned within 6 inches above the top terminal landing. The second device may have various acceptable forms.

      - The reset device is positioned to provide a clear view of both the "up" and "down" runs of the manlift.

      - Ensure the initial limit stop device on a manlift that the lift is stopped before the loaded step reaches a point 24 inches above the top terminal landing.

    - Electrical requirements

      - Switches open the main motor circuit directly.

      - Electronic devices is designed and installed to shut off power to the driving motor in case of failure.

      - In areas with flammable vapors or combustible dust, electrical installations comply.

      - Unless an oil-immersed type, controller contacts carrying the main motor current are copper to carbon or equivalent, except when the circuit is simultaneously broken at two or more points.

    - Emergency Stop

      - Emergency stop is provided.

      - This stop means within easy reach of the ascending and descending runs of the belt.

      - Operations are connected with the control lever or operating mechanism that it will cut off the power and apply the brake when pulled in the direction of travel.

      - Rope is not less than three-eighth inch in diameter.

    - Instruction and Warning Signs

      - Conspicuous and easily readable instruction signs for manlift use is posted at each landing or stenciled on the belt.

      - The instructions read approximately as follows:

        - Face the Belt.

        - Use the Handholds.

        - To Stop—Pull Rope.

      - At the top floor, an illuminated sign with block letters not less than 2 inches in height is displayed, bearing the wording: "TOP FLOOR—GET OFF."

      - The sign is located within easy view of an ascending passenger and positioned not more than 2 feet above the top terminal landing.

      - A red warning light with a minimum 40-watt rating is placed just below the upper landing terminal, positioned to shine in the passenger's face.

      - A conspicuous sign for visitor having the following legend—AUTHORIZED PERSONNEL ONLY—is displayed at each landing

  - Operating Rules

    - Proper use of manlifts. No freight, packaged goods, pipe, lumber, or construction materials of any kind is handled on any manlift.

    - Safety Rules for operating Manlifts

      - Verify That Employees Are Trained & Certified

      - Check the Manlift Inspection Record

      - Use the Proper Safety Gear

      - Inspect Your Surroundings

      - Never Exceed the Weight Limit

      - Follow Correct Shutdown Procedures

  - Periodic Inspection

    - Manlifts undergo inspections by a competent designated person at intervals not exceeding 30 days, with weekly checks on limit switches. If found unsafe, manlifts will not be operated until properly repaired.

    - Periodic inspection is covered but is not limited to the following items:

      - Steps.

      - Step Fastenings.

      - Rails.

      - Rail Supports and Fastenings.

      - Rollers and Slides.

      - Belt and Belt Tension.

      - Handholds and Fastenings.

      - Floor Landings.

      - Guardrails.

      - Lubrication.

      - Limit Switches.

      - Warning Signs and Lights.

      - Illumination.

      - Drive Pulley.

      - Bottom (boot) Pulley and Clearance.

      - Pulley Supports.

      - Motor.

      - Driving Mechanism.

      - Brake.

      - Electrical Switches.

      - Vibration and Misalignment.

      - "Skip" on up or down run when mounting step (indicating worn gears).

    - A certification record for each inspection includes the inspection date, the inspector's signature, and the manlift's serial number or identifier. These records are accessible to the Assistant Secretary of Labor or an authorized representative.

- Responsibility

  - Employer Responsibilities/OSHA Safety Officer

    - Provide a program to guide and help employees keep the servicing and maintenance activity away from injuries and accidents.

    - Provide training and demonstrations to ensure its employees understand the manlift program’s purpose and procedures.

    - Conduct a periodic inspection annually to ensure the standard procedure and requirements are followed.

  - Employee

    - Attend training on how to eliminate hazards in the work area.

    - Follow the procedure and requirements of the standard.

    - Comply with the restrictions and limitations imposed upon them during the manlift.

  - HR Department

    - Organize training and seminars that will strengthen the knowledge and skills of all employees related to safety and health.

    - Ensure that all employees follow the program and its regulations, including conducting essential disciplinary actions against violators and commendations to outstanding employees.

- Corrective Action

  - Review the safety work methods in the program and improve operating procedures if possible.

  - Provide additional hazard awareness training to strengthen employees’ knowledge about hazardous energy.

  - An incident investigation must be done by trained personnel. PPEs must be used in eliminating and controlling discovered hazards.

  - If a hazard cannot be corrected immediately, evacuation procedures must be initiated to protect the employees.

- Review – Quality

  - Review of the requirements for manlift designs.

  - Review of the requirements for manlift machines.

  - Review of machines and equipment inside the workplace.

  - Review of mechanical and electrical requirements.

  - Review of safety protocols when performing service and maintenance.

- History

| Revision No.: | Revision Date: | Description of Change: | Originator / Author Name: | Title / Department: |
| --- | --- | --- | --- | --- |
| 0 | 20241212 | Original | Arnel Ryan | PCQI / Compliance |

Requirements:

- The trainee has read or received a verbal translation of all or part of the policy, procedure, method, and or SOP for which they are being trained.

- The trainee has demonstrated the task they are to perform or the procedure for which they are being trained as required.

- The trainee has demonstrated the ability to perform the task with acceptable proficiency and with minimal supervision as required.

| TRAINEE (Print Name): | TRAINEE (Signature): | Date: | Department: |
| --- | --- | --- | --- |
| Trainer / Supervisor (Print Name): | Trainer (Signature): | Date: | Department: |

UPON COMPLETION – EMAIL TO rfp@consultareinc.com

| NAME (Print): | Date: |
| --- | --- |

# Manlifts Comprehension Quiz

Direction: Choose the best answer and encircle the letter.

- What is the minimum clearance required between the floor or platform and the lower edge of the conical guard on manlift landings?

  - 6 feet 6 inches

  - 7 feet 6 inches

  - 8 feet

- What material must manlift cones on the ascending side be made of?

  - No. 18 U.S. gauge sheet steel or equivalent material

  - Rubber-coated canvas

  - Stainless steel

- Which of the following responsibilities belongs to the Employer/OSHA Safety Officer?

  - Inspect the manlift weekly for worn gears and vibrations

  - Conduct annual inspections to ensure standard procedures are followed

  - Organize training seminars for employees

- What is the required width of manlift belts for travel distances exceeding 150 feet?

  - 12 inches

  - 14 inches

  - 16 inches

- What is the required illumination intensity for conducting manlift operations safely?

  - 1 foot-candle

  - 3 foot-candles

  - 5 foot-candles

UPON COMPLETION – EMAIL TO rfp@consultareinc.com