Detection of Foreign Objects Training Material and Comprehension Quiz

- Policy

  - Specific work instructions are written on monitoring foreign material detection and prevention devices.

  - Metal detectors are validated to ensure they can effectively detect a foreign object within the packaged product that is passed through the device.

  - The facility has a complete, comprehensive, documented risk assessment of all processes and inputs.

  - The facility has established and maintained a supplier approval program that establishes foreign material parameters for incoming ingredients and components. Refer to Approved Supplier Program

  - Documented and thorough inspections of incoming materials are performed to detect foreign material hazards

  - The controls on the proper storage and transport of all ingredients and products are established.

  - Employee training on foreign material hazards for each employee at the facility, as well as suitable awareness training for visitors and contractors are conducted. Refer to Training Program

  - All appropriate policies and procedures to control the risk of wood, metal, glass, plastic, and other known foreign material contaminants are institutionalized.

  - Customer complaint procedure is in place, which also includes periodic trending of complaint data. Refer to Complaint Management Program.

- Procedure

  - Prevention and Detection Methods

    - The frequency of monitoring such devices, the criteria used in monitoring, and the corrective actions to take when foreign materials are discovered or issues are discovered with the effectiveness of the prevention device are defined within the methods.

    - As per Costco requirements, regular and documented inspections of equipment and likely sources of foreign material are conducted, and there is a reporting mechanism in place for employees to report concerns.

    - Good manufacturing practices and pre-requisite programs are maintained to prevent and mitigate foreign material contamination.

  - Sieving

    - Sieves are best placed before raw ingredients or intermediate preparations enter the processing line. However, their efficacy depends on a few factors:

      - Mesh size - wider than the filtered particles by a specific ratio to let the product flow through the sieve, but also small enough to block undesired materials.

      - Shape of foreign objects - spherical particles and regular, compact forms are easier to sieve, while long and narrow objects may or may not remain trapped depending on their position.

      - Size of food particles - with a size below < 100 µm (1/10th of a millimeter) are not suitable for sieving, as they tend to lump together due to adhesive forces, remaining trapped in the mesh.

      - Electrostatic behavior of the bulk - electrically charged particles of foodstuff tends to stick to surfaces but also plastic foreign objects (e.g., whey powder). Other than making sieving less effective, it also poses a risk of dust explosions.

      - Product moisture - with high humidity, particles tend to lump together and clog the sieve.

  - Metal Detectors

    - Metal detectors are strategically located close to the final packaging to ensure that finished products are free from foreign materials.

    - Works with all types of metals, although they are more effective with ferrous metals (like iron) than non-ferrous metals (like brass, copper, zinc, and aluminum or non-magnetic stainless steel), as the latter has less conductivity.

      - Metal detection is not effective with plastic or glass or with quality defects on products.

      - The position of the fragment is also important and works in conjunction with the type of metal.

      - If the foreign object is positioned lengthways, ferrous metals like iron are detected more easily. By contrast, an upright or horizontal position favors non-ferrous metals.

      - Food products with high conductivity also detract from a metal detector's efficacy.

      - Conductivity depends on several factors such as moisture, salt content, texture, composition, temperature, product quantity and dimensions, and packaging materials.

    - As per Costco requirements, all facilities (other than low-risk operations with a documented and accurate risk assessment) are required to have a properly installed and calibrated foreign material detection device for the finished product (metal detector or x-ray).

    - Metal detectors are regularly calibrated; refer to Calibration Program.

  - Detector Challenges

    - To ensure that all detection devices are operating correctly, Costco requires facilities to perform documented detector challenges;

      - During production run at an interval of two hours or less.

      - At the beginning and end of daily production.

      - When changes occur in the production batch.

      - When changes in machine settings occur

      - After downtime for repairs

    - All devices in the facility are operational and calibrated to the proper sensitivity using certified test pieces (Ferrous, Non-Ferrous, and Stainless Steel). Refer to Metal Detector Monitoring Log

    - Challenge materials conformed to the manufacturer’s recommendations (i.e., metal, glass, plastic, wood, etc.) and reflect the known and likely contaminants of the product or production process.

    - Costco requires auditors to witness challenges to foreign material detection devices, regardless of whether these devices are being used for Costco products during the audit period.

      - In facilities with 5 or fewer detection devices, all units are challenged;

      - When the device count exceeds 5 units, Costco expects 5 devices to be challenged in addition to 30% of the remaining devices.

    - All foreign material detection devices have a proper rejection mechanism, i.e., belt stops, air-jet, auto-reject, alarm system, etc.

    - Rejected products are physically controlled and have a restricted access until evaluation is completed by Quality personnel. Refer to Metal Detector Monitoring Log

    - A written procedure is in place to control a product rejected by a foreign material detection device and to segregate that product from general production (this action is documented).

    - Site employees demonstrate appropriate knowledge of foreign material detector procedures and product disposition.

- Responsibility

  - All Personnel

    - Adheres to this policy.

  - Preventive Control Qualified Individual (PCQI)

    - Visually inspect areas for compliance with this policy.

    - Oversees (Verify) compliance of procedure.

    - Ensures personnel is trained on this policy and procedures.

    - Monitors and take corrective action to ensure compliance with this policy.

  - Supervisors or department heads and team leaders are responsible to;

    - Ensures (inspect) and verify this policy and procedure is adhered to and;

    - Take corrective action to ensure compliance with this policy.

  - Quality Personnel

    - Validates training and adherence to policy and procedure.

    - Monitors and documents non-conformance observation also known as discrepancy using Corrective Action Preventive Action Form.

    - Evaluates products and conducts investigation on the rejected by the foreign matter detection device.

    - Documents control, records, and archive.

- Corrective Action

  - Train all personnel on this policy and reference SSOPs.

  - Non-compliance will require re-training or dismissal from job duties or termination of employment.

  - Products affected by the non-conforming or out of specification due to foreign objects:

    - Quality verifies affected products for non-conformances.

    - Segregated and tagged for disposal

  - When a product is rejected, isolate and restrict access until evaluation is completed by the Quality personnel.

  - Conduct an investigation to identify the source of foreign matter contaminant.

- Review – Quality

  - Methods of monitoring, maintenance, or calibration filters and screens.

  - Responsibility is assigned for the monitoring, maintenance, or calibration of filters and screens.

  - Methods are effectively implemented to monitoring, maintenance, or calibration of filters and screens.

  - Methods are documented for the monitoring, maintenance, or calibration of physical contaminant detection devices.

  - Responsibility is assigned for the monitoring, maintenance, or calibration of physical contaminant detection devices.

  - Methods are effectively implemented to monitoring, maintenance, or calibration of physical contaminant detection devices.

  - Physical contaminant detection technology is routinely monitored.

  - Physical contaminant detection technology is validated.

  - Records are maintained of foreign body inspections.

  - Records are not maintained of the validation of foreign-body detection equipment.

- History

| Revision No.: | Revision Date: | Description of Change: | Originator/Author Name: | Title / Department: |
| --- | --- | --- | --- | --- |
| 0 | 20220806 | Original | Arnel Ryan | PCQI / Compliance |
| 1 | 20230911 | - Updated policy and procedure to align with Costco requirements. - Updated responsibility for Quality Personnel. - Updated corrective action. - Updated Comprehension Quiz and added Answer Key | Arnel Ryan | PCQI / Compliance |
| 2 | 20240624 | - Changed Lesson to Training Material and Comprehension Quiz. - Updated email address to services@consultareinc.com. | Arnel Ryan | PCQI / Compliance |

Requirements:

- The trainee (employee/visitor) has read or received a verbal translation of all or part of the policy, procedure, method, and/or SOP for which they are being trained.

- The trainee has observed a demonstration of the task they are to perform or the procedure for which they are being trained as required.

- The trainee has demonstrated the ability to perform the task with acceptable proficiency and minimal supervision.

| TRAINEE (Print Name): | TRAINEE (Signature): | Date: | Department: |
| --- | --- | --- | --- |
| Trainer / Supervisor (Print Name): | Trainer (Signature): | Date: | Department: |

UPON COMPLETION – EMAIL TO services@consultareinc.com

| NAME (Print): | Date: |
| --- | --- |

Detection of Foreign Objects Comprehension Quiz

Direction: Choose the best answer and encircle the letter.

- Which of the following foreign object shapes is the easiest to sieve?

  - Spherical particles and regular

  - Triangular and pointed

  - Irregular and jagged

- How does the electrostatic behavior of food particles affect sieving?

  - It decreases the risk of dust explosions

  - It creates a greater risk of dust explosions

  - It makes the sieve larger in size

- ___________ works with all types of metals, although they are more effective with ferrous metals.

- Magnetic separation

- Metal detectors

- None of the above

- What happens to particles with high humidity in a sieve?

  - They change color

  - They become more adhesive

  - They lump together and clog the sieve

  - None of the above

- When the device count exceeds 5 units, Costco expects 5 devices to be challenged in addition to ______ of the remaining devices.

  - 30%

  - 20%

  - 10%

UPON COMPLETION – EMAIL TO services@consultareinc.com