Control of Foreign Matter Contamination Training Material and Comprehensive Quiz

- Policy

  - Prevention of foreign matter contamination responsibilities and methods are documented, implemented, and communicated to all staff.

  - The plant and equipment are inspected to ensure good condition and that the equipment is free from potential contaminants.

  - In all cases where there is a risk for potential foreign matter contamination, procedures are in place to eliminate or minimize the risk of foreign matters entering the product. The supplier needs to be aware of potential sources of foreign matter contamination; however, customer complaints may indicate the prevalence and priority.

  - Temporary repairs are utilized outside of general processing facilities.

  - Materials that can wear, such as gaskets and rubber impellers, are inspected.

  - Employees are trained to identify and report foreign matter-related incidents.

  - Visitors and contractors follow this policy to prevent foreign matter contamination.

- Procedure

  - Raw Material and Primary Packaging Material Control

    - Include physical hazard parameters for all raw materials and primary packaging specifications.

      - Raw materials, including primary packaging materials, are inspected for physical hazards based on documented and verified inspection procedures.

      - Trend analysis is conducted, and corrective actions are identified.

    - As per Costco requirements, controls for the prevention of foreign matter contamination during storage, handling, and transport, and corrective actions are identified in the following programs;

      - Receipt, Storage and Handling of Goods Program

      - Loading, Transport and Unloading Practices Program

  - Prevention of Glass, Wood, Brittle Plastic and Metal

    - Foreign matters such as glass protocol outline the sources of foreign issues, the methods of control, and the responsibility for acting when foreign matters or glass are detected in the manufacturing environment. Refer to the Glass and Brittle Plastic Control Policy.

    - The protocol includes removing all tools and machine parts from the processing areas when maintenance has been completed, implemented, and supervised.

    - Personnel are encouraged to report all recognized sources of potential contaminants, including potential deterioration of, e.g., metal blades in mixers and other areas where metal/metal wear can cause metal swarf to tear off.

    - Small parts such as bolts, nuts, screws, etc., and small tools are kept in a spill-proof container to ensure they do not end up in any product. Tools and parts are inventoried upon entry and exit into production rooms.

      - Any missing parts or tools must be found, or the product involved shall be deemed adulterated and properly discarded.

      - Any missing parts or tools must be found to ensure they do not end up in any critical equipment areas.

    - Fabricated equipment covers are used wherever possible to prevent potential contamination from nuts, bolts, etc.

    - Temporary repairs are utilized outside of general processing facilities. Using plastic, tape, string, cardboard, or other non-permanent materials to repair or alter the operation or equipment must be avoided.

      - In the event temporary repairs are applied. The area or equipment being repaired is identified, and the repair date is noted.

    - The site includes within its maintenance process control measures to be taken when repairs are needed during the process to protect the product from foreign materials that could impact food safety.

    - Containers, equipment, and other utensils made of glass, porcelain, ceramics, or other like materials are not permitted in any processing or food handling area.

    - Quality control staff must replace all laboratory glass containers with plastic containers if possible and avoid using glass instruments in processing areas.

    - Regular inspections are made to ensure that these areas are glass-free. Staff must adhere to the company’s foreign matter and glass protocol.

    - All overhead lighting must be protected and shielded. Refer to Lightings and Light Fittings.

    - The food safety plan development includes the risk assessment of foreign matter contamination and preventative controls.

    - As per Costco, risk assessments of all processes and inputs are documented.

    - Institute all appropriate policies and procedures to control the risk of wood, metal, glass, plastic, and other known foreign matter contaminants.

    - Wooden pallets management

      - Inspected for broken or missing slats

      - Inspected for missing pieces

      - Wood protruding pieces that could pose a product risk are removed.

    - Knives and cutting instruments are counted, controlled, and kept clean to avoid cross-contamination. Refer to Utility Knife Control Log

    - Materials that are not easily cleanable or prone to creating foreign matter contamination are prohibited from being used in a product zone or area. This includes carpet, wood, cardboard, and others.

  - Foreign Matter Origination

    - External sources include pests, raw materials, and packaging material (e.g., plastic and/or cardboard embedded in the product by the supplier).

    - As per Costco requirements, inspections of incoming materials to detect foreign matter hazards are documented.

    - Internal sources of foreign matter include the building (e.g., rust, insects, and insulation), surface coatings (e.g., flaking paint, damaged render), equipment (e.g., nuts, pins, screws, washers, etc.), metal swarf, glass (e.g., from windows, or utensils) and wood (e.g., from pallets or brooms or other equipment).

    - Clothes and towels are prohibited from being used as cleaning material to prevent contamination.

  - Indicator Control

    - Customer Complaints indicate the prevalence and priority that foreign matter may have contaminated or adulterated the product.

    - Trend analysis for foreign matter contamination-related complaints is documented, and preventive actions are identified.

    - As per Costco requirements, the customer complaint procedure includes periodic training of complaint data. Refer to Customer Complaint Program

    - Metal Detection indicates ferrous, non-ferrous, and stainless-steel foreign matter detection to prevent contamination and adulteration of the product. Refer to Metal Detector Monitoring Log

    - Material detection devices for the finished product are installed and calibrated. Refer to Detection of Foreign Objects Program and Calibration Program.

  - Employee Training

    - The management ensures that the staff are trained and can carry out their responsibilities and duties competently. Refer to Training Program.

    - Foreign matter hazard training is conducted for each employee.

    - Employees are trained and encouraged to report concerns that include specific language for foreign matter contamination.

    - There is an annual training on foreign matter hazards for all employees at the facility, including a reporting mechanism that employees can use for foreign matter-related concerns.

  - Visitor and Contractors

    - Visitors and contractors are oriented on RFP’s operation requirements and foreign matter policy.

    - Visitors and contractors acknowledge RFP’s operation requirements.

- Responsibility

  - All Personnel

    - Shall adhere to this policy.

  - Quality Department/Preventive Control Qualified Individual (PCQI)

    - Visually inspect areas for compliance with this policy.

    - Oversee (Verify) compliance of procedure.

    - Ensure personnel is trained on this policy and procedures.

    - Ensure that visitors and contractors follow this policy.

    - Reviews analyze trends of foreign matter collected or detected during raw material and primary packaging material receiving inspection.

    - Monitor and take corrective action to ensure compliance with this policy.

    - Validate training and adherence to policy and procedure.

    - Monitor and document non-conformance observation, also known as discrepancy, using a Corrective Action Preventive Action (CAPA) Report.

    - Document control, records, and archive.

  - Supervisors or department heads and team leaders are responsible.

    - Ensure (inspect) and verify this policy and procedure are adhered to.

    - Take corrective action to ensure compliance with this policy.

- Corrective Action

  - Train all personnel on this policy and reference SSOPs.

  - Non-compliance will require re-training, dismissal from job duties, or termination of employment.

  - Products affected by the non-conforming or out of specification due to foreign matter:

    - Quality verifies affected products for non-conformances.

    - Segregated and tagged for disposal.

- Review – Quality

  - Foreign matter control procedures are documented.

  - Foreign matter control procedures identify all potential sources of foreign matter contamination.

  - Foreign matter control procedures are effectively implemented.

  - Foreign matter control procedures include responsibility for foreign matter control.

  - Foreign matter control procedures are communicated to staff.

  - Inspections are conducted to prevent foreign matter contamination of the product.

  - Temporary repairs are not used within the processing areas or where food is handled or stored.

  - A glass register has been developed.

  - The glass register is complete and covers all glass located at the site.

  - The glass register includes brittle plastic and other materials.

  - Glass inspections are conducted regularly, including instrument dial covers and thermometers.

  - The wood used in the processing/handling area is well-maintained and clean.

  - There are no loose materials on processing equipment.

  - Knives and cutting instruments are clean and sanitized.

  - Employees are aware of the reporting mechanism related to foreign matter incidents.

  - Foreign matter detected during raw material inspection is recorded, and trends are analyzed.

  - Customer complaints about foreign matters are logged, and corrective actions are identified.

  - Trends for foreign matter-related customer complaints are reviewed and analyzed.

- History

| Revision No.: | Revision Date: | Description of Change: | Originator / Author Name: | Title / Department: |
| --- | --- | --- | --- | --- |
| 0 | 20220806 | Original | Arnel Ryan | PCQI / Compliance |
| 1 | 20230913 | - Aligned with the updated program. - Updated the Policy, Procedure, Responsibility, and Review-Quality to meet Costco requirements. | Arnel Ryan | PCQI / Compliance |
| 2 | 20240612 | Changed Lesson to Training Material and Comprehensive Quiz | Arnel Ryan | PCQI / Compliance |

Requirements:

- The trainee (employee/visitor) has read or received a verbal translation of all or part of the policy, procedure, method, and/or SOP for which they are being trained.

- The trainee has observed a demonstration of the task they are to perform or the procedure for which they are being trained as required.

- The trainee has demonstrated the ability to perform the task with acceptable proficiency and minimal supervision.

| TRAINEE (Print Name): | TRAINEE (Signature): | Date: | Department: |
| --- | --- | --- | --- |
| Trainer / Supervisor (Print Name): | Trainer (Signature): | Date: | Department: |

UPON COMPLETION – EMAIL TO rfp@interlinkiq.com

| NAME (Print): | Date: |
| --- | --- |

# Control of Foreign Matter Contamination Comprehension Quiz

Direction: Choose the best answer and encircle the letter.

- The protocol includes removing all tools and machine parts from the processing areas when maintenance has been completed, implemented, and supervised.

    - True

    - False

- _____________ are encouraged to report all recognized sources of potential contaminants.

A.   Customer

  - Supplier

  - Personnel

- Small parts such as __________________ are kept in a spill-proof container to ensure that they do not end up in any product.

  - Bolts

  - Screws

  - Small tools

  - All of the above

- _____________ staff must replace all laboratory glass containers with plastic containers if possible and avoid using glass instruments in processing areas.

  - Quality Control

  - HR Department

  - Sanitation Department

- __________ cutting instruments are counted, controlled, and kept clean to avoid cross-contamination.

A.   Scissors

  - Knives

  - Wedge

UPON COMPLETION – EMAIL TO rfp@interlinkiq.com