Water Supply Training Material and Comprehension Quiz

- Policy

  - The use of non-potable water is controlled such that:

    - There is no cross-contamination between potable and non-potable water lines;

    - Non-potable water piping and outlets are clearly identified; and

    - Hoses, taps, or other similar sources of possible contamination are designed to prevent backflow or back-siphonage.

  - Potable water, or drinking water, is safe enough to be consumed by personnel or used with a low risk of harm.

  - The availability of sufficient supplies of water both as a processing ingredient and for cleaning purposes is ensured.

  - Water potability tests are conducted regularly based on established frequency by a certified third-party laboratory.

  - To prevent contamination, only potable water is used for processing and sanitation activities..

- Procedure

  - Source of Water Supply

    - Potable water supply at the correct temperature and pressure prescribed by applicable legislation has sufficient capacity for all scheduled production needs and meets cleaning and sanitation requirements (refer to 11.2.5).

    - The water supply is adequate for the operations intended and derived from an adequate source. Any water that contacts food, food-contact surfaces, or food-packaging materials is safe and of adequate sanitary quality. Running water at a suitable temperature, and under pressure, as needed, is provided in all areas where required for the processing of food, for the cleaning of equipment, utensils, and food-packaging materials, or employee sanitary facilities.

    - In most developed countries, sufficient quantities of potable water are delivered to storage and distribution facilities for operational purposes. In some countries, however, and some regions in developed countries, the potability of municipal water cannot be relied on.

    - The potable water source, on-site storage (if applicable), and reticulation are identified.

    - Non-potable water is not connected or refluxed into the potable water system.

  - Emergency Plan/s

    - A documented contingency plan is in place in the event of an incidence where the potable water system is deemed to be contaminated for inappropriate use.

    - Filtration

4.2.2.1. Water will pass through filters of varying compositions (sand, gravel, and charcoal) and pore sizes, to remove dissolved particles, such as dust, parasites, bacteria, viruses, and chemicals.

    - Disinfection

4.2.3.1. It is recommended that the pipes used in the potable water system be disinfected.

4.2.3.2. After filtration, a disinfectant (i.e., chlorine, chloramine) is added to kill any remaining parasites, bacteria, and viruses.

  - Water Supply Testing

    - Potable or drinking water is safe enough to be consumed by humans or used with a low risk of harm.

    - Water potability testing is conducted to verify the cleanliness of the supply, the monitoring activities, and the effectiveness of the treatment measures implemented.

    - Collection, monitoring, and testing of the water source and water used in the Production. Refer to KAV- Water Testing Result Log

    - Internal monitoring and testing of the water quality and microbiology.

    - Testing of non-potable water meets specified quality requirements relevant to the application.

    - Test and sample methods are recognized and approved by the government agency or agencies having jurisdiction over the approval of the water source.

    - Analysis of the sample may be performed for the plant by competent commercial laboratories (e.g., Environmental Protection Agency (EPA) and State-certified laboratories).

    - Records of test results are maintained.

  - Availability of Water Supply

    - The site ensures the availability of sufficient supplies of water for personnel hygiene and cleaning purposes.

  - Storage and Distribution Facilities

    - Even though the water supply comes from the town or regional water supply in which the water is treated, safety tested, and maintained by the local authority, it is required that storage and distribution facilities ensure their water at the point of use and comply with regulations

  - Water in Processing Operation

    - Water used in any processing operation from a microbiologically potable source.

    - A separate and clearly identified line exists for potable and non-potable water.

    - Hoses and water lines are protected against cross-connection or backflow to prevent contamination of potable water.

    - When recirculated or reused water is used for rinsing, washing, or conveying food, testing is conducted to ensure it does not increase the food contamination level.

  - Customer Requirements

    - Sprouts – cGMP Food Processing

4.7.1.1. The facility have a written program that ensures the safety of the water supply if water is used in any fashion (i.e., cleaning, process aid, cooling, ice, or ingredient). Program includes:

4.7.1.1.1. Definition of water sources: (i) City (tested at least annually by independent lab), and (ii) Well (tested at least every 6 months by independent lab).

4.7.1.1.2. If city water is used, the city potable water report shall be available.

4.7.1.1.3. Tests on all water sources yearly, for quantitative non-detect of generic E. coli from a qualified laboratory.

4.7.1.1.4. Potability shall be checked at least annually with a sample taken at the plant location, and proper records maintained. Water tested by independent lab for Total Coliforms and E. coli.

4.7.1.2. The facility have all water lines protected against backflow or backsiphonage with documented proof.

4.7.1.3. The facility have documented proof updated annually.

4.7.1.4. Water reaches effective temperature (120℉) when used for cleaning.

    - Whole Foods

4.7.2.1. The facility demonstrate that the water supply is potable and at a minimum, meet the requirements of the National Primary Drinking Water regulations (40 CFR Part 141).

4.7.2.2. Water that comes in contact with food and food contact surfaces and water that is used for handwashing and cleaning and sanitation must comply with local, national, or internationally recognized potable water microbiological and quality standards.

4.7.2.3. The facility water is chlorinated or otherwise treated to assure potability.

4.7.2.4. Potable water in Ready-to-Eat (RTE) areas should be checked frequently for chlorine levels and must be tested at least monthly for microbiological levels.

4.7.2.5. Vendor facilities using private wells demonstrate potability on a continuing basis and must have an acceptable potability test monthly for coliform and E. coli, as well as annually for heavy metals and nitrates. If water treatment is conducted, a system is in place to prevent further use of the water if the chlorination system fails (i.e. alarm system or automated controls).

4.7.2.6. The facility have an identification system for potable and non-potable water lines and current schematics. Dead ends on potable water lines must be eliminated.

4.7.2.7. An ISO 17025 accredited laboratory document potability testing at least annually. In house sampling is recommended, but potability certificates from municipal water suppliers may be acceptable.

4.7.2.8. The facility conducts an organoleptic evaluation of the water source evaluating for off flavors, off odors, off colors, and turbidity; and the results recorded at the time of sampling.

4.7.2.9. The facility have backflow prevention devices on main water lines entering the plant and on individual water lines and hosed drops within production areas. A trained inspector must verify and document the backflow prevention system annually.

4.7.2.10. The facility have an adequate supply of both hot and cold water for production and sanitation. Hand wash facilities must have an immediate supply of tempered water (90-105°F within 10 seconds).

4.7.2.11. The facility utilize a certified laboratory to test and document water potability at least annually. Potability certificates available from municipal water suppliers are acceptable. If using private well, monthly potability testing for coli form/ E. coli and annual testing for H.M and nitrates are required.

    - Trader Joe’s

4.7.3.1. A documented program for analysis of contact surfaces of food equipment, water is implemented for microbiological and chemical attributes which pose potential concern for spoilage or contamination taking into account the source of water, on site storage and distribution facilities, previous sample history and usage.

- Responsibilities

  - Quality Personnel

    - Perform internal monitoring and testing of the water source and water quality used in the production;

    - Ensure adequate potable water for cleaning requirements;

    - Ensure the availability and effectiveness of the contingency plan for possible potable water contamination.

    - Review and analyze water potability and Escherichia coli test results.

    - Retain records of water test results.

  - Senior Management

    - Ensure that there is secure potable water source used in manufacturing;

    - Ensure available potable water is adequate for operational needs;

    - Ensure (inspect) and verify this policy and procedure is adhered to and;

    - Take corrective action to ensure compliance with this policy.

- Corrective Action

  - In the event, the main water source is deemed contaminated,

    - Temporarily close the water line to prevent misuse;

    - Temporarily change to alternative water source/s.

    - Contaminated water is not used for processing and cleaning unless the water potability test and Escherichia coli are within the safe limits.

  - Use of backup disinfection plants or spot dosing may be used to correct disinfection system failure within the water supply.

  - Apply established procedures in the contingency plan.

- Review – Quality

  - The Water Supply Program is reviewed annually or whenever there are changes.

  - Water used in manufacturing is from a potable source.

  - Potable water availability is adequate for operational needs.

  - Potable water availability is adequate to meet cleaning requirements.

  - Water is free from Escherichia coli before processing and sanitation.

  - Water potability test results from various sites within the facility are reviewed and analyzed.

  - Water used for processing and cleaning activities does not contaminate food products.

  - Hot water is available for cleaning purposes.

  - An emergency or contingency plan is in place and effective for suspected potable water contamination.

- History

| Revision No.: | Revision Date: | Description of Change: | Originator / Author Name: | Title / Department: |
| --- | --- | --- | --- | --- |
| 0 | 20210421 | Original | Arnel Ryan | PCQI |
| 1 | 20240612 | 1. Aligned with the updated program (KAV-QMS-11.5.1.P0-Water Supply Program-20240425.R2-For Approval) based on annual review. 2. Updated the Policy, Procedure, Responsibility, Corrective Action, and Comprehension Quiz | Arnel Ryan | PCQI |

Requirements:

- The trainee has read or received a verbal translation of all or part of the policy, procedure, method, and or SOP for which they are being trained.

- The trainee has demonstrated the task they are to perform or the procedure they are being trained as required.

- The trainee has demonstrated the ability to perform the task with acceptable proficiency and minimal supervision as required.

| TRAINEE (Print Name): | TRAINEE (Signature): | Date: | Department: |
| --- | --- | --- | --- |
| Trainer / Supervisor (Print Name): | Trainer (Signature): | Date: | Department: |

UPON COMPLETION – EMAIL TO services@consultareinc.com

| NAME (Print): | Date: |
| --- | --- |

Water Supply Comprehension Quiz

Direction: Choose the best answer and encircle the letter.

- Non-potable water is connected or refluxed into the potable water system.

- True

- False

- After filtration, a _________ is added to kill any remaining parasites, bacteria, and viruses.

- Disinfectant

- Pesticide

- pH meters

- ___________ is conducted to verify the cleanliness of the supply, the monitoring activities, and the effectiveness of the treatment measures implemented.

  - Internal monitoring

  - Filtration

  - Water potability testing

- The facility have all water lines protected against ____________ with documented proof.

- Backflow or backsiphonage

- Reused water

- Non-potable water

- In the event, the main water source is deemed contaminated, EXCEPT.

- Temporarily close the water line to prevent misuse.

- Temporarily change to alternative water source/s.

- Contaminated water is not used for processing and cleaning unless the water potability test and Escherichia coli are above limits.

Comprehension Quiz – ANSWER KEY

Direction: Choose the best answer and encircle the letter.

- Non-potable water is connected or refluxed into the potable water system.

- True

- False

- After filtration, a _________ is added to kill any remaining parasites, bacteria, and viruses.

- Disinfectant

- Pesticide

- pH meters

- ___________ is conducted to verify the cleanliness of the supply, the monitoring activities, and the effectiveness of the treatment measures implemented.

- Internal monitoring

- Filtration

- Water potability testing

- The facility have all water lines protected against ____________ with documented proof.

- Backflow or backsiphonage

- Reused water

- Non-potable water

- In the event, the main water source is deemed contaminated, EXCEPT.

- Temporarily close the water line to prevent misuse.

- Temporarily change to alternative water source/s.

- Contaminated water is not used for processing and cleaning unless the water potability test and Escherichia coli are above limits.