Premises Location and Approval Lesson

- Policy

  - The premises and their surroundings are kept free of contaminants to the products from the external environment.

  - The structures, instructions, procedures, etc., that verify the control of external environmental conditions and the safety of the process and/or product produced, if applicable, are maintained.

  - Exposed products or materials in the food factories and storage facilities are protected from air-borne contaminants from neighboring facilities. Measures include but are not limited to physical barriers and sealed factories.

  - Plans and specifications submitted to a local authority for approval include:

    - Locality map showing the site with the area;

    - Site plan showing all salient features of the site and a description of adjoining sites, including the location of the premises north compass points, roads, stormwater, wastewater;

    - Floor plans showing the layout of the premises, processing areas, permanent fixtures, and layout of equipment;

    - Details of major items of equipment used in the processing area;

    - A diagram of product/process flow;

      - Specifications generally include details of construction materials, surface finishes (walls, floors, ceilings, etc.), product contact surfaces, essential services, and the number of personnel;

      - Also, refrigeration equipment and operating temperatures of cold storage rooms, storage capacity, and means of loading into and out of cold stores need to be included.

    - All applicable certificates or inspection documents from local, state, federal, or international governing agencies are current and kept on file.

- Procedure

  - Locality map showing the site with the area

    - Refer to Locality Map

    - Site plan showing all salient features of the site and a description of adjoining sites, including the location of the premises north compass points, roads, stormwater, wastewater;

      - Refer to Site Plan

      - Refer to Landfill

      - Refer to Wastewater Treatment

    - Floor plans showing the layout of the premises, processing areas, permanent fixtures, and layout of equipment;

      - Refer to Floor Plans

    - Details of major items of equipment used in the processing area;

      - Refer to Equipment List

    - A diagram of product/process flow;

      - Refer to Product / Process Flow

    - Specifications generally include details of construction materials, surface finishes (walls, floors, ceilings, etc.), product contact surfaces, essential services, and the number of personnel;

      - Plant design meets customer and regulatory food safety requirements

      - The plant layout is such that the exposed product is adequately protected from contamination.

      - Interior floors, walls, and ceilings will be kept clean and in good repair. No standing water.

      - Wall perimeters are uncluttered, and equipment is placed to perform proper cleaning and maintenance. Floor juncture coved.

      - All building lighting is to be covered with shatterproof covers.

      - All refrigeration drip pans drain into the sewer.

      - Floors are well-drained, smooth, and clean.

      - Seams between floors and walls will be sealed.

      - All drains will have traps and covers. Drains are kept free from odors. Objectionable odors, fumes, and vapors shall not be present.

      - Hands-free handwash sinks are installed with adequate hot water, soap, and towels in each processing room. Hand wash and product wash sinks do not drain directly onto the floor.

      - All outside door openings are kept closed.

      - Doors left open have screens.

      - Ceilings, walls, and overhead equipment are non-porous, cleanable, in good repair, and free of loose calking, rust, or holes. Penetrations for pipes, cords, etc., are kept sealed.

      - Overhead structures are maintained clean and protected to prevent product contamination.

      - Outside ventilation shall be screened and/or louvered.

      - Sealing and wall penetrations will be sealed.

      - Ceiling and wall openings not in use will be sealed.

      - All walls, ceilings, and fans will be free of condensation, clean, and in good repair.

      - The insulation material used on pipes will be smooth and easy to clean.

      - All skylights and windows must be tight-fitting or screened. Roof to be uncluttered and free of standing water.

      - Ceiling to wall joints must be sealed.

      - Pipe and line supports must be made of proper materials.

    - Also, refrigeration equipment and operating temperatures of cold storage rooms, storage capacity, and means of loading into and out of cold stores need to be included.

      - Refrigeration equipment and operating temperature of cold storage rooms are designed per manufacturers' specifications and regulatory requirements for food safety.

    - All applicable certificates or inspection documents from local, state, federal, or international governing agencies are current and kept on file.

      - Refer to Records.

- Responsibility

  - Senior Management

    - Identifies any risks that may have an adverse impact on product safety and implement controls for any identified risks.

    - Seek approval of the relevant authority for the construction and ongoing operation of the            premises

  - Quality Department

    - Assess the site environment for any risks that may have an adverse impact on product safety.

- Corrective Action

  - In the event of changes in the local environment, identify any risks that may have an adverse impact on product safety and implement controls for any identified risks.

- Review – QA

  - Investigation of the external environment and surrounding land use to determine risk;

  - Understanding of the supplier to the risk from the external environment;

  - Physical measures in place to manage exterior environmental risks;

  - Procedural measures in place to manage exterior environmental risks;

  - The measures are effective in managing the exterior environmental risk.

- History

| Revision No.: | Revision Date: | Description of Change: | Originator / Author Name: | Title / Department: |
| --- | --- | --- | --- | --- |
| 0 | 20221118 | Original | Arnel Ryan | PCQI / Compliance |

Requirements:

- The trainee (employee/visitor) has read or received a verbal translation of all or part of the policy, procedure, method, and/or SOP for which they are being trained.

- The trainee has observed a demonstration of the task they are to perform or the procedure for which they are being trained as required.

- The trainee has demonstrated the ability to perform the task with acceptable proficiency and minimal supervision.