Detection of Foreign Objects Lesson

- Policy

  - Specific work instructions are written on the monitoring of foreign material detection and prevention devices.

  - Metal detectors are validated to ensure that they can effectively detect a foreign object within the packaged product that is passed through the device.

- Procedure

  - Prevention and Detection Methods

    - The frequency of monitoring such devices, the criteria used in monitoring, the corrective actions to take when foreign materials are discovered, and issues discovered with the effectiveness of the prevention device are defined within the methods.

    - Sieving – Sieves are best placed before raw ingredients, or intermediate preparations enter the processing line. Their efficacy, however, will depend on a few factors.

      - Mesh Size – It must be wider than the filtered particles by a specific ratio to let the product flow through the sieve but also small enough to block undesired materials.

      - The shape of foreign objects – Spherical particles and regular, compact forms are easier to sieve, while long and narrow objects may or not remain trapped depending on their position.

      - Size of Food Particles – Those with a size below < 100 µm (1/10th of a millimeter) will not be suitable for sieving, as they will tend to lump together due to adhesive forces, remaining trapped in the mesh.

      - Electrostatic Behavior of the Bulk – Electrically charged particles of foodstuff tend to stick to surfaces but also plastic foreign objects (e.g., whey powder). Other than making sieving less effective, it also poses a risk of dust explosions.

      - Product Moisture – With high humidity, particles will tend to lump together and clog the sieve.

    - Metal Detectors – In principle, metal detectors work with all types of metals, although they are more effective with ferrous metals (like iron) than non-ferrous metals (like brass, copper, zinc, and aluminum or non-magnetic stainless steels), as the latter has less conductivity.

      - Metal detection will not be effective with plastic or glass or with quality defects on products (for example, broken skin on fruit).

      - The position of the fragment is also important and works in conjunction with the type of metal.

      - If the foreign object is positioned lengthways, ferrous metals like iron will be detected more easily. By contrast, an upright or horizontal position will favor non-ferrous metals.

      - Food products with high conductivity will also detract from a metal detector’s efficacy.

      - Conductivity will depend on several factors, such as moisture, salt content, texture, composition, temperature, product quantity and dimensions, and packaging materials.

- Responsibility

  - All Personnel

    - Shall adhere to this policy.

  - Quality Assurance Manager

    - Visually inspect areas for compliance with this policy.

    - Oversee (Verify) compliance of procedure.

    - Ensure personnel is trained on this policy and procedures.

    - Monitor and take corrective action to ensure compliance with this policy.

  - Supervisors or department heads and team leaders are responsible to;

    - Ensure (inspect) and verify this policy and procedure is adhered to and;

    - Take corrective action to ensure compliance with this policy.

  - Quality Personnel

    - Validate training and adherence to policy and procedure.

    - Monitor and document non-conformance observation, also known as discrepancy, using the Corrective and Preventive Action Form.

    - Document control, records, and archive.

- Corrective Action

  - Train all personnel on this policy and reference SSOPs.

  - Non-compliance will require re-training or dismissal from job duties or termination of employment.

  - Products affected by the non-conforming or out of specification due to foreign objects:

    - Quality verifies affected products for non-conformances.

    - Segregated and tagged for disposal

- Review – Quality Department

  - Methods of monitoring, maintenance, or calibration of filters and screens;

  - Responsibility is assigned for the monitoring, maintenance, or calibration of filters and screens;

  - Methods are effectively implemented for the monitoring, maintenance, or calibration of filters and screens;

  - Methods are documented for the monitoring, maintenance, or calibration of physical contaminant detection devices;

  - Responsibility is assigned for the monitoring, maintenance, or calibration of physical contaminant detection devices;

  - Methods are effectively implemented for the monitoring, maintenance, or calibration of physical contaminant detection devices;

  - Physical contaminant detection technology is routinely monitored;

  - Physical contaminant detection technology is validated;

  - Records are maintained of foreign body inspections;

  - Records are not maintained of the validation of foreign-body detection equipment.

- History

| Revision No.: | Revision Date: | Description of Change: | Originator / Author Name: | Title / Department: |
| --- | --- | --- | --- | --- |
| 0 | 20230116 | Original | Arnel Ryan | PCQI / Compliance |

Requirements:

- The trainee (employee/visitor) has read or received a verbal translation of all or part of the policy, procedure, method, and/or SOP for which they are being trained.

- The trainee has observed a demonstration of the task they are to perform or the procedure for which they are being trained as required.

- The trainee has demonstrated the ability to perform the task with acceptable proficiency and minimal supervision.