Mold Prevention and Remediation Policy Lesson

- Policy

  - Company Name implements guidelines for mold prevention and details procedures to safeguard the health of building occupants and staff involved in mold cleanup and prevention.

- Procedure

  - Mold Prevention

    - Repair plumbing leaks and leaks in the building structure as soon as possible.

    - Look for condensation and wet spots.

    - Fix source(s) of moisture incursion problem(s) as soon as possible.

    - Prevent moisture from condensing by increasing surface temperature or reducing the moisture level in the air (humidity).

    - To increase surface temperature, insulate or increase air circulation.

    - To reduce the moisture level in the air, repair leaks, increase ventilation (if outside air is cold and dry), or dehumidify (if outdoor air is warm and humid).

    - Keep HVAC drip pans clean, flowing properly, and unobstructed.

    - Perform regularly scheduled building/ HVAC inspections and maintenance, including filter changes.

    - Maintain indoor relative humidity below 70% (25 - 60%, if possible).

    - Vent moisture-generating appliances such as dryers to the outside, wherever possible.

    - Vent kitchens (cooking areas) and bathrooms according to local code requirements.

    - Clean and dry wet or damp spots as soon as possible, but no more than 48 hours after discovery.

    - Provide adequate drainage around buildings and slope the ground away from building foundations. Follow all local building codes.

    - Pinpoint areas where leaks have occurred, identify the causes and take preventive action to ensure they do not reoccur.

  - Remediation Plan

    - Determine and repair the conditions that promote mold growth and the elements contributing to mold growth to safely and successfully remove mold-damaged materials.

    - Investigate and resolve the source of the water problem to avoid having to remediate the same site multiple times (mold development in an occupied structure is a sign of a water problem).

    - When water is brought into an indoor environment, the damaged area must be dried as quickly as possible (within 24-48 hours) to prevent mold growth.

    - After the water source, the problem has been located and remedied, and there are numerous options for removing apparent mold growth. Which strategy is best for each situation will be determined by the circumstances?

    - Methods

Damage to building materials and furnishings caused by moisture control issues and mold growth can be remedied using various remediation methods. Depending on the type of material impacted, a specific approach or collection of processes will be utilized. The following are some examples of methods that could be used:

      - Wet vacuum

        - It removes water from floors, carpets, and hard surfaces where water has accumulated.

        - They are used only on wet materials, as spores may be exhausted into the indoor environment if insufficient liquid is present. Since mold and mold spores may adhere to equipment surfaces, these vacuums' tanks, hoses, and attachments are thoroughly cleaned and dried after use.

      - Damp wipe

        - Wipe with plain water or a water/detergent solution to dampen

        - Scrub as needed.

        - Never combine bleach and ammonia.

        - Bleach and ammonia should never be mixed.

        - Toxic gases could be released.

        - Detergent is approved.

      - HEPA vacuum

        - Use a HEPA vacuum on completely dry surfaces.

        - Dispose of HEPA contents in a well-sealed plastic bag.

      - Disposal of Damaged Materials

        - Put contaminated material in a sealed plastic bag and throw it away.

        - After removing the item from the HEPA vacuum area, dispose of the HEPA contents in a well-sealed plastic bag.

    - Personal Protective Equipment (PPE)

Employees working on mold abatement have the following personal protective equipment (PPE) on hand:

      - Safety glasses/goggles

      - N95 Respirator

      - Disposable Coveralls

      - Gloves

    - Work Area Containment

For all mold remediation projects, general isolation will be required. The following are best management practices:

      - Close all doors and restrict public access to the workplace while actual remediation is taking place.

      - If possible, work when the building is least used, such as on nights or weekends.

      - Turn off and/or re-blank off HVAC systems in the immediate area of the work where applicable.

      - Close windows in the workplace.

      - Switched off any portable fans.

    - Disposal

      - Dispose of waste as usual trash once mold-contaminated things have been removed and packed in plastic bags.

  - Mold Remediation Guidelines

The regulations are in place to protect the health of cleanup crews and other workers during the remediation process. These recommendations depend on the size of the mold-infested space.

    - Small Areas of Mold Remediation (i.e., <30ft.2)

      - Ensure that the work area is unoccupied.

      - Wear non-vented goggles, gloves, and protective clothing such as disposable coveralls.

      - Containment of the work area is not necessary.

      - Cover surfaces in the work area that could become contaminated with secured plastic sheets.

      - At a minimum, use a half-face or full-face N, R, or P-95 respirator.

    - Large Areas of Mold Remediation (i.e., 30-100 ft.2)

      - The work area and areas directly adjacent to it is unoccupied.

      - Cover surfaces in the work area and adjacent areas that could become contaminated and seal ventilation ducts/grills with secured plastic sheets.

      - At a minimum, use a half-face or full-face N, R, or P-95 respirator.

      - Wear non-vented goggles, gloves, and protective clothing such as disposable coveralls.

      - If remediation is expected to generate a lot of dust or the visible concentration of mold is heavy, follow the extensive contamination procedures (2.3.3).

    - Extensive and Visible Mold Contamination

      - At a minimum, use a half-face or full-face N, R, or P-100 respirator.

      - Wear non-vented goggles, gloves, and protective clothing such as disposable coveralls.

      - Ensure that protective clothing covers the entire body, including the head and feet.

      - Consult with experienced health and safety professionals prior to this level of remediation.

- Responsibility

  - Top Management

    - Obligated by the Occupational Safety & Health Administration to provide the employees with a workplace that is free from recognized hazards likely to cause death or serious physical harm.

    - Ensure that this space is a clean and healthy environment for all who work there.

- Corrective Action

  - To avoid human exposure and further damage to building materials and furnishings, fix the moisture problem and remove moldy and contaminated materials. Mold can permeate porous things and grow on or fill in empty spaces or cracks; therefore, wet porous items with mold growing on them may have to be destroyed. It may be difficult or impossible to eliminate this mold entirely.

  - As a general rule, simply eliminating the mold with a biocide, for example, is insufficient. The mold must be eliminated since even dead mold has chemicals and proteins that can induce a human reaction.

  - Damage to building materials and furnishings caused by moisture control issues and mold growth can be remedied using various remediation methods. Depending on the type of material impacted, a specific approach or collection of processes will be utilized.

- Review

  - Review of the safety and health program, goals, and objectives and the application of this policy.

  - Review the Occupational Safety and Health Administration about Sanitation and Molds.

- History

| Revision No.: | Revision Date: | Description of Change: | Originator / Author Name: | Title / Department: |
| --- | --- | --- | --- | --- |
| 0 | Enter Revision Date Here | Enter Description of Change Here | ENTER AUTHOR NAME | ENTER TITLE / ENTER DEPARTMENT |

Requirements:

- The trainee has read or received a verbal translation of all or part of the policy, procedure, method, and or SOP for which they are being trained.

- The trainee has demonstrated the task they are to perform or the procedure for which they are being trained as required.

- The trainee has demonstrated the ability to perform the task with acceptable proficiency and with minimal supervision as required.

*Should you need assistance in customizing this document please email services@consultareinc.com