| Document Title: SSOP-Cleaning and Sanitation-Production- Ingredient Preparation Room | Document Control No.: [Enter Document Control No.] | Rev. No.: [Enter Revision No.] |
| --- | --- | --- |
| Department: [Enter Department] | Supersedes: Original | Effective Date: [Enter Date] |
| Prepared by: [Affix Name, Signature, Position Title] | Reviewed by: [Affix Name, Signature, Position Title] | Approved by: [Affix Name, Signature, Position Title] |
| Date: [Enter Date] | Date: [Enter Date] | Date: [Enter Date] |

- Purpose:

  - To ensure that the food-contact and non-food-contact surfaces in the ingredient preparation room are maintained in a clean and sanitary condition to decrease the likelihood of product contamination.

- Frequency:

  - Food contact surfaces, including equipment - Every end of shift

  - [ADD FREQUENCY HERE]

- Tools and Equipment:

  - Polypropylene plastic broom

  - Vacuum cleaner

  - Dustpan

  - Mop and mop squeezer

  - Microfiber cloth and duster

  - Spatula

  - Scrubbing pad

  - Pail/ bucket/ basin

  - Spray bottle

  - Plastic film

  - Personal Protective Equipment (PPE)

    - Apron

    - Rubber gloves – chemical-resistant

    - Eye goggles

    - Facemask

    - Hair net

  - Trash bin and polybag for waste collection

  - Cleaning and sanitation recording tools

    - Ballpen

    - Clipboard

    - Records/ forms

  - [ENTER LIST OF TOOLS AND EQUIPMENT HERE]

- Consumable:

  - Potable supply of water

  - All-purpose liquid detergent

  - Sanitizer

  - Test strips for disinfectant and sanitizer concentration

  - [ENTER LIST OF CONSUMABLES HERE]

- Responsibility:

  - Cleaning crew - responsible for ensuring that the cleaning and sanitation program is followed to maintain the ingredient preparation room in a clean and sanitary condition.

  - Production supervisor - makes sure that the task is completed in accordance with the master cleaning and sanitation plan and this document's instructions;

  - Quality assurance personnel – verifies that the production team is adhering to the cleaning and sanitation program in the ingredient preparation room;

  - Production head/ manager – reviews and approves the revised guidelines, as necessary.

  - [ADD OR CUSTOMIZE THE RESPONSIBLE PERSON HERE]

- Procedure/Instructions:

  - Pre-cleaning and sanitation activity

    - When preparing chemicals and doing cleaning and sanitation tasks, wear the proper personal protection equipment (PPE) for safety against the harsh effects of chemicals.

    - Prepare the cleaning solution and sanitizer by following the manufacturer’s instructions that can be seen on the label.

      - Using the color-changing test strip, check the concentration of the prepared cleaning detergent and sanitizer by dipping the test strip in the solution, and comparing it to the color chart.

      - Frequently check the concentration of the solutions to ensure that their effectiveness is maintained; otherwise, prepare a new solution.

      - Transfer generous amounts of the freshly produced solutions into the basin or bucket or the hand-held spray bottles (for easy-to-reach regions).

    - Prepare the cleaning supplies and tools, such as a mop, vacuum cleaner, dustpan, spatula, and microfiber cloth.

    - Turn off the main power supply of the electrical equipment in the area where the cleaning and sanitation will be conducted.

    - Cover all equipment and other contact surfaces with plastic film before starting the cleaning of the ceiling to protect them from the dust that could fall off while cleaning.

    - Prepare an industrial fan to provide proper ventilation inside the ingredient preparation room to ease the drying of all sanitized surfaces.

  - Cleaning and sanitation of ceilings and light fixtures

    - Using a vacuum cleaner, remove cobwebs, dirt, dust, and debris lodged in the crevices of the ceiling.

    - Use a brush with a long handle or a spatula to scrub the cleaning solution into the thickened dust, filth, and stains on the ceiling.

    - Carefully clean the fluorescent light bulbs using a dry duster to remove dirt, cobwebs, and dust.

    - Use a clean microfiber cloth with a long handle to clean the ceiling. Continue until the suds are completely removed.

    - Spray the sanitizer solution on the ceiling with at least 10-15 minutes of contact time before wiping it with a clean microfiber cloth.

  - Cleaning and sanitation of walls and floors

    - Using a scrubbing pad dipped in the cleaning solution, scrub thickened dust, grime, and stains off of walls.

    - Spray the sanitizer solution on the walls. Let it stand for 10-15 minutes and wipe it using a clean microfiber cloth.

    - Remove sweepings left on the floor using a broom or vacuum cleaner while ensuring that the coving and corners of the wall-floor junction are reached thoroughly.

    - Sanitize the floor using a clean mop dipped in sanitizer in the bucket. Let it stand for at least 15 minutes, then wipe it with a clean mop.

  - Cleaning and sanitation of food-contact surfaces (Scope: sinks, tables, equipment, utensils, thermometers, trays, etc.)

    - Remove the accumulated dirt on the surfaces using a duster or clean microfiber cloth.

    - Spray the cleaning solution on the microfiber cloth and wipe the surfaces completely.

    - Using a wet microfiber cloth, wipe the surfaces until the detergent suds are completely removed.

    - Sanitize the surfaces by spraying the sanitizing solution mixed at a concentration specified on the manufacturer’s label. Let the sanitizer stand in contact with the surfaces for about 10-15 minutes before wiping with a clean cloth.

    - Let it dry.

  - Post-Cleaning Activity

    - Fill a poly bag with all the trash from the rubbish bin and transfer it to the scrap room. Clean the trash can with a dry duster and put it in a fresh trash bag.

    - Re-apply the sanitizing solution on the entire floor and let it dry.

    - Discard any leftover liquid detergent and disinfectant solution down the sink with running water.

    - Return all cleaned cleaning aids to their proper storage location.

    - Ensure clean and sanitized surfaces are dry before returning to working operations.

    - Record the activities in the designated forms provided. Ensure that it is checked and verified timely by the assigned personnel.

- Corrective Action:

  - If the cleaning and sanitizing techniques or procedure were found inadequate for the ingredient preparation room, conduct the following:

    - Conduct re-cleaning of the uncleaned area in accordance with procedures and guidelines stated in this document.

    - Discard the contaminated ingredient properly.

    - Review the existing procedure and make adjustments, either on the tools used, chemical preparation, frequency of cleaning, or revise the procedure itself.

  - If there is negligence on the part of employees, e. g., improper cleaning and sanitation, chemical mishandling, failure to follow the pre-cleaning procedure, failure to follow the frequency of cleaning and sanitation, or failure to monitor and verify the activity, the following must be administered:

    - Conduct coaching to the concerned personnel to demonstrate proper solution preparation up to the proper cleaning procedure.

    - Reiterate the importance of compliance with this procedure or guidelines.

  - If the solution does not reach the appropriate reading, discard it according to its proper waste disposal and prepare a new solution per the manufacturer’s recommendation. If the concentration is still not obtained, proceed with the following:

    - Check the expiration date of the chemicals.

    - Prepare a new solution using unexpired chemicals.

  - In the event that personnel come in contact with hazardous and/or concentrated chemicals, administer the following:

    - Wash the area with running water to remove the chemicals from the eyes or skin.

    - For first aid instructions, refer to the chemical SDS.

    - Inform the first aider, medical staff, and supervisor immediately for further treatment.

    - Retrain the personnel on proper chemical handling.

  - If potential cross-contamination occurred during the cleaning and sanitation activity that may have affected any food items in the ingredient room, conduct microbiological swabbing to ensure no pathogenic or disease-causing bacteria will be present both on the surfaces and the product.

  - Issue a non-conformance and corrective action report for historical tracking and monitoring of the corrective action.

  - [ADD OR CUSTOMIZE CORRECTIVE ACTION HERE]

- Reference Document:

  - Test Strip Color Chart

  - Chemical SDS and Recommended Preparation

  - Disinfection and Sanitation Chemical Preparation Form

  - Ingredient Preparation Room Cleaning and Sanitation Checklist

  - Cleaning of Cleaning tools checklist

  - [ADD OR CUSTOMIZE REFERENCE DOCUMENT HERE]

- History/Changes:

| Revision No.: | Revision Date: | Description of Change: | Originator / Author Name: | Title / Department: |
| --- | --- | --- | --- | --- |
| 0 | YYYYMMDD | Original | ENTER AUTHOR NAME | ENTER TITLE/ENTER DEPARTMENT |

*Should you need assistance in customizing this document, please email services@consultareinc.com