Repairs and Maintenance Lesson

- Policy

  - Maintenance procedures are carefully planned, designed, documented, and implemented to avoid contamination of products, materials, or equipment and ensure that maintenance staff, including contractors, understand the safety implications of maintenance activities.

  - Maintenance of equipment or building structures are completed in a manner that does not pose a risk to the product, materials (including packaging materials), or equipment

  - The maintenance supervisor and the site supervisor are informed if any repairs or maintenance activities pose a potential threat to product safety (e.g., pieces of electrical wire, damaged light fittings, and loose overhead fittings). When possible, maintenance is conducted outside operating times.

  - The maintenance supervisor is notified in the event of any breakage or damage that could expose products, packaging, or equipment to contamination.

  - The maintenance supervisor is notified when work has been completed

  - Plant supervisors and operators ensure appropriate and effective clean-up measures are taken once all maintenance or service contractor activity is completed and before the commencement of plant operations.

  - Supplier staff, maintenance personnel, and service contractors adhere to the correct procedures when completing maintenance on all equipment.

  - Before use, repair equipment is inspected for missing parts (nuts, bolts, springs, etc.).

  - Those responsible for reporting and completing repairs and cleaning the equipment after repairs are specified in maintenance procedures.

  - The use of temporary fasteners such as string, wire, or tape is not permitted.

  - Food-grade lubricant is used in machinery that exists over product lines or food contact surfaces.

  - Food-grade lubricant is used sparingly to avoid contact with the product.

  - Where paint is used on equipment, roofs, walls, or floors, it is in good condition and suitable for use.

  - Paint is not allowed to be used on food contact surfaces.

- Procedure

  - Maintenance activities – Preventive Maintenance Program

    - Preventive Maintenance Procedure - The following activities are included in the Preventive Maintenance Checklist

      - At the beginning of each week, refer to the Preventative Maintenance checklist to schedule the maintenance to be performed for that week.

      - Daily

        - The Pre-operation and Operational reports (filled out by Quality Personnel) will inform maintenance of what tasks need to be performed.

      - Monthly / Bimonthly

        - Tasks listed on the Preventive Maintenance Checklist

      - Quarterly

        - Tasks listed on the Preventive Maintenance Checklist.

      - Annually

        - Tasks listed on the Preventive Maintenance Checklist

        - Refrigeration equipment checked and serviced by an outside contractor (Contractor will perform service and record results on their invoice). All Contractors must sign the Contractor Rules and Confidentiality agreement annually.

        - Inspect rooftop fans and intake filters.

    - Employees are encouraged to report problems they notice to their supervisor or the Quality department. Notification occurs when the potential product risk is evident through maintenance reports in food processing areas.

    - Maintenance issues that are related to maintaining food safety will be treated as “high priority” by the Maintenance personnel. Other requirements to ensure food safety are:

      - All tools are maintained in a sanitary manner, cleaned daily, and removed when the repair is complete by both Maintenance employees and Contractors.

      - GMPs and food safety hygiene are followed by both Maintenance employees and Contractors

      - Maintenance Procedures that pose no risk to product safety and integrity are followed

      - Maintenance of equipment or building structures are completed in a manner that does not pose a risk to the product, materials (including packaging materials), or equipment

      - Temporary fixes such as wire, tape, string, etc., are not permitted.

      - Equipment is cleaned, washed, and inspected by Quality Department before use

      - Paint is suitable for use and NOT used on Food contact surfaces.

    - New equipment, equipment that is not used regularly, and/or equipment that is kept out of the production areas needs to be:

      - Labeled

      - Properly covered

      - Properly cleaned and sanitized BEFORE going back into service

      - The oil and grease that is used on food contact surfaces for plant maintenance and equipment purposes are food grade.

        - Food grade and Non-Food grade grease are labeled and stored separately in a sanitary manner.

    - Once the Preventive Maintenance task has been completed, all relevant information (unusual findings) will be recorded in the work order management system.

    - If there was nothing unusual to report, then simply update the date that the maintenance was.

    - Supplier staff, maintenance personnel, and service contractors adhere to the correct procedures when completing maintenance on all equipment. As part of maintenance procedures, repaired equipment is inspected for missing parts (nuts, bolts, springs, etc.) before use.

    - Reporting and completing repairs and cleaning the equipment after repairs

      - Maintenance staff and service contractor will report completed repairs to the Maintenance Manager, and cleaning of completed equipment is reported to the Sanitation Supervisor.

    - Where machinery that exists over product lines or food contact surfaces requires lubrication, only food-grade lubricant is used.

    - Where paint is used on equipment, roofs, walls, or floors, it is inspected for good condition and suitable for use. Paint is not used on food contact surfaces.

  - Chemicals and Lubricants

    - All chemicals or lubricants are securely stored during periods of non-use in a place separate from all other products in the warehouse.

      - No chemicals or lubricants can be stored on the floor. Chemicals are sealed in a leak-proof container. Any leaking containers are removed and legally disposed of.

      - All containers of chemicals are labeled and used for the intended purpose.

      - All chemicals are listed in the USDA list of proprietary substances, and non-food compounds, or an MSDS is on file listing the safe uses of the chemicals stored.

      - Under this program, report to your supervisor any violation, unsafe or unauthorized use, or storage of chemicals that could directly or indirectly cause the product to become contaminated.

      - All grease cartridges are food grade. For production, open food-grade grease lubricants and guns are stored inside a labeled, clean container. Only food-grade lubricants are to be used in the processing areas. All food-grade lubricant guns will be labeled (engraved) to indicate food use only. No other items are to be stored in a food-grade lubricant bin. Food-grade lubricant guns stored in the maintenance shop are labeled and stored hygienically.

  - Repairs and Maintenance

    - Facility repairs are documented on a work order, with a target date, the work order is signed off after completion and verification of the finished item. These work orders can come from weekly production meetings, weekly maintenance meetings, or facility inspections. After the work order is documented, it goes directly to the maintenance department until the item is completed and signed off.

    - Equipment Repairs & Maintenance and Servicing During Production

      - The preventive maintenance program is documented in the “Weekly Preventative Maintenance lubrication schedule,” verified by Quality personnel once a week and filed in the production records cabinet.

      - Monthly preventative maintenance overview/activities are logged and verified by a Quality personnel person once monthly and filed in the production records cabinet.

      - Temporary repairs adhere to GMPs. A permanent solution must be given within a reasonable time frame.

      - Use the PIP program to create work orders for any facility repairs or other repairs that require a frame time to get the fix.

    - Maintenance tools are sanitized before entering a production room during operation hours; sanitation is documented on the “Daily Operational Sanitation Record” by a Quality personnel staff or trained employee.

- Responsibility

  - All Personnel

    - Shall adhere to this policy.

  - [ENTER POSITION TITLE – Food Safety Team Lead, PCQI or SQF Practitioner].

    - Visually inspect areas for compliance with this policy.

    - Oversee (Verify) compliance of procedure.

    - Ensure personnel is trained on this policy and procedures.

    - Monitor and take corrective action to ensure compliance with this policy.

    - To ensure compliance with this policy.

  - Quality Control Personnel.

    - Validate training and adherence to policy and procedure.

    - Monitor and document non-conformance observation is also known as discrepancy using Corrective Action Preventive Action-CAPA Report.

  - Quality / Compliance Department.

    - Document control, records, and archive.

  - Maintenance Manager

    - Plans and executes the Maintenance plan.

  - Plant supervisors and operators

    - Ensure the appropriate and effective clean-up measures are taken once all maintenance or service contractor activity is completed and before the commencement of plant operations.

- Corrective Action

  - In the event of product, materials, or equipment contamination due to maintenance activities,

    - Train the key personnel involved in the implementation of the program

    - Review the procedures for effectiveness and revise if needed.

- Review – Quality Personnel

  - Annual review of compliance to Repairs and Maintenance requirements by observing the following:

    - There is a planned maintenance schedule;

    - The maintenance schedule includes critical equipment and areas of the site;

    - There are maintenance procedures that include food safety issues;

    - The planned maintenance schedule is being followed;

    - Maintenance procedures afford no risk to product safety and integrity;

    - Maintenance procedures are known by maintenance personnel and contractors;

    - Maintenance procedures are being followed;

    - Maintenance procedures include food safety and hygiene practices;

    - Maintenance staff follow food safety and hygiene practices;

    - Preventative maintenance activities are documented;

    - Plant and equipment failures are documented;

    - The maintenance schedule is adjusted for plant and equipment failures;

    - Operating staff and supervisors are notified when repairs are made/completed;

    - All tools, parts, and debris are removed from repair areas;

    - Sanitation activity occurs after maintenance repair in food processing areas;

    - Notification occurs when the potential risk to the product is evident through maintenance activities or breakdowns;

    - Food grade lubricant is used in food contact zones, conveyors, and on all motors over food contact surfaces;

    - Food grade lubricant is used sparingly and does not come into contact with the food product, materials, or food contact surfaces;

    - Paint is not used on product contact surfaces;

    - Maintenance records are available and complete.

- History

| Revision No.: | Revision Date: | Description of Change: | Originator / Author Name: | Title / Department: |
| --- | --- | --- | --- | --- |
| 0 | 20230126 | Original | Arnel Ryan | PCQI / Compliance |

Requirements:

- The trainee (employee/visitor) has read or received a verbal translation of all or part of the policy, procedure, method, and/or SOP for which they are being trained.

- The trainee has observed a demonstration of the task they are to perform or the procedure for which they are being trained as required.

- The trainee has demonstrated the ability to perform the task with acceptable proficiency and minimal supervision.