Cleaning and Sanitation-Equipment and Utensils Training Material and Comprehensive Quiz

- Policy

  - All equipment, utensils, and other contact surfaces used to manufacture, package, label, or hold components or dietary supplements must be maintained, cleaned, and sanitized as needed.

  - Establish and keep documentation of the method of cleaning and sanitizing of equipment, utensils, and other contact surfaces.

- Procedure

  - Definitions - Cleaning and Sanitation

    - Cleaning - thorough removal of dirt with the use of suitable detergent chemicals under recommended conditions.

    - Sanitize refers to the reduction of microorganisms to levels that are deemed safe from the perspective of public health.

      - The following are some common sanitization methods:

        - Thermal Sanitization – employs the use of hot water or steam for a predetermined period of time and temperature.

        - Chemical Sanitization – requires using a known chemical sanitizer at a predetermined concentration and contact period.

  - General Cleaning and Sanitation Steps

    - Step 1: Preparation – Remove loose dirt and food particles before applying a cleaning chemical

    - Step 2: Cleaning – Wash with hot water (140ºF/60ºC) and appropriate detergent. This removes the remaining soil.

    - Step 3: Sanitizing – This is the bacteria-killing step. It may be steam or extremely hot water, or a chemical sanitizer.

    - Step 4: Air Drying – Keep it dry.

  - Sanitation Standard Operating Procedure

    - Sanitation Standard Operating Procedure for each equipment and utensil is developed.

      - List all the areas and equipment to be cleaned. Refer to Area and Equipment Master Cleaning List;

      - A full description of the cleaning and sanitation procedures for each piece of equipment and utensils, which will include the four steps indicated in section 4.2;

        - Physically remove solid particles by sweeping or wiping.

        - Pre-rinse with water

        - Apply a suitable detergent in the correct concentration to remove grease and other residues.

        - Rinse off residues and detergent.

        - Apply a suitable sanitizer in the correct concentration to reduce or eliminate microbiological contaminants if required.

        - Rinse to remove residual sanitizer, if indicated on the product label.

        - Dry, as indicated, in a manner that will prevent recontamination.

      - Ensure no cross-contamination with other product contact surfaces occurs;

      - Operators involved in cleaning, including contract cleaners, are fully trained in cleaning and sanitation procedures;

      - Chemicals are approved for use by the appropriate authority. The Safety Data Sheets (SDS) for each chemical used is on file;

      - Chemical cleaners and sanitizers are used and stored only in an approved manner;

      - A description of the chemicals used, their dilution rate, and their method of application. Chemicals and sanitizers used at the site that are mixed to proper concentration levels are mixed according to the manufacturer’s directions;

      - Mixed chemical solutions are stored in appropriate containers that are labeled and identified;

      - Mix concentration is checked and results recorded;

      - An inventory is kept of chemicals purchased and used;

  - Frequency of Cleaning

    - Cleaning and sanitizing of utensils and equipment is conducted in a manner that protects against contamination. Clean all equipment and utensils after use. Refer to Equipment and Utensils SSOP

    - Each process line has a clearly established cleaning schedule (such as daily, after production runs, or more often if necessary). The method of cleaning needed was also determined (such as deep cleaning, etc.).

      - The maximum cleaning interval for equipment food-contact surfaces and utensils in contact with potentially hazardous food is at least every four hours in a day.

      - Utensils and equipment are used to prepare food in a refrigerated room or area that is kept at one of the temperatures specified in the chart below.

| Temperature | Cleaning Frequency (Hours) |
| --- | --- |
| 5.0° C (41° F) or less | 24 |
| > 5.0° C and ≤ 7.2° C | 20 |
| (> 41° F and ≤ 45° F) |  |
| > 7.2° C and ≤ 10° C | 16 |
| (> 45° F and ≤ 50° F) |  |
| > 10° C and ≤ 12.8° C | 10 |
| (> 50° F and ≤ 55° F) |  |

Table 1. Cleaning frequency corresponds to the temperature.

      - In-use utensils are stored intermittently in a container of water that is kept at 58°C (135°F) or above, and the utensils and container are cleaned at least every 24 hours or at a frequency necessary to prevent soil residue formation.

      - The following is the recommended cleaning interval for equipment food-contact surfaces and utensils in contact with food that is not potentially hazardous:

        - Whenever there is a chance of contamination;

        - At the manufacturer's specified frequency.

        - In the absence of manufacturer specifications, at a frequency sufficient to prevent soil or mold accumulation.

      - Non-food-contact equipment surfaces are cleaned on a regular basis to avoid the formation of soil residues.

    - Evaluation and inspection procedures are used to determine whether cleaning/sanitizing methods are adequate. Compliance with approved written processes (inspection, swab testing, and direct observation of personnel) is regularly checked, and records are kept monitoring long-term compliance.

  - Methods of Cleaning

    - Dry Cleaning

      - Dry cleaning procedures such as brushing, scraping, and vacuuming should only be used on surfaces that are dirty with non-hazardous dry food residues.

      - Cleaning equipment used for dry cleaning food-contact surfaces may not be utilized for any other purpose.

    - Precleaning

      - Food residue on utensils and equipment is scraped over a waste disposal system or garbage bin, or it can be removed in a ware-washing machine using a prewash cycle.

      - Utensils and equipment are pre-flushed, presoaked, or scrubbed with abrasives as needed for proper cleaning.

    - Wet cleaning

      - Equipment with food-contact surfaces and utensils are properly cleaned to remove or totally loosen soils using the necessary manual or mechanical methods, such as the use of detergents including wetting agents and emulsifiers, acid, alkaline, or abrasive cleaners; hot water, brushes, scouring pads, high-pressure sprays, or ultrasonic devices.

      - The equipment or utensil's type, purpose, and the sort of soil to be removed all have a role in the washing procedures that are selected.

  - Control and Monitoring

    - All equipment is installed and maintained to facilitate the cleaning of the equipment and of all surrounding areas.

    - Clean and sanitized equipment, utensils, and single-use containers are stored in a location that is free of splashes, dust, and other contaminants.

    - Any employees who come into direct contact with raw materials, in-process or finished dietary items, processing equipment, utensils, or packaging materials while on duty, conform to the hygienic procedures, as necessary.

- Responsibility

  - All Personnel

    - Shall adhere to this policy.

  - Sanitation Inspector and Supervisor

    - Implement proper cleaning procedure as outlined in this program.

    - Verifies the effectiveness of sanitation.

  - cGMP Team Lead

    - Visually inspect areas for compliance with this policy.

    - Oversee (Verify) compliance of procedure.

    - Ensure personnel are trained in this policy and procedures.

    - Monitor and take corrective action to ensure compliance with this policy.

  - Supervisors or department heads and team leaders are responsible to;

    - Ensure (inspect) and verify this policy and procedure is adhered to and;

    - Take corrective action to ensure compliance with this policy.

  - QC Personnel and Supervisor

    - Validate training and adherence to policy and procedure.

    - Monitor and document non-conformance observation, also known as discrepancy, using Corrective Action Preventive Action-CAPA Report.

    - Document control, records, and archive.

- Corrective Action

  - Train all personnel in this policy and reference SSOPs.

  - Non-compliance will require re-training or dismissal from job duties or termination of employment.

- Review – Quality

  - Annual review of compliance with Cleaning and Sanitation Program requirements by observing the following:

    - The site has an effective and appropriate cleaning program in place;

    - All critical equipment and utensils are covered in the cleaning program;

    - Cleaning methods include what is to be cleaned, how it is to be cleaned, frequency of cleaning, and responsibility for cleaning;

    - The cleaning and sanitation program includes measures for verification of the effectiveness of sanitation;

    - The cleaning of manufacturing equipment and equipment is effective;

    - Personnel conducting pre-operational inspections are trained and qualified;

    - A sanitation verification schedule is available;

    - An inventory of purchased chemicals is available and is current;

    - Detergents and sanitizers meet local regulatory requirements;

    - SDS sheets are available for all cleaning chemicals purchased;

    - Personnel handling cleaning chemicals are properly trained;

    - Cleaning chemicals are disposed of as per regulatory requirements;

    - Empty cleaning chemical containers are labeled and securely stored;

    - Mixed concentration levels are to be mixed to manufacturer instructions, tested for verification, and recorded; and

    - Records of cleaning and sanitation activities are maintained and complete.

- History

| Revision No.: | Revision Date: | Description of Change: | Originator/Author Name: | Title / Department: |
| --- | --- | --- | --- | --- |
| 0 | YYYYMMDD | Original | ENTER AUTHOR NAME | ENTER TITLE / ENTER DEPARTMENT |

Requirements:

- The trainee (employee/visitor) has read or received a verbal translation of all or part of the policy, procedure, method, and/or SOP for which they are being trained.

- The trainee has observed a demonstration of the task they are to perform or the procedure for which they are being trained as required.

- The trainee has demonstrated the ability to perform the task with acceptable proficiency and minimal supervision.

| TRAINEE (Print Name): | TRAINEE (Signature): | Date: | Department: |
| --- | --- | --- | --- |
| Trainer / Supervisor (Print Name): | Trainer (Signature): | Date: | Department: |

*Should you need assistance in customizing this document please email shop@consultareinc.com