Maintenance, Safeguards, and Operational Features for Exit Routes Training Material and Comprehension Quiz

- Policy

  - The management secures the safety of employees by minimizing dangers on exit routes.

  - The management ensures that adequate and appropriate lighting and markings along exit routes are provided.

  - Fire retardant properties of paints or solutions are renewed and maintained.

  - The management sees to it that exit routes are maintained during construction, repair, and alterations.

  - The management makes sure that employee alarm systems are functional.

  - Review and inspection are performed during construction, repair, and alterations.

- Procedure

  - Exit Route Maintenance

    - Keep away any explosives, highly flammable furnishings, or other decorations that are blocking the exit routes.

    - Ensure that exit routes are arranged so employees do not have to travel toward a danger zone.

    - Ensure that exit routes are free and unobstructed and that no materials or equipment are placed, either permanently or temporarily, within the exit route. Stairs or a ramp are provided where the exit route is not substantially level.

    - Make sure sprinkler systems, alarm systems, fire doors, and exit lighting are always functional.

  - Lightings and Markings

    - Ensure that lights in exit and exit routes are adequately functional.

    - Ensure that each exit has a clear, readable, and visible sign “Exit”.

    - Ensure that the exit route door is free of decorations or signs that obscure the visibility of the exit route door.

    - If the exit path is unclear, post signs indicating the nearest exit and exit discharge. The exit sign always remains visible.

    - Each doorway or passage that could be mistaken for an exit along exit access is marked with a sign indicating its actual use or be labeled "Not an Exit".

    - Every exit sign is lit up with a reliable light source to a surface value of at least five-foot candles (54 lux) and is easily identifiable by its distinctive color. Self-luminous or electroluminescent signs are allowed only if they have a minimum luminance surface value of at least 0.06 foot-lamberts (0.21 cd/m2).

    - It is required that every exit sign have the word "Exit" written in legible letters, which is not less than six inches (15.2 cm) in height. The main strokes of the letters in the word "Exit" are not less than three-fourths of an inch (1.9 cm) wide.

  - Fire retardant properties of paints or solutions

    - Regularly renew fire-retardant paints or solutions to maintain their fire-retardant properties.

  - Exit route maintenance during construction, repairs, or alterations.

    - Ensure that employees do not occupy a workplace during new construction until the exit routes are completed and ready for employees to use for the portion of the workplace they occupy.

    - Ensure that employees do not occupy a workplace during repairs or alterations, unless the exit routes are available and existing fire protections are maintained, or until alternate fire protection is furnished that provides an equivalent level of safety.

    - Assure employees are not exposed to hazards of flammable or explosive substances or equipment used during construction, repairs, or alterations, that are beyond the normal permissible conditions in the workplace, or that would impede exiting the workplace.

  - Employee alarm system

    - Provide alarm systems that are functional and enough to warn employees of fire or other emergencies.

    - Ensure that a functional employee alarm system is installed and maintained.

- Responsibility

  - Management

    - Arrange exit routes so employees will not be exposed to high-hazard areas while traveling.

    - Provide and design safeguards that will protect employees when an emergency arises (e.g., sprinkler systems, alarm systems, fire doors, exit lightings).

    - Provide emergency alarms to alert employees, unless employees can promptly see or smell a fire or other hazard in time to provide adequate warning to them

  - Quality Department

    - Install Exit signs that are readable.

    - Post signs along the exit access indicating the direction of travel to the nearest exit.

  - Safety Team

    - Conduct periodic inspections during exit route maintenance.

    - Help in maintaining workplace exit routes in good working order.

    - Keep exit routes free of explosives or highly flammable furnishings.

    - Keep away from all obstructions that block the exit route.

  - Employee

    - All employees should follow the employee alarm systems.

- Corrective Action

  - Review the general criteria for maintaining exit routes.

  - Keep exit routes always clear.

  - Report any obstructions of exit routes to the safety committee.

  - Report any faulty safeguards to any safety committee.

  - Lighting and markings shall always be checked.

  - Additional analysis and evaluation are done to ensure safe and orderly exit routes.

- Review - Quality

  - Review of the basic requirements in exit route maintenance.

  - Review of exit route safeguards and operational features.

  - Review of lighting and markings provided in each exit route.

  - Review of employee alarm systems in the workplace.

- History

| Revision No.: | Revision Date: | Description of Change: | Originator / Author Name: | Title / Department: |
| --- | --- | --- | --- | --- |
| 0 | 20240115 | Original | Arnel Ryan | PCQI/Compliance |
| 1 | 20241127 | - Change the title part from Comprehensive to Comprehension - Changed the email address from rfp@interlinkiq.com to rfp@consultareinc.com. - Added title to the Comprehension Quiz | Arnel Ryan | PCQI/Compliance |

Requirements:

- The trainee has read or received a verbal translation of all or part of the policy, procedure, method, and/or SOP for which they are being trained.

- The trainee has observed a demonstration of the task they are to perform or the procedure for which they are being trained as required.

- The trainee has demonstrated the ability to perform the task with acceptable proficiency and with minimal supervision as required.

| TRAINEE (Print Name): | TRAINEE (Signature): | Date: | Department: |
| --- | --- | --- | --- |
| Trainer / Supervisor (Print Name): | Trainer (Signature): | Date: | Department: |

UPON COMPLETION – EMAIL TO rfp@consultareinc.com

| NAME (Print): | Date: |
| --- | --- |

Maintenance, Safeguards, and Operational Features for Exit Routes Comprehension Quiz

Direction: Choose the best answer and encircle the letter.

- This is used to warn employees of fire or other emergencies.

- Lights

- Alarm system

- Markings

- Who minimizes exit route danger?

- HR Department

- Management

- Administrative Department

- Providing adequate and appropriate lighting and markings in the exit route is mandatory.

- True

- False

- Listed below are examples of safeguards in the exit route except one.

- Sprinkler system

- Alarm system

- Personal Protective Equipment

- Exit routes shall be maintained during construction, repairs, and alterations.

- True

- False

UPON COMPLETION – EMAIL TO rfp@consultareinc.com