Lighting and Light Fittings Lesson

- Policy

  - Adequate light intensity is provided from receipt, processing and inspection tasks, storage, cleaning, to dispatch operations where the product is exposed and handled.

  - Light fittings in food processing and handling areas are fitted with protective covers or have shatterproof lights installed to minimize risk of breakage and prevent contamination.

- Procedure

  - Lightings and light fittings

    - Provide adequate lighting in hand-washing areas, dressing and locker rooms, and toilet rooms and in all areas where food is examined, processed, or stored and where equipment or utensils are cleaned.

      - Ensure that lighting provides minimum lux (foot candle) intensity as prescribed by applicable legislation or in their absence, meet Good Manufacturing Best Practices appropriate to the commodity being processed.

        - In general, processing and food handling areas are illuminated to a minimum intensity of 200 lux (18.58 ft.c.).

        - Inspection areas require higher illumination; 500 lux (46.45 ft.c.) is generally recommended.

        - At least 540 lux (50 ft.c.) at a surface where a food employee is working with food or working with utensils or equipment such as knives, slicers grinders, or saws where employee safety is a factor.

    - Provide safety-type light bulbs, fixtures, skylights, or other glass suspended over exposed food in any step of preparation or otherwise protect against food contamination in case of glass breakage.

      - Install shatterproof lights or light fittings with protective covers, in food processing and handling areas.

      - Keep documentation on file and which includes specifications from the manufacturer with a description of the product.

      - Recess the light into the ceiling, where possible, or have it fits flush to the ceiling. Where light fittings are not able to be recessed, protect from accidental damage.

      - In circumstances where light fittings are suspended from cables, the top of the fitting is sloped at an angle that permits easy cleaning.

      - Include exposed light fittings in the cleaning and sanitation schedule.

  - Lighting Preventive Measures

    - For the safety and comfort of employees, all general lights are maintained at 30 ft candles. Inspection areas will be held at 50-foot candle.

    - The lighting fixtures are adequately protected from breakage and possible contamination.

    - All glass lighting is wholly enclosed in shatterproof materials to prevent glass contamination of the product. This includes receiving, packaging, operating, warehouse, and shipping areas.

    - Light fixtures are maintained clean and free of cracks, dust, or other materials that could cause contamination.

    - Verification is conducted monthly, followed by the PIP until the item is corrected.

- Responsibility

  - Preventative Control Qualified Individual (PCQI)

    - Visually inspect areas for compliance with this policy.

    - Oversee (Verify) compliance of procedure.

    - Ensure personnel is trained on this policy and procedures.

    - Monitor and take corrective action to ensure compliance with this policy.

  - Supervisors or department heads and team leaders are responsible to;

    - Ensure (inspect) and verify this policy and procedure is adhered to and;

    - Take corrective action to ensure compliance with this policy.

  - Quality Department

    - Validate training and adherence to policy and procedure.

    - Monitor and document non-conformance observation is also known as discrepancy using Corrective Action Preventive Action-CAPA Report.

    - Document control, records, and archive.

  - Maintenance

    - Ensures compliance to the required standards when installing light fixtures.

- Corrective Action

  - Train all personnel on this policy and reference SSOPs

  - Non-compliance will require re-training and performance evaluation.

- Review – Quality

  - Annual review of compliance through observation of the following:

    - Lighting intensity is sufficient in food processing areas;

    - Lighting intensity is sufficient at inspection stations;

    - Lighting intensity is sufficient in warehousing and storage areas;

    - Light fixtures are shatterproof or protected, and pose no threat to product safety;

    - Light fittings are intact – there is no sign of breakage;

    - Light fittings are clean and part of a regular cleaning regime.

6.   History

| Revision No.: | Revision Date: | Description of Change: | Originator / Author Name: | Title / Department: |
| --- | --- | --- | --- | --- |
| 0 | 20220328 | Original | Arnel Ryan | PCQI / Compliance |
| 1 | 20231201 | - Aligned with the updated program (KAV-QMS-11.1.2.P0-Building Materials Program-20231013.R2-For Approval) based on annual review. - Updated the Policy, Procedure, and Comprehension Quiz | Arnel Ryan | PCQI / Compliance |

Requirements:

- The trainee has read or received a verbal translation of all or part of the policy, procedure, method, and or SOP for which they are being trained.

- The trainee has demonstrated the task they are to perform or the procedure for which they are being trained as required.

- The trainee has demonstrated the ability to perform the task with acceptable proficiency and with minimal supervision as required.