FSVP Hazard Analysis Program Lesson

- Policy

  - The company has a hazard analysis program.

  - The qualified individual of the company conducts the hazard analysis.

- Procedures

  - Hazard Analysis of Critical Control Points Principle

    - Conduct a hazard analysis.

    - Determine the critical control points (CCPs).

    - Establish critical limits.

    - Establish monitoring procedures.

    - Establish corrective actions.

    - Establish verification procedures.

    - Establish record-keeping and documentation procedures.

  - Hazard Identification

    - The analysis of the known or reasonably foreseeable hazards in each food is included in the following types of hazards:

      - Biological hazards, including microbiological hazards such as parasites, environmental pathogens, and other pathogens. Please see the reference guide for common sources of Biological Hazards:

| Primary Source | Bacteria | Parasites | Viruses |
| --- | --- | --- | --- |
| Ingredient-related (e.g., contamination of raw materials and other ingredients) | - Salmonella spp. (e.g., poultry, produce, nuts) - E. coli O157:H7 & similar STEC (e.g., ruminant animals, dropped fruit, sprouts) • Campylobacter spp. (e.g., poultry and raw milk) • B. cereus (e.g., rice and other grains) • C. botulinum (spores may be found in soil and on certain root crops.) • C. perfringens (e.g., spices, may come in soil on produce) • L. monocytogenes (e.g., raw agricultural commodities, other contaminated products used as ingredients) | - Cryptosporidium parvum (contaminated water used as an ingredient) - Cyclospora cayetanensis (berries) - Toxoplasma gondii (meat) | - Norovirus (produce, shellfish) - Hepatitis A virus (produce, fruits) |
| Process-related (e.g., poor or ineffective process controls, including by a supplier) | - Salmonella spp. survive inadequate heat treatment - C. perfringens (improperly cooled cooked foods) - L. monocytogenes (raw agricultural commodities, contaminated products) | - Cryptosporidium parvum (contaminated water source) | N/A |
| Facility-related (may be caused by poor sanitation practices (e.g., inadequate cleaning and sanitizing of potential harbourage sites), poor plant and equipment design, and poor pest management practices) | - L. monocytogenes (e.g., reservoirs include floors, cold wet areas, equipment, drains, condensate, coolers, and soil) - Salmonella spp. (pests) | N/A | - Norovirus (only when active shedding occurs in the facility through vomiting and diarrhea) |
| People-related (individuals who are carriers, showing no signs of disease, who are shedding the hazard, or who are infected and are actively ill) | - S. aureus - Shigella spp. - Salmonella spp. | - Cryptosporidium parvum | - Hepatitis A virus - Norovirus - Rotavirus |

      - Chemical hazards, including radiological hazards, pesticide and drug residues, natural toxins, decomposition, unapproved food or color additives, food allergens, and (in animal food) nutrient deficiencies or toxicities. Please see the reference guide for common sources of Chemical Hazards.

| Source | Example |
| --- | --- |
| Ingredient-related chemical hazards | - Pesticide residues on produce raw agricultural commodities - Drug residues in milk - Heavy metals in or on produce raw agricultural commodities (e.g., lead, cadmium, arsenic, and mercury) - Environmental contaminants (e.g., dioxins) - Mycotoxins in grains - Histamine in some aged cheeses - Radiological hazards in foods from areas after a nuclear accident - Unapproved food or color additives - Food allergens and substances associated with a food intolerance or food disorder (e.g., sulfites, gluten) |
| Process-related chemical hazards | - Undeclared food allergens due to mislabelling or cross-contact - Improper addition of substances associated with food intolerance (e.g., sulfites) - Improper use of a color additive such as Yellow No. 5 - Contamination with industrial chemicals such as cleaners or sanitizers - Radiological hazards from the use of contaminated water supply |
| Facility-related chemical hazards | - Heavy metals due to leaching from equipment, containers, or utensils |

        - Hazard Analysis for food packaging and labeling- Common critical control point is an allergen. When packaging is labeled or decorated (which may include food ingredients statements) it is critical for the organization to have controls in place to use the most current version of the ingredients statement.

        - Chemical hazards that can be either ingredient-related or process-related - to prevent life-threatening reactions, food allergens are avoided by allergic consumers. Undeclared food allergens are chemical hazards that can get into food because either:

          - The food manufacturer did not properly declare a food allergen ingredient on the product label.

          - Unintended (and, thus, undeclared) food allergens are present in food due to incorrect labeling or due to allergen cross-contact.

        - Undeclared food allergen hazards due to incorrect label design- Food Allergen Labeling and Consumer Protection Act of 2004 (FALCPA) also amended section 403 of the FD&C Act (21 U.S.C. 343) to prescribe certain requirements for what you must declare on the product label for any food product that contains any of the “big eight allergens” (e.g., crustacea, egg, fish, milk, peanut, soy, tree nuts, and wheat)

          - An undeclared food allergen (including a food allergen contained in flavorings, colorings, and incidental additives) due to an incorrect label design that does not address all the labelling requirements of FALCPA is a chemical hazard.

        - Undeclared food allergen hazards due to incorrect application or use of a product label- If you apply the wrong label to food, or use the wrong packaging (e.g., using packaging for “chocolate ice cream” rather than for “chocolate ice cream with almonds”), consumers who have a food allergy could purchase food that causes an allergic reaction. An undeclared food allergen due to applying the incorrect food label to a product, or using the wrong packaging, is a chemical hazard.

        - Undeclared food allergen hazards due to allergen cross-contact- The unintentional incorporation of undeclared allergens into foods can cause cross-contact. Cross-contact can occur either between foods that contain different food allergens or between foods with and without food allergens. Introduction of an allergen through cross-contact may occur during receiving, handling, processing and storage of ingredients and foods, utensils, and packaging; through improper handling and cleaning of equipment, utensils, and facilities; and through improper facility design. An undeclared food allergen due to allergen cross-contact is a chemical hazard. Allergen cross-contact can result from:

          - Failure to schedule the production of two different products appropriately, resulting in an allergen-containing product contaminating a product without food allergens.

          - Failure to adequately clean between two different formulations of a product that do and do not contain allergens, resulting in an allergen-containing product contaminating a product without the allergen.

          - Failure to store allergen-containing ingredients separately from ingredients that do not contain allergens, where leakage of allergen-containing materials results in contamination of the non-allergen-containing product.

          - Failure to handle powdered allergens in a way that prevents particles from blowing onto foods or food contact surfaces for foods that do not contain that allergen.

        - Chemical hazards due to incorrect labeling of substances associated with food intolerance or food disorder- Although the mechanisms whereby persons experience food intolerance or food disorder are different from the mechanisms that cause food allergy, reactions due to food intolerance or food disorder can cause significant health problems for those affected. Consumers avoiding the symptoms of food intolerance are the same as eluding the symptoms of food allergy. For example, people who are intolerant to lactose, a sugar that is a component of milk, avoid food products containing milk to avoid the symptoms associated with lactose intolerance. In addition, people who have celiac disease avoid food products containing wheat and other sources of gluten. Control strategies to prevent incorrect labeling of substances associated with a food intolerance or food disorder are analogous to those used to prevent incorrect labeling of food allergens.

      - Physical hazards (such as stones, glass, and metal fragments). Please see the reference guide for common sources of Physical Hazards.

| Source | Metal – Ferrous & Nonferrous | Plastic, Ceramic, and Glass | Other |
| --- | --- | --- | --- |
| Ingredient-related | - Farm field debris - Precut, ground, injected, and sliced, items, where the supplier does not properly control metal. | - Farm field debris, - Packaging materials | - Pits or pit fragments, shells |
| Facility-related and process-related processing/production environment, equipment, and pests (insects, birds, rodents, reptiles)) | - Equipment - Grinders, slicers, knives - Sieves, screens, wire mesh belts - Mixing paddles - Metal cans (shavings, lids) - Pumps - Cook Kettles with swept surface paddles - Drop buckets | - Equipment (inspection belts, small wares, buckets) - Facility (glass light fixtures, glass windows in doors, plastic strip curtains) - Glass containers - Scoops - Mixing paddles - Buckets | - Incomplete removal of pits or pit fragments, shells - Poor Design -- Particle size of food inappropriate for the consumer – choking hazard |
| People-related (actions or behaviors) | - Jewelry - Hairpins | - Buttons - Zipper pulls | N/A |

    - The analysis includes the known or reasonably foreseeable hazards that may be present in food for any of the following reasons:

      - The hazard occurs naturally.

      - The hazard may be unintentionally introduced.

      - The hazard may be introduced intentionally for purposes of economic gain.

  - Hazard Evaluation

    - The hazard analysis includes an evaluation of the hazards identified to assess the probability that the hazard will occur in the absence of controls and the severity of the illness or injury if the hazard were to occur.

    - The hazard evaluation includes an evaluation of environmental pathogens whenever a ready-to-eat food is exposed to the environment before packaging and the packaged food does not receive a treatment or otherwise include a control or measure (such as a formulation lethal to the pathogen) that significantly minimize the pathogen.

    - The hazard evaluation considers the effect of the following on the safety of the finished food for the intended consumer:

      - The formulation of the food.

      - The condition, function, and design of the establishment and equipment of a typical entity that manufactures/processes, grows, harvests, or raises this type of food.

      - Raw materials and other ingredients.

      - Raw materials and other ingredients.

      - Transportation practices.

      - Harvesting, raising, manufacturing, processing, and packing procedures.

      - Packaging and labeling activities.

      - Storage and distribution.

      - Intended or reasonably foreseeable use.

      - Sanitation, including employee hygiene.

      - Any other relevant factors, such as the temporal (e.g., weather-related) nature of some hazards (e.g., levels of natural toxins).

- Responsibility

  - Senior Management/FSVP Importer

    - Manages the organization’s Food Safety Management System (FSMS)

    - Overseeing food safety processes, audits, corrective actions, and recalls and keeping the organization informed of food safety issues.

    - Reviews and approves the organization’s HACCP Plan

    - Reviews and assesses the hazard analysis of the foreign supplier.

  - Qualified Individual

    - Analyzes the known or reasonably foreseeable hazards for the food to determine whether any hazards require control.

- Corrective Action

  - If the Hazard Analysis does not satisfy the requirements of regulating body or auditor,

    - Review the records;

    - Identify the cause and make a corrective action and preventive action report;

    - Update the HACCP Plan if necessary;

    - Review and assess the updated HACCP Plan.

- Review - Quality

  - The auditor reviews the HACCP Food Safety Plan at the initial audit to ensure all products within the site’s scope are covered, all potential hazards are identified, and the HACCP implementation steps have been followed. The HACCP Food Safety Plan, plus changes to the HACCP plan, are reviewed in full at every subsequent site audit. Implementation of the HACCP plan checked by interview, observation, and review of records. Evidence may include:

    - The HACCP team is in place, includes the expertise of the subject process, and members are trained in HACCP principles.

    - The HACCP team is fully involved in the development and review of the HACCP system.

    - The product and process scope of the HACCP plan is defined.

    - Product descriptions are available and include relevant safety information.

    - The intended use of the product is clearly defined.

    - A process flow diagram is developed and includes all process steps, inputs, and outputs. The HACCP team confirm it.

    - Potential hazards are identified for all process steps, and a hazard analysis was conducted using a consistent and valid method.

    - Control measures are in place for all identified hazards, and procedures/work instructions are effectively implemented.

    - CCPs are correctly identified using a valid methodology.

    - Critical limits are in place for every CCP and are validated to ensure consistent product safety;

      - All critical limits are monitored, and test procedures, responsibilities, and frequency are fully documented and implemented.

- History

| Revision No.: | Revision Date: | Description of Change: | Originator / Author Name: | Title / Department: |
| --- | --- | --- | --- | --- |
| 0 | 20230711 | Original | Arnel Ryan | PCQI / Compliance |

Requirements:

- The trainee has read or received a verbal translation of all or part of the policy, procedure, method, and or SOP for which they are being trained.

- The trainee has demonstrated the task they are to perform or the procedure they are being trained as required.

- The trainee has demonstrated the ability to perform the task with acceptable proficiency and minimal supervision as required.

| TRAINEE (Print Name): | TRAINEE (Signature): | Date: | Department: |
| --- | --- | --- | --- |
| Trainer / Supervisor (Print Name): | Trainer (Signature): | Date: | Department: |

UPON COMPLETION – EMAIL TO kav@interlinkiq.com

| NAME (Print): | Date: |
| --- | --- |

# Comprehension Quiz

Direction: Choose the best answer and encircle the letter.

- When conducting hazard analysis, the following are the types of hazards that are included EXCEPT:

  - Chemical hazards

  - Physical hazards

  - Workload hazards

  - Biological hazards

- The word CCP stands for:

- Critically Controlled Points

- Critical Control Points

- Critical Controllable Points

- What type of hazard do bacteria, viruses, and pathogens belong to?

  - Biological hazards

  - Physical hazards

  - Chemical hazards

- The Qualified Individual of the company conducts the hazard analysis

  - True

  - False

- Food allergens and substances associated with a food intolerance or food disorder is an example of what type of hazard?

  - Biological hazards

  - Physical hazards

  - Chemical hazards

UPON COMPLETION – EMAIL TO kav@interlinkiq.com