Cold Storage Freezing and Chilling of Foods Lesson

- Policy

  - Freezing and cold storage apply to sub-zero temperatures and storage of the product at that temperature for preservation. Ideally, frozen food is stored at 11 F or below.

  - Chilling refers to the process of reducing the temperature of food to 0 - 5°C (32 - 41°F) and storing it within that temperature range, to minimize pathogen growth and extend shelf-life.

  - In both cases, the equipment required to chill, freeze, or store product must be effective and cater to maximum throughput.

- Procedure

  - Freezing and cold storage equipment

    - Frozen goods are currently not received by COMPANY NAME.  Frozen storage is maintained at –26 to 11 F.

  - Refrigeration equipment

    - Have the capacity to maintain an ambient temperature at or below 5°C (41°F) except when loading or unloading the product from the cooler unless other temperatures are prescribed by legislation. During these operations, the ambient temperature returns to 5°C (41°F) within a short time after access doors are closed.

    - Verification is demonstrated through historical temperature recordings. Refrigeration facilities can reduce the temperatures of products at rates suitable to maintain food safety and/or quality or as prescribed by legislation appropriate to the commodities being processed.

  - Methods

    - Loading area must be designed to minimize unnecessary product exposure to conditions that would cause a negative impact on the product or package integrity. i.e. Protect product that requires cold storage from high temperatures.

    - All vehicles for the shipment of refrigerated or frozen products must be pre-cooled to 38 F or below, prior to loading. The refrigerator unit should be running during loading.  Carriers that are not at the specified temperature may not be loaded.  The temperature of the trailer is measured using an IR gun, and is recorded on the delivery docket.

    - Vehicles (e.g. trucks/vans/containers) shall be secured from tampering using a seal or other agreed-upon, acceptable device or system.

    - Documentation of floor materials is included in the site plan or description of the plant/processing area.

    - A written SOP addresses timely and effective water removal or excessive ice build-up. Dense waterproof concrete is generally used for flooring and needs to be smooth and graded to reduce water accumulation.

    - The tops of refrigerated rooms are covered with rodent-proof material.

    - Receiving employees are responsible for checking all incoming raw materials and product temperatures of all refrigerated items for compliance.  Refrigerated goods must be received at 41 F or below.   Storage temperature operational limits are 32 to 41 F. Frozen goods is currently not received by COMPANY NAME.  Frozen storage is maintained at –26 to 10 F.

    - Refrigerated items are promptly moved to their respective storage areas after receiving to prevent temperature abuse.

    - Inaccessible cavities are sealed to prevent the access of rodents or other pests. Storage racks and shelving are constructed of non-corrosive material and are easily cleanable.

    - The product on these racks or shelves is at least 30 cm (twelve inches) from the walls and 150 mm (6 inches) off the floor to prevent contamination and allow for adequate air circulation around the product.

    - Condensation from cooling equipment is piped to the facility drainage system or to the exterior of the building in a manner that does not create pools or any standing water. When defrosting refrigeration units, it is necessary that the timing of the defrosting be such that it does not pose a threat to the sanitary conditions of the area or product.

    - The surface temperature of goods from each outbound load and the condition of the trailer are recorded on the delivery docket.   One random box from the load is opened and the IR gun is used to measure surface temperature.  The box is resealed by the COMPANY NAME shipping personnel.  Trailer temperature, product temperature, description and lot number of goods in the load, trailer condition, initials, ship date.  Ensure the code date is recorded for each product in the load.  Refrigerated outbound goods as applicable must be 41 F or below.  Surface temperature of outbound frozen goods must be 11 F or below.

  - Validation and Verification

    - Monitoring and validation of the cooler temperature are done following the site’s Food Safety Plan or similar document.

    - Cooling or storage temperatures are verified and validated as prescribed by legislation.

    - Manual monitoring of cold storage rooms on a predetermined frequency is acceptable provided there is a justification for the frequency, and documentation is kept on file with corrective actions, if applicable.

- Responsibility

  - All Personnel

    - Adhere to this policy.

  - Receiving Personnel

    - Inspects inbound and outbound goods.

    - Ensure all refrigerated raw materials are in compliance with the required storage temperatures.

  - Supervisors or department heads and team leaders

    - Visually inspect areas for compliance with this policy.

    - Oversee (Verify) compliance of procedure.

    - Ensure personnel is trained on this policy and procedures.

    - Monitor and take corrective action to ensure compliance with this policy.

  - Quality Control Personnel

    - Validate training and adherence to policy and procedure.

    - Monitor and document non-conformance observation also known as discrepancy using Corrective Action Preventive Action-CAPA Report.

    - Verify the storage temperature of the refrigerated raw materials.

  - QC / Compliance Department

    - Maintain document control, records, and archive.

- Corrective Action

  - Train all personnel on this policy and reference SSOPs.

  - Non-compliance will require re-training or dismissal from job duties or termination of employment.

- Review – Quality

  - Cold storage, freezing, chilling procedures (SOPs), and temperature validation procedures are reviewed for the initial desk audit. Subsequently, they are audited as part of each site audit through observation, review of records, and interviews with refrigeration mechanics and operating personnel. Evidence includes:

    - SOPs exist for chilling, freezing, and cold storage;

    - SOPs exist for validation of chilled and frozen temperatures and times;

    - The site can confirm the effective operation of the chillers/freezers; the site can confirm the effective operation of the chilled and cold storage rooms; Cold storage rooms are properly designed and constructed;

    - Cold storage rooms are easily cleaned;

    - Cold storage areas are easily accessible for inspection;

    - There is adequate refrigeration capacity;

    - There is adequate freezer capacity;

    - There is no condensation in the cold storage area;

    - There is no frost or ice build-up in the cold storage area;

    - Defrost water is discharged appropriately.

    - Temperature monitoring is adequate;

    - Temperature records are retained.

- History

| Revision No.: | Revision Date: | Description of Change: | Originator / Author Name: | Title / Department: |
| --- | --- | --- | --- | --- |
| 0 | YYYYMMDD | Original | ENTER AUTHOR NAME | ENTER TITLE / ENTER DEPARTMENT |

Requirements:

- The trainee (employee/visitor) has read or received a verbal translation of all or part of the policy, procedure, method, and/or SOP for which they are being trained.

- The trainee has observed a demonstration of the task they are to perform or the procedure for which they are being trained as required.

- The trainee has demonstrated the ability to perform the task with acceptable proficiency and minimal supervision.

| TRAINEE (Print Name): | TRAINEE (Signature): | Date: | Department: |
| --- | --- | --- | --- |
| Trainer / Supervisor (Print Name): | Trainer (Signature): | Date: | Department: |

*Should you need assistance in customizing this document please email services@consultareinc.com