Ventilation Lesson

- Policy

  - Adequate and proper ventilation is provided to prevent condensate build-up in cooking areas or other areas where heat or steam are applied, and which can result in contamination due to condensate dripping onto the product or food-contact surfaces.

  - Fans and exhaust vents are insect-proofed and placed in locations where they do not pose a contamination risk.

  - In high-risk processing areas, positive air pressure is maintained to prevent airborne contaminants from being drawn into the area.

- Procedure

  - Ventilation

    - Provide adequate ventilation to cooker/washer steam and other equipment which produces heat.

    - Design and construct the ventilation in enclosed food processing areas based on applicable legislation.

    - Design and construct the ventilation in enclosed food processing areas such that condensation over food and surfaces of food contact equipment is prevented.

    - Place screens on vents and exhausts to prevent the ingress of flying insects.

    - Whole Foods requires the following:

      - Adequate heating, ventilation or refrigeration shall be provided in all areas to maintain proper environmental and sanitary conditions for ingredients, finished product, equipment, and packaging materials.

      - Heating and ventilation must be balanced to prevent condensation on walls or ceilings in product areas while maintaining positive static air pressure.

      - Overhead structures such as ventilation units must be clean and free of product buildup, dust, mold, rust, peeling paint and condensation.

  - Positive Air Pressure

    - Identify high-risk processing area/s.

    - Provide and maintain positive air pressure in segregated rooms or areas where high-risk food processes are performed, and which require a higher level of hygienic practice to prevent contamination by pathogenic organisms.

      - Potential airborne contaminants can include particulates, bacteria, mold spores, viruses, water vapor, water aerosols, oil aerosols, and allergens.

    - Consider factors such as room dimensions, potential contaminants, and product risk, temperature, humidity, filtration, pressurization, sanitation methods, maintenance access, and employee comfort when designing the systems and equipment supplying clean, filtered air in high-risk area/s.

    - Design airflow to keep food and food contact surfaces clean and free from contaminants and ensure that air flows from high-risk to low-risk areas.

    - Conduct risk assessment to determine the requirement for airflow, filter grades, and the minimum air changes per hour required to maintain air quality. The risk assessment will determine the amount of filtration required to minimize the contamination risk. Although specific requirements may vary at each site, high-risk food processing areas generally required a combination of prefilters and final filters with MERV ratings of 16.

    - The Preventive Maintenance programs covers appropriate filtration and proper maintenance to air supply source or equipment and cleaning or changing of filters at a frequency appropriate to the product and process. Any maintenance is done hygienically.

- Responsibility

  - Quality Department / Preventative Control Qualified Individual (PCQI)

    - Visually inspect areas for compliance with this policy.

    - Oversee (Verify) compliance of procedure.

    - Ensure personnel is trained on this policy and procedures.

    - Monitor and take corrective action to ensure compliance with this policy.

    - Validate training and adherence to policy and procedure.

    - Monitor and document non-conformance observation is also known as discrepancy using Corrective Action Preventive Action-CAPA Report.

  - Maintenance

    - Ensure compliance with Ventilation standard requirements when designing and constructing ventilation equipment and installation of appropriate positive air pressure.

    - Performs preventive maintenance of Ventilation/Positive Air Pressure system.

- Corrective Action

  - When condensates build up in cooking areas or other areas where heat or steam is applied, check for the adequacy of ventilation in the area. Install appropriate equipment to ensure proper ventilation.

  - If contaminated air is sucked into a high-risk area, check for the effectiveness of the positive air system.

  - Train all personnel in this policy and related SSOPs.

- Review – Quality

  - Annual review of compliance with Ventilation requirements by observing the following:

    - Food processing areas have adequate ventilation;

    - Cooking areas are adequately exhausted;

    - There is no condensation present over product or food contact surfaces in cooker areas;

    - Exhaust vents are adequately fly-proofed;

    - Positive air pressure exists in high-risk processing areas.

    - Exhaust fans and compressors are cleaned.

    - Condition of compressor filters.

6.   History

| Revision No.: | Revision Date: | Description of Change: | Originator / Author Name: | Title / Department: |
| --- | --- | --- | --- | --- |
| 0 | 20220328 | Original | Arnel Ryan | PCQI / Compliance |
| 1 | 20231206 | - Aligned with the updated program (KAV-QMS-11.1.6.P0-Ventilation Program-20231023.R2-For Approval) based on annual review. - Updated the Procedure, Corrective Action, and Comprehension Quiz | Arnel Ryan | PCQI / Compliance |

Requirements:

- The trainee has read or received a verbal translation of all or part of the policy, procedure, method, and or SOP for which they are being trained.

- The trainee has demonstrated the task they are to perform or the procedure for which they are being trained as required.

- The trainee has demonstrated the ability to perform the task with acceptable proficiency and with minimal supervision as required.