| Document Title: Shipping – Cold Products SOP | Document Control No.: [Enter Document Control No.] | Rev. No.: [Enter Revision No.] |
| --- | --- | --- |
| Department: [Enter Department] | Supersedes: Original | Effective Date: [Enter Date] |
| Prepared by: [Affix Name, Signature, Position Title] | Reviewed by: [Affix Name, Signature, Position Title] | Approved by: [Affix Name, Signature, Position Title] |
| Date: [Enter Date] | Date: [Enter Date] | Date: [Enter Date] |

- Purpose:

  - To ensure that all cold products to be shipped and received are fresh and safe by properly executing the loading and departure of the shipment.

- Frequency:

  - Every shipping of the cold products.

  - [ADD FREQUENCY HERE]

- Tools and Equipment:

  - Forklift

  - Shipping containers/Trailers (with refrigeration unit)

  - Pallets

  - Flash Light

  - Personal Protective Equipment (PPE)

    - Hard Hat

    - Reflectorized Vest

    - Leather Gloves

    - Safety Shoes

  - Calibrated thermometer

  - Clip Board

  - [ENTER LIST OF EQUIPMENT HERE]

- Consumable:

  - None

  - [ENTER LIST OF CONSUMABLES HERE]

- Responsibility:

  - Shipping Supervisor

    - To verify and contact the receiver for any discrepancies.

    - To verify the shipper’s specifications, the refrigeration unit is adequately prepared for transportation.

    - To ensure meeting other sanitary conditions for food transportation.

  - Shipping Personnel

    - To ship, inspect and fill out the inspection checklist.

    - To check and monitor the temperature of the refrigeration unit.

    - To ensure the safety of loads during operations.

    - To ensure the cleanliness of docks and vehicles during operations.

    - To manage the warehouse and vehicle loading.

  - Forklift Operator

    - To operate, inspect and fill out the Daily Pre Operational Checklist-Forklift.

  - Quality Control Head

    - To provide training to the concerned person before the implementation of the SOP.

    - To ensure the procedure is followed.

  - Quality Assurance Officer/Personnel

    - To review and check the SOP.

    - To ensure that the procedure is followed.

  - [ADD OR CUSTOMIZE RESPONSIBLE PERSON HERE]

- Procedure/Instructions:

  - Pre-cooling

    - Before loading, cool the refrigeration trailer unit to remove the residual heat and protect the cargo from heat-related damage

    - Set the thermostat to the desired carrying temperature for a single-temperature trailer or multiple temperatures for the multi-temperature host and remote evaporator unit.

    - Run the refrigeration unit at the desired carrying temperature until the unit shifts to low-speed operation just before loading the cargo.

    - Accomplish pre-cooling in less than two hours.

    - Adjust the trailer refrigeration settings to not more than 4°C (39°F) and pre-cool as a single compartment.

    - If possible, use refrigerated loading docks with cold tunnels to prevent outside ambient air from entering the refrigerated dock and the interior of pre-cooled containers.

    - Turn off the refrigeration unit AFTER the shipping containers reach the desired pre-cooled temperature during the loading process, as this will minimize heat and humidity entering the containers.

  - Loading

    - Trailer Assignment and Pre-Loading Safety Inspection

      - Secure the containers when they have been backed into place for loading.

      - Secure the Glad Hand Locks

      - Secure the Wheel Chocks

      - Ensure the containers are properly spotted in the Dock Door.

      - Ensure that the dock door is not opened until the containers are spotted and secured.

      - Check the condition of the compartments and components of the shipping containers; ensure cleanliness, free of putrid odors, and organized to prevent cross-contamination.

    - Refrigerationetation Unit – Pre-loading

      - Before picking up a load, run the unit in high-speed cool for at least 20 minutes and perform an automatic Pre-Trip. The unit ensures passing the Pre-Trip test. If it doesn’t, follow your company’s prescribed procedures.

      - Set the unit controller to the desired temperature determined by product, customer, and/or shipper requirements.

      - Always double-check the temperature to ensure it was set properly and prevent potential cargo damage.

    - Staging the Pallets and Cargo Temperatures

      - Choose high-quality, crush-resistant packaging (vented-side, top, and bottom packaging for fresh or chilled respiring products).

      - Stage the pallets in a loading zone near the loading dock and ideally pre-positioned in a Cool Dock to maximize trailer loading efficiency.

      - Before loading, use a thermometer to check the products’ internal (or pulp) temperature.

    - Loading the Trailers

      - Use approved jacks and/or forklifts to load the trailers.

      - Load the pallets in the prior sequence based on the Load Diagram.

      - Load the products quickly and efficiently, and use loading patterns that maximize air circulation through the entire load.

      - Ensure that the pallets allow refrigerated air to pass between the load, the walls, and the floor.

      - Consider the following when loading refrigerated containers:

        - Limit access to the trailer during loading to Loader(s) only to ensure safety

        - Load product quickly and efficiently from the staging area

        - Reduce surface contact of the product with the storage area and loading equipment

        - Loading placement is accounted for proper airflow around all product loaded

          - Refrigeration return air and discharge air openings are not blocked

          - Pallets openings are not obstructed (i.e., no-shrink wrapping or slip sheets for respiring cargoes like fruit and vegetables)

        - Pallet placements re-enforced the stability of the load o Place pallets side by side to avoid tipping during transit

          - Spacer pallets are used in an empty grid position

        - Bulkheads are properly secured between frozen and chilled compartment

        - Bulkheads are opened for the load to be unloaded safely

          - Secure bulkhead safely into place with strap(s)

        - Load bars and straps are used to secure and stabilize the load properly

          - Lock and secure E-track connectors

          - Tighten straps and secure loose ends, so they don’t touch the floor

      - Once loaded, physically stabilize the load to help prevent potential cargo damage due to load shifts that may occur during transit.

      - Use strip curtains to minimize inside and outside air exchange and resulting box temperature change during loading.

    - Refrigerationetation Unit – Post-loading

      - After loading is complete, close the doors immediately.

      - Start the refrigeration unit and re-confirm the correct operating mode and set-point that have been selected.

      - Initiate a manual defrost cycle to remove frost or ice from the evaporator coil until it reaches the desired cargo temperature.

      - Once the trailer is loaded, conduct a final review before closing the trailer doors, including but not limited to the following components:

        - Verify Bill of Lading (BOL) paperwork

        - Verify that all pallets have been loaded

        - Verify Load Diagram

        - Check a load for proper airflow

        - Check that bulkheads, load bars, and straps are properly secured

        - Close Trailer door, latch, lock, and secure door seals

      - Safely move the trailer to the staging area of the yard. Typical steps to follow are:

        - Raise dock plate

        - Close dock door

        - Ensure the dock light turns “Red.”

        - Proceed outside wearing a reflective safety vest

          - Remove glad hand lock

          - Remove wheel chocks

        - Ensure any changes or special information for the driver is marked clearly on the load diagram

        - Place the load diagram into the route folder and turn it into the supervisor

        - Notify appropriate personnel that the trailer is now ready to be moved to the staging area

  - Departure

    - Outbound Carrier

      - Release the trailers from the facility by the driver

      - Proceed to the pre-determined check-out location.

      - During the check-in procedure, retrieve any relevant and appropriate outbound paperwork associated with the load. The paperwork is contained with:

        - Bill of Lading

        - Loading Diagram

        - Copy of Order

        - Temperature Recorder Paperwork

        - Any carrier control forms required

  - Best Practices

    - Verify the product being loaded is within the appropriate temperature limits for the product type. The shipper of the product typically establishes these limits.

    - Temperature verification can usually be accomplished by placing a calibrated thermometer between two cases of product and recording the stabilized temperature. Perform this verification for a product being loaded in the trailer’s nose, middle, and tail.

    - If a multi-delivery load, pallets to be delivered last will be loaded first (in the front of the trailer).

    - If the order does not completely fill the trailer, a “Load Bar” are used to stabilize the load. The load bar is placed between the wall of the truck and the palletized products, directly behind the last pallets loaded onto the trailer.

    - Any product damaged during trailer loading is immediately addressed according to the customer’s expectations.

    - Care is taken to ensure that the top cases in the trailer do not inhibit airflow from the refrigeration unit. Maintaining proper and unobstructed airflow around the pallets in the trailer is important to prevent temperature variations or abuse.

    - Bulkheads are used to separate refrigerated from frozen sections of the trailer.

    - Loading the trailer is proceeded expeditiously to prevent product temperature abuse.

    - After the load has been verified by all interested parties (including the driver), ensure the material handling equipment has been removed from the trailer, raise the dock plate to its stored position, and release the dock restraint.

    - Place a unique security seal on the truck door and record the seal number on the shipping paperwork.

    - After the trailer has moved from the dock, close the dock door to prevent pest ingress.

    - Appropriate records are maintained for all activities for traceability/recall purposes(lot number and quantity shipped per item, shipping destinations, product temperatures, trailer temperatures – set-point and actual, etc.

  - [ADD OR CUSTOMIZE PROCEDURE HERE]

- Corrective Action:

  - If the containers are not properly pre-cooled, previously completed before the container was positioned at the dock, trailer air temperatures are to be confirmed in the forward (nose) section and the back (tail) section of the trailer.

  - Reject the following cold products:

    - Containing signs of deterioration of canned packaging, i.e., swollen sides or ends, flawed seals or seams, dents, or rust, punctured.

    - Foods with outdated expiration dates.

    - Foods that are out of safe temperature zone or deemed unacceptable by established rejection policy.

  - These are the following measures to do for the rejected/non-conforming products:

    - Segregate, properly label, and store at the Quality Quarantine Hold Area.

    - Investigate and issue CAPA Report.

    - Document the item’s discrepancy, and a photo will be for reference.

  - Retrain any employee found not following the procedures in this SOP.

  - [ADD OR CUSTOMIZE CORRECTIVE ACTION HERE]

- Reference Document:

  - Daily Pre Operational Checklist-Forklift

  - Daily Receiving Log

  - Bill of Lading (BOL)

  - Loading Diagram

  - Purchase Order

  - Temperature Log

  - Shipping Log Register

  - Outgoing Log

  - CAPA Report

  - [ADD OR CUSTOMIZE REFERENCE DOCUMENT HERE]

- History/Changes:

| Revision No.: | Revision Date: | Description of Change: | Originator / Author Name: | Title / Department: |
| --- | --- | --- | --- | --- |
| 0 | YYYYMMDD | Original | ENTER AUTHOR NAME | ENTER TITLE/ENTER DEPARTMENT |

*Should you need assistance in customizing this document, please email services@consultareinc.com