| Document Title: Cleaning and Sanitation – HVAC System – SSOP | Document Control No.: [Enter Document Control No.] | Rev. No.: [Enter Revision No.] |
| --- | --- | --- |
| Department: [Enter Department] | Supersedes: Original | Effective Date: [Enter Date] |
| Prepared by: [Affix Name, Signature, Position Title] | Reviewed by: [Affix Name, Signature, Position Title] | Approved by: [Affix Name, Signature, Position Title] |
| Date:[Enter Date] | Date:[Enter Date] | Date:[Enter Date] |

- Purpose:

  - To properly clean the HVAC system of the facilities.

- Frequency:

  - Monthly

  - [ADD FREQUENCY HERE]

- Tools and Equipment:

  - A vacuum cleaner with hose attachment

  - Nozzled pressure washer

  - Power washer

  - Screwdriver

  - Soft-brittle brush

  - Long-handle brush

  - Wire brush

  - Replacement air filter

  - Damp cloth

  - Dust cloth

  - Camera (photos)

  - Personal Protective Equipment (PPE)

    - Rubber gloves

    - Goggles

    - Mask

    - Waterproof boots

    - Waterproof apron

  - [ENTER LIST OF TOOLS AND EQUIPMENT HERE]

- Consumable(s):

  - Non-acidic  cleaner

  - [ENTER LIST OF CONSUMABLES HERE]

- Responsibility:

  - Maintenance personnel – to perform the cleaning and sanitation of HVAC systems.

  - Quality control personnel – to check the quality of the air circulating from the HVAC system through environmental sampling.

  - Maintenance manager – to ensure that the proper procedures in cleaning and sanitation of HVAC system are applied.

  - [ADD OR CUSTOMIZE RESPONSIBLE PERSON HERE]

- Procedure/Instructions:

  - Pre-cleaning

    - Wear rubber gloves, mask,  waterproof boots, a waterproof apron, and goggles for protection.

    - Prepare all the cleaning materials.

    - Turn off the power supply.

    - Lift the grillings of the HVAC system.

  - Furnace filter

    - Remove accumulated dust and dirt using a wire brush.

    - Loosen up black soot from each chamber.

    - Use a damp cloth if necessary.

    - Vacuum the entire block assembly to clean out all loosened dust and dirt particles.

  - Coils

    - Remove large debris from the coils.

    - Clean the coils using a nozzled pressure washer; make sure not to bend the aluminum fins.

    - Apply non-acidic cleaner.

    - If necessary, use a soft-bristle brush to remove accumulated dust and dirt further.

    - Rinse thoroughly with water using a power washer.

  - Air ducts

    - Unscrew air duct covers.

    - Cover supply vents with a cloth.

    - Set the thermostat to a fan.

    - Gently brush all accessible ducts to remove accumulated dust and dirt particles.

    - Vacuum accumulated dust and dirt in the supply registers.

    - Reach the bottom-most part of the duct to remove the debris using a long-handled brush.

    - Brush the return registers.

  - Blower compartment

    - Remove the panels on the front of the furnace to access the blower compartment.

    - Vacuum accumulated dust build-up in the blower compartment and furnace fan as well.

  - Electric terminals

    - Check for the presence of corrosion.

    - Clean corroded terminals using a wire brush.

  - Replace old filters at least once a month.

  - Return disassembled parts such as panels, grills, air ducts, etc.

  - Inspect if the HVAC system is cleaned correctly.

  - Check the air quality through environmental sampling.

  - Fill out the checklist.

  - [ADD OR CUSTOMIZE PROCEDURE HERE]

- Corrective Action:

  - If the environmental sampling results are unsatisfactory, re-clean the entire HVAC system.

  - Conduct preventive maintenance procedures of the HVAC system.

  - Issue incidence report and take photos for reference.

  - Issue CAPA (Corrective and Preventive Action Report) to involved personnel.

  - [ADD OR CUSTOMIZE CORRECTIVE ACTION HERE]

- Reference Document:

  - Monthly Cleaning and Sanitation Monitoring of HVAC Systems

  - Preventive Maintenance Checklist

  - SSOP Program of Clean and Sanitized Equipment

  - [ADD OR CUSTOMIZE REFERENCE DOCUMENT HERE]

- History/Changes:

| Revision No.: | Revision Date: | Description of Change: | Originator / Author Name: | Title / Department: |
| --- | --- | --- | --- | --- |
| 0 | YYYYMMDD | Original | ENTER AUTHOR NAME | ENTER TITLE/ENTER DEPARTMENT |

*Should you need assistance in customizing this document please email services@consultareinc.com