| Document Title: SSOP-Cleaning and Sanitation –Pinch Conveyor | Document Control No.: [Enter Document Control No.] | Rev. No.: [Enter Revision No.] |
| --- | --- | --- |
| Department: [Enter Department] | Supersedes: Original | Effective Date: [Enter Date] |
| Prepared by: [Affix Name, Signature, Position Title] | Reviewed by: [Affix Name, Signature, Position Title] | Approved by: [Affix Name, Signature, Position Title] |
| Date:[Enter Date] | Date:[Enter Date] | Date:[Enter Date] |

- Purpose:

  - To provide a procedure for cleaning and sanitation of the pinch conveyor.

- Frequency:

  - Every start-up.

  - Every end of production.

  - [ADD FREQUENCY HERE]

- Tools and Equipment:

  - Fabric cloth

  - Container/bucket

  - Spray bottle

  - Vacuum

  - Personal Protective Equipment (PPE)

    - Rubber gloves

    - Goggles

    - Apron or scrub suit

    - Rubber boots

    - Facemask

3.6. [ENTER LIST OF TOOLS AND EQUIPMENT HERE]

- Consumable(s):

  - Unscented liquid detergent – Food Grade

  - Unscented liquid disinfectant/sanitizer – Food Grade

  - [ENTER LIST OF CONSUMABLES HERE]

- Responsibility:

  - Cleaning Aide

    - In-charge in cleaning and sanitizing the pinch conveyor.

  - Production Operator

    - Conduct cleanliness and sanitary preliminary inspection on the pinch conveyor before start-up and after production.

    - Ensure that procedure for cleaning and sanitation of the pinch conveyor is followed.

  - Quality Assurance Personnel

    - Perform the verification of the overall cleanliness and sanitation of the pinch conveyor.

  - [ADD OR CUSTOMIZE RESPONSIBLE PERSON HERE]

- Procedure/Instructions:

  - Pre-cleaning Safety Precautionary Measure

    - Use appropriate Personal Protective Equipment (PPEs). Wear rubber gloves, goggles, an apron or scrub suit, rubber boots, and a face mask for protection.

    - Ensure no skin has any direct contact with unscented liquid detergent and unscented liquid disinfectant/sanitizer upon preparation, during, and after the cleaning and sanitation.

    - Read the labels on unscented liquid detergent and unscented liquid disinfectant/sanitizer containers carefully.

    - Ensure that the unscented liquid detergent and unscented liquid disinfectant/sanitizer are not expired and are good to use.

    - Ensure the pinch conveyor is shut down, turned off, and unplugged before proceeding with cleaning and sanitation. (Refer to Lock-Out Tag-Out SOP)

  - Pre-Cleaning

    - Prepare the unscented liquid detergent. Dilute the unscented liquid detergent in a designated bucket (follow the manufacturer’s direction for dilution). Note: Only prepare the exact amount used for cleaning.

    - Prepare the unscented liquid disinfectant/sanitizer. Dilute the unscented liquid disinfectant/sanitizer in a designated bucket (follow the manufacturer’s direction for dilution). Note: Only prepare the exact amount used for sanitizing.

    - Put the prepared liquid detergent and liquid disinfectant solution on a separate spray bottle with a label.

  - Cleaning of Pinch Conveyor

    - Dry clean first the pinch conveyor. Vacuum up the pinch conveyor to remove dust and other debris on the pinch conveyor.

    - Wipe the pinch conveyor with a clean and dry fabric cloth.

    - Wet clean the pinch conveyor. Pre-wet a clean fabric cloth by spraying a diluted unscented liquid detergent solution on it.

    - Wipe all parts/surfaces of the pinch conveyor with fabric cloth pre-wetted with diluted unscented liquid detergent solution.

    - Remove any stain and dirt embedded on the pinch conveyor.

    - Rinse off the pinch conveyor. Pre-wet a clean fabric cloth by spraying clean water on it.

    - Wipe all parts/surfaces of the pinch conveyor with fabric cloth pre-wetted with clean water.

    - Repeat the process until the pinch conveyor is clear of diluted unscented liquid detergent

solution.

  - Sanitation of Pinch Conveyor

    - Sanitize the pinch conveyor. Pre-wet a clean fabric cloth by spraying a diluted unscented liquid disinfectant/sanitizer solution.

    - Wipe the pinch conveyor with fabric cloth pre-wetted with diluted unscented liquid disinfectant/sanitizer solution.

    - Run the pinch conveyor at a slow speed.

    - Hold the fabric cloth pre-wetted with diluted unscented liquid disinfectant/sanitizer solution and wipe the pinch conveyor’s outer surface stationarily as it runs at a slow speed. Note: Do not let go of the fabric cloth.

    - Ensure complete sanitation of all food contact parts/surfaces.

    - Let the conveyor continue to run at a slow speed and allow it to air dry.

  - Post Cleaning and Sanitation

    - Stop the pinch conveyor once it is dry.

    - Dispose of the unused diluted unscented liquid detergent down the sink with running water.

    - Dispose of the used disposable PPEs after cleaning the pinch conveyor on its designated trash bin. Always replace the plastic liner of the trash bins after cleaning (refer to SOP on proper waste disposal).

    - Wash your hands thoroughly using soap and clean water after cleaning and sanitizing the pinch conveyor.

  - [ADD OR CUSTOMIZE PROCEDURE HERE]

- Corrective Action:

  - If stain or dirt was unable to remove after the first application of the diluted unscented liquid detergent solution, do the following:

    - Re-apply the diluted unscented liquid detergent solution to the stain or dirt.

    - Use a soft bristle brush for the removal of stains or dirt.

    - Source out alternative unscented liquid detergent for cleaning the pinch conveyor.

  - In the event one or more surface/part of the pinch conveyor was unproperly cleaned or sanitized, do the following:

    - Call the attention of the cleaning aide.

    - Repeat the cleaning procedure with strict observance.

    - Conduct training, demonstration, or workshop on correct cleaning of surfaces for all personnel doing and supervising the cleaning and sanitation of pinch conveyors.

  - Generate CAPA (Corrective and Preventive Action Report) on any noncompliance observed and encountered.

  - [ADD OR CUSTOMIZE CORRECTIVE ACTION HERE]

- Related Forms:

  - Conveyor Cleanliness and Sanitation Checklist

  - Schedule of Cleaning and Sanitation of Equipment and Utensils

  - Equipment Maintenance Checklist

  - Standard Operating Procedure for Verification and Inspection for Compliance

  - [ADD OR CUSTOMIZE REFERENCE DOCUMENT HERE]

- History/Changes:

| Revision No.: | Revision Date: | Description of Change: | Originator / Author Name: | Title / Department: |
| --- | --- | --- | --- | --- |
| 0 | YYYYMMDD | Original | ENTER AUTHOR NAME | ENTER TITLE/ENTER DEPARTMENT |

*Should you need assistance in customizing this document, please email services@consultareinc.com