Hazard Communication Lesson

- Policy

  - Company Name implements and maintains a hazard communication program that addresses the criteria for labels and other forms of warning, safety data sheets, and employee information and training.

  - Where Company Name is required to perform non-routine tasks that may require the use of hazardous substances, a special training course will be conducted to educate employees about the potentially hazardous chemicals they may be exposed to during the non-routine operation and how to avoid those exposures.

- Procedure

  - Labels and other Forms of Warning

    - Ensure that all containers containing hazardous chemicals in the workplace are appropriately labeled, tagged, or marked with either 2.1.1.1. to 2.1.1.5 or 2.1.16

      - Product Identifier

      - Signal Word

      - Hazard statement(s)

      - Pictogram(s)

      - Precautionary statement(s); or

      - Product identifier and words, pictures, symbols, or combinations thereof that provide at least general information about the hazards of the chemicals and that, when combined with other information immediately available to employees under the hazard communication program, provide specific information about the hazardous chemical's physical and health hazards.

    - May use signs, placards, process sheets, batch tickets, operating procedures, or other similar written materials for affixing labels to individual stationary process containers. The alternative method identifies the containers it applies and conveys the information required by 2.1.1.6.

    - Portable containers that are used to transfer hazardous chemicals from labeled containers and are intended solely for the immediate use of the employee performing the transfer are not required to be labeled.

    - Unless the container is immediately marked with the required information, do not remove or deface existing labels on incoming containers of hazardous chemicals.

    - Ensure that workplace labels or other forms of warning are legible, in English, prominently displayed on the container, or readily accessible in the work area during each work shift.

    - If there are employees who speak other languages, supplement the material presented with information in their native language, as long as the information is also presented in English.

    - If significant new information about a chemical's hazards becomes available, revise the chemical's labels within six months of becoming aware of the new information.

  - Safety Data Sheets (SDS)

    - Obtain a copy of the safety data sheet for each hazardous chemical being used in the workplace from the supplier.

    - Ensure that the supplier's safety data sheets contain the required information:

      - Section 1: Identification

      - Section 2: Hazard(s) Identification

      - Section 3: Composition/Information on Ingredients

      - Section 4: First-Aid Measures

      - Section 5: Fire-Fighting Measures

      - Section 6: Accidental Release Measures

      - Section 7: Handling and Storage

      - Section 8: Exposure Controls/Personal Protection

      - Section 9: Physical and Chemical Properties

      - Section 10: Stability and Reactivity

      - Section 11: Toxicological Information

      - Section 12: Ecological Information

      - Section 13: Disposal Considerations

      - Section 14: Transport Information

      - Section 15: Regulatory Information

      - Section 16: Other Information, including date of preparation or last revision.

    - Ensure that the information provided accurately reflects the scientific evidence used to classify the hazard.

    - Maintain copies of all required safety data sheets for each hazardous chemical in the workplace. Ensure that they are readily accessible to employees during each work shift when they are in their work area (s).

    - Electronic access and other alternatives to maintaining paper copies of safety data sheets are permitted, provided that such options do not create barriers to immediate employee access in each workplace.

    - Ensure that employees can quickly obtain necessary information in an emergency.

  - Employee Information and Training

    - Train employees on the hazardous chemicals in their work area before the initial assignment and when new hazards are introduced.

    - Develop information and training to address specific types of hazards, such as flammability, carcinogenicity, or the presence of specific chemicals.

    - Inform employees on:

      - Requirements of this program.

      - Any operations in their work area where hazardous chemicals are present.

      - The location and accessibility of the written hazard communication program, including the required hazardous chemical list(s) and safety data sheets.

    - Employee training includes:

      - Methods and observations that may be used to determine whether a hazardous chemical is present or has been released in a work area (such as monitoring conducted, continuous monitoring devices, visual appearance or odor of hazardous chemicals when being released, etc.)

      - The physical, health, simple asphyxiation, combustible dust, pyrophoric gas hazards, and hazards not otherwise classified of the chemicals in the work area.

      - The precautions that employees can take to safeguard themselves against chemical hazards, how to protect themselves from hazardous chemical exposure, appropriate work practices, emergency procedures, and the personal protective equipment to be used.

      - Workplace labeling system.

      - Safety Data Sheet, including the order of information and how employees can obtain and use the appropriate hazard information.

- Responsibility

  - Employer

    - Ensures that all hazardous chemicals in workplaces are labeled and have safety data sheets available for exposed workers.

    - Provides effective information and training to employees about hazardous chemicals in their work area at the time of their initial assignment and whenever a new chemical hazard is introduced into their work area that they have not previously been trained about.

  - Employee

    - Attends all mandatory training sessions.

    - Adheres to the hazard communication program

- Corrective Action

  - Evaluate and reassess the hazard communication program, periodically ensuring a current and relevant program.

- Review

  - Ensure that Hazard Communication programs are evaluated at least once a year to determine their efficacy.

- History

| Revision No.: | Revision Date: | Description of Change: | Originator / Author Name: | Title / Department: |
| --- | --- | --- | --- | --- |
| 0 | Enter Revision Date Here | Enter Description of Change Here | ENTER AUTHOR NAME | ENTER TITLE / ENTER DEPARTMENT |

Requirements:

- The trainee has read or received a verbal translation of all or part of the policy, procedure, method, and or SOP for which they are being trained.

- The trainee has demonstrated the task they are to perform or the procedure for which they are being trained as required.

- The trainee has demonstrated the ability to perform the task with acceptable proficiency and with minimal supervision as required.

*Should you need assistance in customizing this document please email services@consultareinc.com