| Document Title: Interlink–SSOP–Cleaning and Sanitation–Bag Sealer | Document Control No.: [Enter Document Control No.] | Rev. No.: [Enter Revision No.] |
| --- | --- | --- |
| Department: [Enter Department] | Supersedes: Original | Effective Date: [Enter Date] |
| Prepared by: [Affix Name, Signature, Position Title] | Reviewed by: [Affix Name, Signature, Position Title] | Approved by: [Affix Name, Signature, Position Title] |
| Date:[Enter Date] | Date:[Enter Date] | Date:[Enter Date] |

- Purpose:

  - To provide a proper procedure for cleaning and sanitation of the bag sealer.

- Frequency:

  - Every start-up.

  - Every end of production.

  - [ADD FREQUENCY HERE]

- Tools and Equipment:

  - Fabric Hand Towel

  - Container/Bucket

  - Spray Bottle

  - Soft Bristle Brush

  - Personal Protective Equipment (PPE)

    - Goggles

    - Rubber Gloves

    - Facemask

    - Apron

    - Rubber Boots

3.5. [ENTER LIST OF TOOLS AND EQUIPMENT HERE]

- Consumable(s):

  - Liquid detergent – Food Grade

  - Liquid disinfectant – Food Grade

  - [ENTER LIST OF CONSUMABLES HERE]

- Responsibility:

  - Cleaning Staff

    - Performs the cleaning and sanitation of the bag sealer.

    - Follow the procedure for cleaning and sanitation of the bag sealer.

  - Cleaning Team Leader/Supervisor

    - Perform an initial inspection after cleaning and sanitation of the bag sealer.

    - Ensure that the procedure for cleaning and sanitation of the bag sealer is followed.

  - Quality Assurance Personnel

    - Perform the verification of the overall cleanliness and sanitation of the bag sealer.

  - [ADD OR CUSTOMIZE RESPONSIBLE PERSON HERE]

- Procedure/Instructions:

  - Pre-cleaning Safety Precautionary Measure

    - Observe proper use of Personal Protective Equipment (PPEs). Wear goggles, rubber gloves, an apron, rubber boots, and a facemask for protection.

    - Ensure that the liquid detergent and liquid disinfectant are not expired and safe to use.

    - Ensure the bag sealer is shut down, turned off, and unplugged before proceeding with cleaning and sanitation.

    - Ensure no parts/surface of the bag sealer is at high or hot temperature to avoid any burns or injury before proceeding with cleaning and sanitation.

  - Pre-Cleaning

    - Dilute liquid detergent with clean water into a separate container/bucket (follow the manufacturer’s direction for dilution).

    - Dilute liquid disinfectant with clean water into a separate container/bucket (follow the manufacturer’s direction for dilution).

  - Cleaning of Bag Sealer

    - Perform dry cleaning on the bag sealer. Prepare a clean and dry fabric cloth.

    - Wipe all parts/surfaces of the bag sealer with a clean fabric hand towel.

    - Remove all dirt, dust, and other debris on the bag sealer.

    - Perform wet cleaning on the bag sealer. Prepare a clean fabric cloth pre-wetted with the diluted liquid detergent solution.

    - Wipe all parts/surfaces of the bag sealer with a clean fabric hand towel pre-wetted with diluted liquid detergent solution.

    - Remove all stains and any embedded dirt on the bag sealer.

    - Perform rinsing on the bag sealer. Prepare a clean fabric cloth pre-wetted with clean water.

    - Wipe all parts/surfaces of the bag sealer with a clean fabric hand towel pre-wetted with clean water.

    - Ensure the absence of diluted liquid detergent solution on all parts/surfaces of the bag sealer.

  - Sanitation of Bag Sealer

    - Perform sanitation of the bag sealer. Prepare a clean fabric cloth pre-wetted with the diluted liquid disinfectant solution.

    - Wipe all parts/surfaces of the bag sealer with fabric cloth pre-wetted with diluted liquid disinfectant/sanitizer solution.

    - Ensure complete sanitation of all food contact parts/surfaces.

    - Set aside the bag sealer and allow it to air dry.

  - Post Cleaning and Sanitation

    - Dispose of unused diluted liquid detergent and diluted liquid disinfectant solution down the sink with running water.

    - Ensure proper disposal of disposable PPEs into their designated trash bin. Always replace the plastic liner of the trash bins daily after cleaning the office(refer to SOP on proper waste disposal).

    - Wash your hands thoroughly with soap and clean water after cleaning and sanitation the bag sealer.

  - [ADD OR CUSTOMIZE PROCEDURE HERE]

- Corrective Action:

  - If stain or dirt was unable to remove after the first application of the diluted liquid detergent solution, do the following:

    - Re-apply the diluted liquid detergent solution to the stain or dirt.

    - Prepare a newly diluted liquid detergent solution and re-apply it to the stain or dirt.

    - Use a soft bristle brush to remove stains or dirt.

    - Source out alternative liquid detergent for cleaning of bag sealer.

  - If one or more surface or part of the bag sealer was unproperly cleaned or sanitized, do the following:

    - Call the attention of the cleaning staff responsible.

    - Repeat the cleaning and sanitation procedure with strict observance.

    - Conduct training, demonstration, or workshop for all personnel doing and supervising the cleaning and sanitation of bag sealer.

  - Create CAPA (Corrective and Preventive Action Report) on any non-compliance.

  - [ADD OR CUSTOMIZE CORRECTIVE ACTION HERE]

- Related Forms:

  - Sealer Cleanliness and Sanitation Checklist.

  - Schedule of Cleaning and Sanitation of Equipment and Appliances.

  - Standard Operating Procedure for Verification and Inspection for Compliance

  - [ADD OR CUSTOMIZE REFERENCE DOCUMENT HERE]

- History/Changes:

| Revision No.: | Revision Date: | Description of Change: | Originator / Author Name: | Title / Department: |
| --- | --- | --- | --- | --- |
| 0 | YYYYMMDD | Original | ENTER AUTHOR NAME | ENTER TITLE/ENTER DEPARTMENT |

*Should you need assistance in customizing this document please email services@consultareinc.com