Validation and Effectiveness Lesson

- Policy

  - The effectiveness of pre-requisite programs and other food safety controls is confirmed by observation, inspection, or audit.

  - Ensure that the programs and limits achieve their intended purpose in producing safe food and that critical food safety limits (as confirmed by scientific analysis) are validated.

  - Validation involves testing over and above daily monitoring to ensure that established food safety limits are effective, i.e., achieve the desired results, so that management can have confidence that the product and storage are safe.

  - Validation methods vary depending on the risk to finished product safety.

  - For hazards assessed as high risk, the critical limits are re-validated annually.

- Procedures

  - Effectiveness and Validation Methods for GMP and SSOPs

    - Water

      - Water that comes into direct contact with food or food contact surfaces is derived from a safe and sanitary source or is treated to make it safe.

        - Validation is through a Water potability test in-house and/or may request the water quality from the city.

    - Cleaning and Sanitation

      - All food-contact surfaces, including utensils and food-contact surfaces of equipment, are cleaned as frequently as necessary to protect against allergen cross-contact and contamination of food. Refer to 21 CFR 117.35 (d)

      - Refer to Cleaning and Sanitation Monitoring Log

    - Personnel Practices

      - Fat & Weird Cookie Company requires and issues line workers’ lab coats, single-use work gloves, effective hair restraints, and facemasks worn in the food processing plant. Where applicable, beard and mustache covers are supplied to cover and fully contain hair and beards.

      - The line supervisor will ensure that all production staff and visitors entering the food processing plant are issued protective gear. Employees are not allowed to use personal gear to replace these items unless authorized by the line supervisor and Quality Assurance.

      - Handwashing

        - Handwashing stations will be located in the processing room, break room, restrooms, and where good sanitary practices require employees to wash and sanitize their hands.

        - The Production Supervisor or Quality Assurance will check the handwashing stations for adequate supplies before operation begins and every four hours during operation. Results will be recorded on the daily sanitation audit form.

      - Infectious Disease

        - Anyone who has or may have, by medical examination or supervisory observation, an illness, infected wound, an open lesion such as a boil or sore, or any other problem that might contaminate food, food contact surfaces, or packaging materials shall be excluded from any operations until the condition is healed or corrected.

        - Validation is based on the Medical results compiled in the Employee’s Personal file.

    - Pest Prevention

      - The presence of rodents, insects, birds, or other pests in the plant is unacceptable.

      - Contract Service Provider has been contracted and is responsible for all facets of pest control within the warehouse. Material data sheets for all pesticides used by the company are on file.

      - The quality control department will inspect the facility for the presence of pests daily before operation. Refer to Pest Control Daily Inspection Checklist and Pest Sighting Log.

    - Training

      - All personnel are effectively trained and understand the Food Safety Management System, including SQF, Good manufacturing practices, Personnel Hygiene, and assigned duties.

      - Refer to Training Program

    - Allergen Management

      - Prevent sources of allergens from contaminating the product.

      - Testing Kits/Swab Testing

        - Qualitative Analysis of food allergens

        - Quantitative Analysis of Food Allergens

    - Calibration

      - Calibration of equipment

      - Authorized Third Party Calibration Service provider.

    - Environmental Monitoring Program

      - Cleaning and Inspection

      - Environmental Testing – EMP Zones.

  - Validation Methods for CCP’s

    - Validation for the critical limits identified for ALL CCPs. Refer to Food Safety Plan

      - Process Preventive Controls

| Process Control | Hazards | Critical Limits | Monitoring and Validation |
| --- | --- | --- | --- |
| Material Testing | Pathogenic E.coli, Salmonella, L. monocytogenes, S. Aureus | Per Microbiological Limits-COA | - Test Result from 3rd Party laboratory- Microbiological limits per customer, COA, Product Specification. - Every Material released and Random analysis testing. - Monitored by QC |
| Material Testing | Chemical Concentration Integrity (Intentional contamination Fraud | Per COA / Product Specification received with the material | - Test Result from 3rd Party laboratory- Chemical Concentration - Compared to COA/Product specification received with the material - Monitored by QC |
| Material Testing | Physical | Per Metal Detection critical limits 1.5 Ferrous 1.5 Non-Ferrous 2.0 Stainless Steel | - Metal Wand Testing - Hourly During production - Monitored by QC or designated personnel |

      - Allergen Control Step

| Allergen Control Step | Hazards | Critical Limits | Monitoring and Validation |
| --- | --- | --- | --- |
| Receiving (Packaging) | Undeclared Allergens | All Finished product labels must declare the allergens present in the formula per listing and label. | - Ingredient listing and allergen information match the product. - Visual Checking of Carton Labels to Match the formula- Label Review - Before release to Storage - Monitored and validated by QC |
| Receiving (Ingredients/ Materials) | Undeclared Allergens | All labels must declare the allergens present in the formula per listing and label. | - Ingredient listing and allergen information match the product. - Visual Checking of Carton Labels to Match the formula – Label Review - Before release to Storage - Monitored and validated by QC |
| Material Verification Testing Before production | Undeclared Allergens | Materials may not include other allergens, not on the formula list | - Allergen declaration on the COA, Product Specification, or Allergen Testing (Allergen Snap or Elisa Allergen) - Before Release for Production - Monitored and validated by QC |
| Material Validation Testing- Postproduction -Prepackaging | Undeclared Allergens | Blended products may not include other allergens on the formula list | - Allergen declaration on the COA, Product Specification, or Allergen Testing (Allergen Snap or Elisa Allergen) - Before Release for Distribution - Monitored and validated by QC |
| Packaging (Product Labels) | Undeclared Allergens | All Finished product labels must declare the allergens present in the formula per listing and label. | - Ingredient listing and allergen information match the product. - Visual Checking of Carton Labels to Match the formula- Label Review - Before release to Distribution - Monitored and validated by QC |

      - Frequency

        - Critical Food Safety limits are validated annually and re-validated when changes to the process occur.

        - Refer to Food Safety Plan

        - Refer to Validation- Food Safety Plan

  - Validation and Verification

    - Validation and verification activities of GMP SSOPs and all applicable elements of SQF are scheduled by the SQF Practitioner for any changes or processes that occur.

    - Refer to Validation-GMP

    - Refer to SQF System Management Review

    - Refer to Validation-Food Safety Plan

- Responsibility

  - The SQF practitioner is responsible for documenting and implementing the methods, responsibility, and criteria for confirming the effectiveness of pre-requisite programs and validating critical food safety limits to ensure they achieve their intended purpose.

  - Senior Management and QA must demonstrate how the validation methods ensure that the selected critical limits achieve the level of control required for the targeted food safety hazard.

  - QA must also have documentation showing that the methods and control measures provide the level of control needed.

- Corrective Action

  - If the processes or procedures are not effectively implemented.

    - Re-review of the processes or procedures.

    - Any changes must be reviewed, approved, and implement.

    - Re-validate.

- Review-Quality

  - Documentation of the methods and responsibility and criteria for ensuring the effectiveness of the pre-requisite programs;

  - Implementation of the methods and responsibility and criteria for ensuring the effectiveness of the pre-requisite programs;

  - Pre-requisite programs achieve their intended purpose;

  - Critical food safety limits are validated annually or when changes to process occur;

  - Methods used to validate critical limits ensure that the process step is safe if critical limits are met;

  - Critical limits effectively provide the designated level of control;

  - Personnel conducting validation activities understand their roles and responsibilities (refer to 2.5.1);

  - Records of all verification activities are current and accurate.

- History

| Revision No.: | Revision Date: | Description of Change: | Originator / Author Name: | Title / Department: |
| --- | --- | --- | --- | --- |
| 0 | 20230110 | Original | Arnel Ryan | PCQI / Compliance |

Requirements:

- The trainee (employee/visitor) has read or received a verbal translation of all or part of the policy, procedure, method, and/or SOP for which they are being trained.

- The trainee has observed a demonstration of the task they are to perform or the procedure for which they are being trained as required.

- The trainee has demonstrated the ability to perform the task with acceptable proficiency and minimal supervision.