Building Materials Lesson

- Policy

  - Floors, drains, and waste traps are designed and constructed in such a way as to minimize the risk to product or process safety.

  - The buildings' design, construction, and condition concerned contain storage and distribution operations, specifically the floors, partitions, doors, and ceilings. It is designed and constructed in such a way as to minimize risk to product safety and, in some instances, to offer protection to the product.

  - The extent to which these elements are relevant will depend on the type of processes housed and whether the product is enclosed or exposed.

  - The construction of material and surfaces used at the site are constructed in a way that is easily cleanable and prevents contamination of products or the process.

- Procedure

  - Floors, drains, and waste traps

    - Provide drains accessible for cleaning indicated on the company's Plant and Grounds-Plant and Construction Design provided by the maintenance department.

    - Provide removal grates.

    - Demonstrate (by maintenance department) risks to products and control those identified food safety risks practices to all employees.

    - Document floor materials included in the site plan or description of the premises and
product handling area.

    - Layout and provide proper drainage based on the company's Plant and Grounds-Plant and Construction program.

    - Provide, where necessary, adequate screening or other protection against pests.

    - Position and construct drain with effective removal of overflow or wastewater under normal working conditions.

    - Based on Plant and Grounds Programs, locate the waste trap system away from any food handling or storage.

  - Walls, Partitions, Doors, and Ceilings

    - Construct windows with shatterproof material or otherwise covered to prevent glass or plastic fragments from entering the product or packaging.

    - Slope downward window ledges for ease of cleaning and to prevent their use for unwanted storage of utensils or other materials.

    - Subject doors routinely a solid construction, impact-resistant, non-corrosive materials, preferably with a smooth, light-colored surface.

    - Protect doors between operational rooms used to transport products against damage by crates, trolleys, forklifts, or similar traffic.

    - For efficiency and ease of cleaning, walls with cement render and smooth finish glazed tiles, fabricated insulated panels, or similar materials are examples of acceptable surfaces.

    - Recess service ducting, conduit, and pipes, ideally into walls or ceilings, suspended from ceilings, housed inside drop ceilings with vertical drops to their point of use, or mounted a sufficient distance from walls or ceilings.

    - Where design does not provide for the avoidance of debris build-up, schedule frequent cleaning to minimize any build-up.

    - Perform pest monitoring in drop ceilings.

    - Drop ceilings, if used, check and clean regularly based on the cleaning and sanitation program.

    - Where drop ceilings are not used, check cleaning regimes and inspections for dust on ledges, loose fittings, glass windows, light fittings, or other areas where dust can accumulate and fall onto the product.

  - Materials and Surfaces

    - Ensure that the product contact surface is impervious, non-corrodible, smooth, easy to clean, light-colored, nontoxic, and impact resistant.

    - Follow the approved product contact surfaces:

      - Stainless

      - Steel

      - Aluminum

      - Hot-dipped

      - Galvanized steel

      - Fiberglass

      - Polyvinyl chloride

      - Nylon

    - Keep clean all other surfaces and prefer light colors.

    - Ensure that product contact surfaces are in good condition and accomplish the facilities checklist at least monthly.

- Responsibility

  - Senior Site Management

    - Ensure and verify this policy and procedure are adhered to.

    - Take corrective action to ensure compliance with this policy.

  - Quality Department

    - Responsible for the verification of the SSOPs for the floor drains, waste traps, walls, partitions, doors, and ceilings.

    - Validate training and adherence to policy and procedure.

    - Monitor and document non-conformance observation is, also known as discrepancy, using the Corrective and Preventive Action Form.

  - Maintenance Department

    - Ensures compliance with the requirement of design and construction of building facility.

  - Sanitation Department

    - Responsible for the implementation of the SSOPs for the floor drains, waste traps, walls, partitions, doors, and ceilings.

- Corrective Action

  - If found any non-conformance in the implementation of the program;

    - Re-train all personnel in this policy.

    - Non-compliance will require re-training or dismissal from job duties or termination of employment.

  - If a part of the site (floors, drains, waste traps, walls, partitions, doors, ceilings) imposes a food safety risk, the following corrective action is performed:

    - Redesign and reconstruction; and

    - Update Blueprints (site plan).

  - If a part of the site (floors, drains, waste traps, walls, partitions, doors, ceilings) has accumulated dirt or debris:

    - Conduct cleaning and sanitation to meet acceptable hygienic standards for these areas.

    - Perform environmental swabbing for microbiological analysis to verify the cleanliness of the areas.

  - If any pests are found in drop ceilings, locate pest traps in the secured area based on the pest management program.

- Review – Quality

  - Floors are smooth and easy to clean.

  - Floors are correctly graded to allow for water run-off.

  - Floors are made of appropriate, smooth, dense, impact-resistant material.

  - There are no areas of water pooling or build-up.

  - Procedures are in place to deal with poorly designed or constructed floor areas.

  - Drain locations do not pose a safety risk.

  - Drain construction does not pose a safety risk.

  - Waste traps are located away from food handling areas or entrances to the site.

  - Walls and partitions are of sound construction and made of suitable materials.

  - Doors are of sound construction for the volume and type of traffic.

  - Ceilings are of sound construction and made of suitable materials.

  - Walls, partitions, ceilings, and doors are kept clean.

  - Where a drop ceiling is used, the area is kept clean and tidy.

  - Service lines are designed and constructed for ease of cleaning.

  - The condition of walls, partitions, doors, and ceilings does not pose a food safety risk.

  - Knowledge of local, state, and federal regulations on the construction and operation of food
premises.

  - The site has been approved by relevant authorities for construction.

  - The site is approved by relevant authorities for the storage and distribution of the applicable
products.

  - Approval has been sought and given for changes to facilities or equipment.

- History

| Revision No.: | Revision Date: | Description of Change: | Originator / Author Name: | Title / Department: |
| --- | --- | --- | --- | --- |
| 0 | YYYYMMDD | Original | ENTER AUTHOR NAME | ENTER TITLE / ENTER DEPARTMENT |

Requirements:

- The trainee (employee/visitor) has read or received a verbal translation of all or part of the policy, procedure, method, and/or SOP for which they are being trained.

- The trainee has observed a demonstration of the task they are to perform or the procedure for which they are being trained as required.

- The trainee has demonstrated the ability to perform the task with acceptable proficiency and minimal supervision.

| TRAINEE (Print Name): | TRAINEE (Signature): | Date: | Department: |
| --- | --- | --- | --- |
| Trainer / Supervisor (Print Name): | Trainer (Signature): | Date: | Department: |

*Should you need assistance in customizing this document please email services@consultareinc.com