Air and Other Gases Training Material and Comprehensive Quiz

- Policy

  - The air quality program will be reviewed during the initial desk audit. Subsequently, this element will be audited as part of each site audit through observation, review of records, and interviews with operating personnel. Evidence may include:

    - The condition of air compressors and compressed air used to transport product or otherwise to come into contact with exposed product and product contact surfaces;

    - Filters are checked or changed at a frequency based on the air quality program;

    - Maintenance staff has the data specification sheet for the filter housing;

    - Follow up with preventative maintenance and SSOPs;

    - Performance characteristics of the filter in place must match the risks identified in the site’s assessment.

    - Compressed air in contact with food is checked for purity using methods and at a frequency based on the air quality program and test procedures.

  - The compressed air used in food operations is appropriate, verified, and validated and does not serve as a source of contamination.

  - Preventive maintenance programs are ensured, and an appropriate filtration program is in place at the point of use.

  - Testing is conducted to validate the compressed air-filtration control system’s effectiveness based on the risk to the product.

- Procedure

  - Compressed Air

    - Compressed air can be a source of chemical and microbiological contamination.

    - Potential contaminants can include particulates, including dirt (microorganisms, atmospheric dirt and solid particulates, rust, and pipe scales), water (water vapor, condensed liquid water, and water aerosols), and oil (oil vapor, liquid oil, and oil aerosols).

    - Food-grade oil is used in the air compressor when air comes in contact with the exposed product or direct product contact surfaces.

  - Preventive Maintenance

    - Filters are cleaned or changed at a frequency appropriate to the product and process or following any air supply source or equipment maintenance.

    - Any maintenance is done hygienically.

    - Nozzles and air hoses are in good condition, properly repaired, and maintained in a hygienic state (e.g., cleaned and sanitized).

    - Hoses and nozzles are kept off the ground.

    - Locate the filtration as close as possible (near the “point of use” or the point where air contacts the product) to not have long lengths of piping/tubing between the microbial removal filter and the air/food contact point.

  - Particulate Controls

    - The following are controls for particulates:

      - Intake filters to remove atmospheric dirt and solid particulates.

      - An effective Preventive Maintenance program should be in place to maintain the integrity of the filter.

      - Water, including vapor, liquid, and condensed.

    - A dryer in the compressed air system provides effective control.

    - The presence of coalescing filters in the compressed air system effectively removes contamination.

- Responsibility

  - Supervisors / Department Heads / Team Leaders

    - Ensure (inspect) and verify this policy and procedure is adhered to and;

    - Take corrective action to ensure compliance with this policy.

  - Quality Personnel

    - Validate training and adherence to policy and procedure.

    - Monitor and document non-conformance observation, also known as discrepancy, using the Corrective and Preventive Action Form.

    - Document control, records, and archive.

  - Maintenance Personnel

    - Ensure compliance with this policy.

    - Performs preventive maintenance of air and other gas systems.

- Corrective Action

  - Review and revise the preventive maintenance procedure.

  - Check and change the filters at a frequency.

  - Maintain cleanliness and properly sanitize nozzles and hoses.

- Review – Quality

  - The condition of air compressors and compressed air used to transport product or otherwise to come into contact with exposed product and product contact surfaces.

  - Filters are checked or changed at a frequency based on the air quality program.

  - The maintenance staff has the data specification sheet for the filter housing.

  - Follow up with preventative maintenance and SSOPs.

  - Performance characteristics of the filter in place must match the risks identified in the site’s assessment.

  - Compressed air in contact with food is checked for purity using methods and at a frequency based on the air quality program and test procedures.

- History

| Revision No.: | Revision Date: | Description of Change: | Originator / Author Name: | Title / Department: |
| --- | --- | --- | --- | --- |
| 0 | 20220805 | Original | Arnel Ryan | PCQI / Compliance |
| 1 | 20231013 | - Aligned with the updated program. - Updated the Policy, Procedure, Responsibility, and Corrective Action | Arnel Ryan | PCQI / Compliance |
| 2 | 20240606 | - Aligned with QMS format. - Changed Lesson to Training Material and Comprehension Quiz. | Arnel Ryan | PCQI / Compliance |

Requirements:

- The trainee (employee/visitor) has read or received a verbal translation of all or part of the policy, procedure, method, and/or SOP for which they are being trained.

- The trainee has observed a demonstration of the task they are to perform or the procedure for which they are being trained as required.

- The trainee has demonstrated the ability to perform the task with acceptable proficiency and minimal supervision.

| TRAINEE (Print Name): | TRAINEE (Signature): | Date: | Department: |
| --- | --- | --- | --- |
| Trainer / Supervisor (Print Name): | Trainer (Signature): | Date: | Department: |

UPON COMPLETION – EMAIL TO rfp@interlinkiq.com

| NAME (Print): | Date: |
| --- | --- |

Air and Other Gases Comprehension Quiz

Direction: Choose the best answer and encircle the letter.

- The air quality program will be reviewed during the initial desk audit. Subsequently, this element will be audited as part of each site audit through observation, review of records, and interviews with operating personnel.

- True

- False

- ____________ is used in the air compressor when air comes in contact with the exposed product or direct product contact surfaces.

- Regular oil

- Food-grade oil

- None of the above

- Nozzles and air hoses are to be in good condition, properly ______________ in a hygienic state (e.g., cleaned and sanitized).

- repaired

- maintained

- Both A and B

- The presence of coalescing filters in the compressed air system effectively removes contamination.

- True

- False

- Compressed air that is in contact with food is checked for purity using methods and at a frequency based on the _____________.

- air quality program

- test procedures

- All the above

UPON COMPLETION – EMAIL TO rfp@interlinkiq.com