Duty To Have Fall Protection and Falling Object Protection of

Walking-Working Surfaces Training Material and Comprehension Quiz

- Policy

  - Management ensures that the regulations of walking-working surfaces meet the standards.

  - An applicable private agency handling the regulations of walking-working surfaces is an option in making the certification.

  - Inspection of the regulations of walking-working surfaces is monitored and always documented.

  - Orientation, testing, and verification of the regulations of walking-working surfaces are documented.

- Procedure

  - General Industry Fall Hazards

    - The employer provides protection to fall hazards from unprotected sides and edges, hoist areas, holes, dockboards, runways, walkways, dangerous equipment, openings, repair pits, service pits, assembly pits, fixed ladders, outdoor advertisements (billboards), stairways, scaffolds, rope descent systems, low-slope roofs, and slaughtering facility platforms.

    - The employer ensures that each employee on a walking-working surface with an unprotected side or edge that is 4 feet (1.2 m) or more is protected from falling by guardrail systems, safety net systems, and personal fall protection systems.

  - Fall Protection System

    - When the employer can demonstrate that it is not feasible or creates a greater hazard to use guardrails, safety nets, or personal fall protection systems on residential roofs, the employer develops and implements a fall protection plan.

    - When the employer can demonstrate that the use of fall protection systems is not feasible on the working side of a platform used at a loading rack, loading dock, or teeming platform, the work may be done without a fall protection system.

    - Protect each employee in a hoist area from falling 4 feet (1.2 m) or more to a lower level by a guardrail system, personal fall arrest system, and travel restraint system.

    - When any portion of a guardrail system, gate, or chains is removed, and an employee can lean through or over the edge of the access opening to facilitate hoisting, the employee is protected from falling by a personal fall arrest system.

    - Protect each employee from falling through any hole (including skylights) that is 4 feet (1.2 m) or more above a lower level by covers, guardrail systems, travel restraint systems, and personal fall arrest systems.

    - Protect each employee from tripping into or stepping into or through any hole that is less than 4 feet (1.2 m) above a lower level by covers or guardrail systems.

    - Protect each employee from falling into a stairway floor hole by a fixed guardrail system on all exposed sides, except at the stairway entrance.

    - Protect each employee from falling into a ladderway floor hole or ladderway platform hole by a guardrail system and toe boards erected on all exposed sides.

    - Safeguard each employee from falling through a hatchway and chute floor hole.

    - Ensure that each employee on a dockboard, runways, and walkways is protected from falling 4 feet (1.2 m) or more to a lower level by a guardrail system or handrails.

    - Protect each employee from falling into or onto dangerous equipment that is less than 4 feet (1.2 m) above by a guardrail system or a travel restraint system, unless the equipment is covered or guarded to eliminate the hazard.

    - Ensure that each employee on a walking-working surface near an opening is protected from falling.

    - The use of a fall protection system is not required for a repair pit, service pit, or assembly pit that is less than 10 feet (3 m) deep.

    - Always check all existing and new fixed ladders.

    - Make sure there is a personal fall arrest system whenever an employee engages in outdoor advertising and climbs a fixed that is not equipped with a cage.

    - Protect each employee exposed to an unprotected side or edge of a stairway or landing that is 4 feet (1.2 m) or more above a lower level by a guardrail or stair rail system.

    - Ensure each employee on a scaffold is protected from falling.

    - When work is performed less than 6 feet (1.6 m) from the roof edge, the employer ensures each employee is protected from falling by a guardrail system, safety net system, travel restraint system, or personal fall arrest system.

    - Protect each employee on the unprotected working side of a slaughtering facility platform that is 4 feet (1.2 m) or more above a lower level from falling.

  - Protection from Falling Objects

    - When an employee is exposed to falling objects, the employer ensures that each employee wears head protection.

    - Protect employees from falling objects by implementing erecting toe boards, screens, or guardrail systems to prevent objects from falling to a lower level.

    - Keep employees safe from falling objects by erecting canopy structures and keeping potential falling objects far enough from an edge, hole, or opening to prevent them from falling to a lower level.

    - Protect employees from falling objects by barricading the area into which objects could fall, prohibiting employees from entering the barricaded area, and keeping objects far enough from an edge or opening to prevent them from falling to a lower level.

- Responsibility

  - Management

    - Make the regulations of fall protection that meet the standards.

    - Make sure that the company is compliant with the regulations of fall protection.

    - Check the documentation records of all employees regarding the regulations of fall protection.

    - Review hazards and incidents associated with fall protection equipment.

    - Assist supervisors with employee training.

  - Quality Department

    - Inspect the effectiveness of the regulations of fall protection for verification of compliance with standards.

  - Supervisor

    - Comply with all Fall Protection and Falling Object Protection procedures.

    - Identify all fall hazards and activities in their workplace and implement preventative measures for these hazards.

    - Ensure all affected employees attend and complete the required training.

    - Ensure that all personal fall arrest or restraint systems are maintained in accordance with the manufacturer’s specifications.

    - Hands-on training of all fall protection equipment is required.

    - Ensure employees are using all fall protective equipment in accordance with OSHA regulations.

  - Employees

    - Comply with all Fall Protection and Falling Object Protection procedures.

    - Maintain all Personal Protective Equipment (PPE) required to work at heights.

    - Inspect all fall protection equipment prior to use

    - Attend and complete all training requirements.

    - Immediately report all damaged or defective fall protection equipment to the supervisor.

    - Use all fall protection equipment in accordance with OSHA regulations.

- Corrective Action

  - Document verification at receiving

    - In the event, that the official fall protection documents are not present during the implementation, do the following:

      - Reject material;

      - Place on hold and call the person-in-charge; and

      - Do not use material until verification is complied with.

  - Quality Requirements

    - In the event, that the fall protection regulations do not meet the quality standards do the following:

      - Reject materials;

      - Place on hold and call the person-in-charge; and

      - Do not use material until verification is complied with.

- Review - Quality

  - Review of this program is conducted periodically or when there are changes in the process that require this program to be revised.

- History

| Revision No.: | Revision Date: | Description of Change: | Originator / Author Name: | Title / Department: |
| --- | --- | --- | --- | --- |
| 0 | 20240105 | Original | Arnel Ryan | PCQI/Compliance |
| 1 | 20241113 | - Changed email from rfp@interlinkiq.com to rfp@consultareinc.com - Added Program Title to Comprehension Quiz section | Arnel Ryan | PCQI/Compliance |

Requirements:

- The trainee has read or received a verbal translation of all or part of the policy, procedure, method, and/or SOP for which they are being trained.

- The trainee has demonstrated the task they are to perform or the procedure for which they are being trained as required.

- The trainee has demonstrated the ability to perform the task with acceptable proficiency and minimal supervision.

| TRAINEE (Print Name): | TRAINEE (Signature): | Date: | Department: |
| --- | --- | --- | --- |
| Trainer / Supervisor (Print Name): | Trainer (Signature): | Date: | Department: |

UPON COMPLETION – EMAIL TO rfp@consultareinc.com

| NAME (Print): | Date: |
| --- | --- |

Duty To Have Fall Protection and Falling Object Protection of Walking-Working Surfaces Comprehension Quiz

Direction: Choose the best answer and encircle the letter.

1. Who is responsible for making the regulations of fall protection that meet the standards?

- Administrative Management

- Purchasing Managing Department

- HR Department

2. The use of a fall protection system is not required for a repair pit, service pit, or assembly pit that is less than _____ deep.

- 8 feet (2.43 m)

- 9 feet (2.74 m)

- 10 feet (3 m)

3. What department inspects the effectiveness of the regulations of fall protection for verification of compliance to standards?

- HR Department

- Quality Department

- Administrative Department

4. Management ensures that the regulations of walking-working surfaces meet the standards.

- True

- False

5. Applicable private agency handling the regulations of walking-working surfaces is an option in making the certification.

- True

- False

## UPON COMPLETION – EMAIL TO rfp@consultareinc.com