Food Safety Plan Training Material and Comprehensive Quiz

- Policy

  - The Food Safety Plan is prepared using the HACCP method, which means that all products and processes included in the SQF scope of certification are addressed in one or more HACCP plans.

  - The Codex HACCP model is to be used.

  - All HACCP principles and implementation steps are included in the HACCP Food Safety Plan.

  - The HACCP Plan is fully developed by the site, meaning the site may use the services of an SQF consultant, but takes full responsibility for the HACCP plan.

- Procedure

  - Introduction to Food Safety Plan

    - A Food Safety Plan (FSP) consists of the primary documents in a preventive controls food safety system that provides a systematic approach to the identification of food safety hazards that must be controlled to prevent or minimize the likelihood of foodborne illness or injury. It contains a collection of written documents that describe activities that ensure food safety during manufacturing, processing, packing, and holding.

  - Food Safety Team

    - The Food Safety Team Leader should be appointed Preventive Control Qualified Individual (PCQI) to manage the Food Safety Plan. PCQI does not need to be an employee of the Facility.

    - Food Safety Team should have the knowledge and expertise to develop Food Safety Plan. The team should be multidisciplinary and should be in different departments.

      - Management

      - Production

      - Quality Assurance

      - Warehouse

      - Laboratory

      - Maintenance

      - Sanitation

      - Purchasing

      - Customer Service

      - Compliance

    - Team Composition

      - In Selecting the team, the Food Safety Leader should focus on:

        - Those who will be involved in hazard identification

        - Those who will be involved in the determination of critical control points

        - Those who will monitor critical control points

        - Those who will verify operations at critical control points

        - Those who will examine samples and perform verification procedures

    - Training

      - The team members must be trained on the Codex General Principles of Food Hygiene and the guidelines for applying the HACCP system to ensure that the team will work together with a common focus and use the same approach and terminology.

        - Refer to Training Records

        - Refer to Training Program

  - Product Description and Intended Use

    - The Food Safety team must make a complete description of each food product - including all ingredients/processing methods/packaging materials/etc. used in the formulation of the product - to assist in identifying all possible hazards associated with the product. The product description should include:

      - Name of the Product

      - Manufactured for

      - Product Description including Important Food Safety Characteristics

      - Ingredients

      - Allergens

      - Packaging Used

      - Packaging Size(s)

      - Packaging Temperature

      - Intended Use

      - Intended Consumer

      - Shelf Life

      - Lot Code Specification

      - Labeling Instructions

      - Storage and Distribution

      - Label Claim

      - Composition

    - Identification of intended use

      - The intended use of the product refers to its normal use by end-users or consumers. The Food Safety Team must specify where the product will be sold and the target group, especially if it happens to be a sensitive portion of the population.

  - Process Flow

    - The Food Safety Team should draw a process flow diagram showing each step-in operation. It was generally representing all the steps that raw materials go through to become a finished product. Step in a process flow may include:

      - Receiving of Raw Materials

      - Store of Ingredients

      - Preparation

      - Cooking

      - Cooling

      - Packing

      - Labeling

      - Storage

      - Distribution

    - Process Flow Format

      - Linear Flow Diagram

        - A table format or other program can be used to develop a flow diagram for the product(s) covered in this food safety plan.

  - Process Narrative Diagram

    - A text description of each step in the flow diagram can provide more food safety and/or control information than can be shown easily in the flow diagram and is important to understanding the process. Refer to Process Flow Program

| Step | Process Description |
| --- | --- |
| 1 | Receiving of Raw Materials- [Enter Description] |
| 2 | Store of Ingredients- [Enter Description] |
| 3 | Preparation- [Enter Description] |
| 4 | Cooking - [Enter Description] |
| 5 | Cooling- [Enter Description] |
| 6 | Packing- [Enter Description] |
| 7 | Labeling- [Enter Description] |
| 8 | Storage- [Enter Description] |
| 9 | Distribution - [Insert Description] |

  - Hazard Analysis & Preventive Controls

    - Hazard Analysis

      - Identification of all potential hazards associated with the product at all stages, from raw materials to consumption. These lists can be used for assistance in the identification of potential hazards.

        - Biological – hazards that can cause adverse health impact

        - Chemical – hazards that can cause both health and physical harm

        - Physical – hazards can cause choking and other health issues to the consumer

        - Allergens – hazards can cause an allergic reaction to an individual to certain foods

        - Intentional Contamination / Intentional Adulteration – a hazard that a person or group of people contaminates a food business that causes significant public health consequences, creates widespread public fear, disrupts trade, and increases food insecurity.

        - Economic Fraud – it is a type of food fraud in which a person has added a substance to food to enhance its apparent value or profitability

    - Hazard Evaluation

      - Risk Assessment

        - Assess if a hazard was to occur, what would the severity of the illness or injury be. Risk assessment models are used to assist in determining which hazards are significant if no preventive controls are put in place.

        - Refer to Risk Assessment Form

      - Decision Tree

        - This determines the appropriate control. Decision trees comprise a sequence of questions which when answered, determine the appropriate control.

        - Refer to Decision Tree

    - Preventive Control

      - Preventive controls should be established for steps through which a list of hazards will be prevented, eliminated, or reduced. The preventive control chosen for controlling a particular hazard is based on a risk-based approach, and the control must be current with food manufacturing practices.

      - Potential preventive controls used for the hazard control

        - Process Preventive Control – for controlling the microbiological hazard

        - Allergen Preventive Control – for the control of allergens through an allergen management procedure

        - Sanitation Preventive Control – for the sanitary condition adequate to significantly minimize or prevent hazards such as environmental pathogens, biological hazards due to employee handling, and food allergen hazards.

        - Supply-Chain Preventive Control – for controlling the hazards of ingredients when the originating Facility does not have controls in place for those hazards

        - Recall Plan – for the safety net that can prevent consumers from buying or eating a potentially harmful food product.

          - Refer to Process Flow Program

          - Refer to Cleaning and Sanitation Program

          - Refer to Raw Material Receiving Protocol Program

          - Refer to Product Recall Program

  - Monitoring Procedures and Corrective Action

    - Monitoring Procedures

    - The food safety plan must include written procedures for monitoring the preventive controls, including the frequency of monitoring procedures.  Monitoring must occur often enough to ensure that preventive controls are consistently performed.

    - The monitoring procedure should address each of the following:

      - Who is responsible for monitoring the activity?

      - What is to be monitored

      - How it is to be monitored

      - When monitoring will take place, and how often (frequency)

    - Corrective Actions

      - The food safety plan must include written corrective action procedures that must be taken if preventive controls are not correctly implemented.

      - The procedures must ensure that action is taken to identify and correct the problem with the implementation of preventive control, that all affected food is evaluated for food safety, and that all affected food does not enter commerce if the food may be adulterated or misbranded.

      - Refer to Corrective Action and Preventive Action (CAPA) Report

  - Verification and Validation

    - Verification procedures are routine and ongoing to prove that the food safety plan is executed as intended. Monitoring are verified, and decisions about corrective actions.

    - Validation are based on sound scientific knowledge from publications, scientific literature, or other resources and must be performed or overseen by the PCQI.

    - The audit is performed annually to compare the actual practices and procedures of the system.

      - Refer to Internal Audits and Inspections Program

    - Calibration should be documented, and the records should be available for review during verification.

      - Refer to Calibration Program

      - Refer to Calibration Plan

    - Reanalysis are conducted once every 3 years. To update the Food Safety Plan's Preventive control.

      - Refer to Reanalysis Procedure

    - Monitoring and control of identified Critical Control Point (CCP) are documented and records are verified.

  - Frequency

    - Food Safety Plan Review should be conducted annually or as often as necessary to reflect changes.

      - Refer to Food Safety Plan Review

    - Food Safety Meeting Minutes and Management Review-SQF System Review should be reviewed monthly.

      - Refer to Food Safety Meeting Minutes

      - Refer to Management Review-SQF System Review

- Responsibility

  - Senior Site Management

    - Lead and support the establishment, documentation, and communication to all relevant staff of the Food Safety Plan

  - Primary and Substitute SQF Practitioner

    - Oversee the development, implementation, review, and maintenance of the Food Safety Plan

  - Food Safety Team Leader/ PCQI

    - The Food Safety Team Leader manages the organization's Food Safety Management System (FSMS), overseeing food safety processes, audits, corrective actions, and recalls and keeping the organization informed of food safety issues.

      - Refer to Job Description

  - Food Safety Member

    - Food Safety Team Members will assist the Food Safety Team Leader with the day-to-day requirements of the Food Safety Department.  The Member will work mainly in the production room verifying quality control, monitoring that food safety procedures and documentation are being followed, problem solving as relates to food safety issues, and assisting with compliance of HACCP, GMP, and SQF.

      - Refer to Job Description

- Corrective Action

  - If hazards are introduced into the process or products,

    - Train the key personnel involved in the implementation of the program,

    - Review the hazard analysis to verify that all potential hazards are identified, and control measures are in place

    - Re-validate the critical limits

    - Review the procedures for effectiveness and revise if needed.

  - When changes occur in the process or new products are introduced:

    - Update and re-validate the HACCP/Food Safety Plan to reflect the changes.

- Review-Quality Annual

  - The HACCP team is in place, includes the expertise of the subject process, and members are trained in HACCP principles;

  - The HACCP team has been fully involved in the development and review of the HACCP system;

  - The product and process scope of the HACCP plan is defined;

  - Product descriptions are available and include relevant safety information;

  - The intended use of the product is clearly defined;

  - A process flow diagram has been developed and includes all process steps, inputs, and outputs. It has been confirmed by the HACCP team;

  - Potential hazards have been identified for all process steps, and a hazard analysis was conducted using a consistent and valid method;

  - Control measures are in place for all identified hazards, and procedures/work instructions are effectively implemented;

  - CCPs are correctly identified using a valid methodology;

  - Critical limits are in place for every CCP and are validated to ensure consistent product safety;

  - All critical limits are monitored and test procedures, responsibilities, and frequency are fully documented and implemented;

  - Corrective actions are documented, are unambiguous, and determine the disposition of the product and action required to prevent recurrence (refer to 2.5.3);

  - The corrective action procedure has been followed when monitoring shows deviation from critical limits (refer to 2.5.3);

  - Staff with responsibility for monitoring, validation, verification of critical limits, or any other food safety control measures are aware of their responsibility, trained, and are carrying out their functions correctly;

  - The SQF practitioner ensures that the Food Safety Plan is effectively developed, implemented, maintained, and verified (refer to 2.1.2.4 i).

- History

| Revision No.: | Revision Date: | Description of Change: | Originator / Author Name: | Title / Department: |
| --- | --- | --- | --- | --- |
| 0 | YYYYMMDD | Original | [ENTER AUTHOR NAME] | [ENTER TITLE/ENTER DEPARTMENT] |

Requirements:

- The trainee (employee/visitor) has read or received a verbal translation of all or part of the policy, procedure, method, and/or SOP for which they are being trained.

- The trainee has observed a demonstration of the task they are to perform or the procedure for which they are being trained as required.

- The trainee has demonstrated the ability to perform the task with acceptable proficiency and minimal supervision.

| TRAINEE (Print Name): | TRAINEE (Signature): | Date: | Department: |
| --- | --- | --- | --- |
| Trainer / Supervisor (Print Name): | Trainer (Signature): | Date: | Department: |

*Should you need assistance in customizing this document please email csuccess@consultareinc.com

| NAME (Print): | Date: |
| --- | --- |

Comprehension Quiz

Direction: Choose the best answer and encircle the letter.

- _______ models determine which hazards are significant if no preventive controls are implemented.

- Control of the Risk

- Risk Management

- Assessment of the Risk

- ______ hazards can cause an allergic reaction to an individual to certain foods.

  - Allergens

  - Economic Fraud

  - Intentional Contamination / Intentional Adulteration

- A _______ includes expertise in the process, product, and food safety.

  - Multidisciplinary food safety team

  - SQF

  - PCQI

- What team should draw a process flow diagram showing each step-in operation? It generally represents all the steps raw materials go through to become a finished product.

    - HACCP Team

    - Food Safety Team

    - SQF Team

- A hazard that a person or group of people contaminates a food business that causes significant public health consequences, creates widespread public fear, disrupts trade, and increases food insecurity.

- Allergens

- Economic Fraud

- Intentional Contamination / Intentional Adulteration

*Should you need assistance in customizing this document please email csuccess@consultareinc.com

Comprehension Quiz: ANSWER KEY

- _______ models determine which hazards are significant if no preventive controls are implemented.

- Control of the Risk

- Risk Management

- Assessment of the Risk

- ______ hazards can cause an allergic reaction to an individual to certain foods.

- Allergens

- Economic Fraud

- Intentional Contamination / Intentional Adulteration

- A _______ includes expertise in the process, product, and food safety.

  - Multidisciplinary food safety team

  - SQF

  - PCQI

- What team should draw a process flow diagram showing each step-in operation? It generally represents all the steps raw materials go through to become a finished product.

    - HACCP Team

    - Food Safety Team

    - SQF Team

- A hazard that a person or group of people contaminates a food business that causes significant public health consequences, creates widespread public fear, disrupts trade, and increases food insecurity.

- Allergens

- Economic Fraud

- Intentional Contamination / Intentional Adulteration