Lighting and Light Fittings Lesson

- Policy

  - Adequate light intensity is provided for processing operations, cleaning, and inspection tasks.

  - Light fittings in food processing and handling areas are fitted with protective covers or shatterproof lights.

- Procedure

  - Lightings and light fittings

    - Provide adequate lighting in hand-washing areas, dressing and locker rooms, and toilet rooms and in all areas where food is examined, processed, or stored and where equipment or utensils are cleaned

      - Ensure that lighting provides minimum lux (foot candle) intensity as prescribed by applicable legislation or, in their absence, meet Good Manufacturing Best Practices appropriate to the commodity being processed. In general, processing and food handling areas are illuminated at a minimum of 200 lux (18.58 ft.c.). Inspection areas require higher illumination; 500 lux (46.45 ft.c.) is generally recommended. At least 540 lux (50 ft.c.) at a surface where a food employee is working with food or utensils or equipment such as knives, slicers, grinders, or saws where employee safety is a factor.

    - Provide safety-type light bulbs, fixtures, skylights, or other glass suspended over exposed food in any step of preparation or otherwise protect against food contamination in case of glass breakage.

      - Install shatterproof lights or light fittings with protective covers in food processing and handling areas.

      - Keep documentation on file, including specifications from the manufacturer with a description of the product.

      - Recess the light into the ceiling, where possible, or have it fits flush to the ceiling. Where light fittings cannot be recessed, protect from accidental damage.

      - When light fittings are suspended from cables, the top of the fitting is sloped at an angle that permits easy cleaning.

      - Include exposed light fittings in the cleaning and sanitation schedule.

- Responsibility

  - Preventative Control Qualified Individual (PCQI)

    - Visually inspect areas for compliance with this policy.

    - Oversee (Verify) compliance of procedure.

    - Ensure personnel is trained on this policy and procedures.

    - Monitor and take corrective action to ensure compliance with this policy.

  - Supervisors or department heads and team leaders are responsible to;

    - Ensure (inspect) and verify this policy and procedure is adhered to and;

    - Take corrective action to ensure compliance with this policy.

  - Quality Department

    - Validate training and adherence to policy and procedure.

    - Monitor and document non-conformance observations, also known as discrepancy, using Corrective Action Preventive Action-CAPA Report.

    - Document control, records, and archive.

  - Maintenance

    - Ensures compliance with the required standards when installing light fixtures.

- Corrective Action

  - Train all personnel on this policy and reference SSOPs

  - Non-compliance will require re-training and performance evaluation.

- Review – Quality

  - Annual review of compliance through observation of the following:

    - Lighting intensity is sufficient in food processing areas;

    - Lighting intensity is sufficient at inspection stations;

    - Lighting intensity is sufficient in warehousing and storage areas;

    - Light fixtures are shatterproof or protected and pose no threat to product safety;

    - Light fittings are intact – there is no sign of breakage;

    - Light fittings are clean and part of a regular cleaning regime.

- History

| Revision No.: | Revision Date: | Description of Change: | Originator / Author Name: | Title / Department: |
| --- | --- | --- | --- | --- |
| 0 | 20221121 | Original | Arnel Ryan | PCQI / Compliance |

Requirements:

- The trainee (employee/visitor) has read or received a verbal translation of all or part of the policy, procedure, method, and/or SOP for which they are being trained.

- The trainee has observed a demonstration of the task they are to perform or the procedure for which they are being trained as required.

- The trainee has demonstrated the ability to perform the task with acceptable proficiency and minimal supervision.