Ventilation – General Industry Training Material and Comprehension Quiz

- Policy

  - Romero’s Food Products Inc. ensures proper operation ventilation standards, including abrasive blasting, grinding, polishing, buffing, and spray finishing promoting safe working conditions and proper environmental control.

  - Local exhaust ventilation systems are designed to prevent dispersion into the air of dust, fumes, mists, vapors, and gases in concentrations causing harmful exposure. Such exhaust systems are designed so that dust, fumes, mists, vapors, or gases are not drawn through employees' work areas.

- Procedure

  - Abrasive Blasting

    - Abrasive blasting operations forcibly apply abrasive materials to surfaces using pneumatic pressure, hydraulic pressure, or centrifugal force.

    - Abrasive blasting hazards

      - Dust hazard - Abrasives and surface coatings on materials are shattered and pulverized during blasting operations. The dust formed will contain particles that could result in respiratory, fire, and/or explosion hazards.

      - Noise hazard - Abrasive blasting operations can be loud and expose operators to potentially hazardous noise levels.

    - Personal Protective Equipment

      - Use only respirators approved by the National Institute for Occupational Safety and Health (NIOSH) under 42 CFR 84.

      - All operators must wear abrasive-blasting respirators;

      - Use dust-filter respirators approved by NIOSH under 42 CFR 84 for short, intermittent, or occasional dust exposures;

      - Equip operators with heavy canvas or leather gloves and aprons or equivalent protection;

      - Comply with the requirements specified by 29 CFR 1910.136 (b) (1) for protective foot wares;

      - Conform to 1910.133 for eye and face protection.

      - Conform to 1910.135 for head protection.

    - Blast-cleaning exposures

      - Ensure exhaust ventilation for blast-cleaning enclosures by maintaining continuous inward airflow in all openings during blasting operations.

    - Exhaust ventilation system

      - Repair dust leaks as soon as possible.

      - Check static pressure drop at exhaust ducts upon installation and periodically thereafter to ensure satisfactory operation.

    - Operational procedures and general safety.

      - Ensure aisles and walkways are clear of dust, dust spills, steel shots, or similar abrasives.

      - Check the use of barriers and curtain walls to isolate the blasting operation from other workers.

  - Grinding, Polishing, and Buffing Operations

    - Provide and use a local exhaust ventilation system to maintain employee exposures within the prescribed limits.

    - Use exhaust hood enclosures and systems to remove dust, dirt, fumes, and gases generated by grinding, polishing, or buffing ferrous and nonferrous metals.

    - Use hoods connected to exhaust systems to catch dust or dirt particles. Hoods are in the direction of the airflow.

    - Design and test all exhaust systems for grinding, polishing, and buffing operations in accordance with American Standard Fundamentals Governing the Design and Operation of Local Exhaust Systems, Z9.2-1960, accordingly.

    - It is the dual function of grinding and abrasive cutting-off wheel hoods to protect the operator from the hazards of bursting wheels and provide a means for removing dust and dirt generated.

  - Spray Finishing Operations

    - Use spray booths or spray rooms to enclose all spray finishing operations.

    - Design and construction of spray booths

      - No combustible materials are used in the construction of a spray booth and supply or the exhaust duct

      - Keep walkways unobstructed.

      - Maintain clear work location in the booth to a booth exit or open booth front.

      - Place overspray filters in locations easily accessible for inspection, cleaning, and replacement and in accordance with 1910.107 (b)(5).

      - Make water chamber closures 18 gages or heavier and adequately protected against corrosion for wet or water-wash spray booths.

      - Provide adequate means of scrubbing for chambers to remove particulate matter from the exhaust stream.

      - Weld steel construction or non-combustible materials for collecting tanks.

      - Construct tanks that discourage the accumulation of hazardous deposits.

      - Make the size of pump manifolds, risers, and headers adequate to ensure sufficient water flow.

    - Design and construction of spray rooms

      - Construct masonry, concrete or non-combustible materials for spray rooms and floors.

      - Build non-combustible fire doors and shutters.

      - Maintain an atmosphere in the breathing zone of spray rooms.

      - Conform to the requirements of spray booths for production spray-finishing operations.

    - Ventilation

      - Provide ventilation in accordance with provisions of 1910.107(d).

      - Provide sufficient strength and rigidity when using a fan plenum to equalize or control the distribution of exhaust air movement through the booth

      - Use non-combustible materials to make inlets or supply ductwork to transport makeup air to spray booths or surrounding areas.

      - Support exhaust ductwork throughout its length to sustain weight, normal accumulation, and negative pressure. Follow American National Standard Z9.2-1960 for proper elements of design.

      - Fireproof roof and wall where ductwork passes through with automatic fire dampers.

      - Do not connect ductwork used for ventilating to any ducts ventilating any other process, chimney, or flue used for conveying any combustion products.

    - Velocity and airflow requirements

      - An adequate air replacement system is one that introduces replacement air upstream or above the object being sprayed.

      - For the spray booth to have an adequate air replacement system, the velocity of air into all openings of a spray booth is not less than that specified in Table G-10 for the operating conditions specified. Refer to Ventilation - Spray Finishing Operations Procedure

    - Makeup air

      - Supply clean, fresh air, free of contamination from adjacent industrial exhaust systems, chimneys, stacks, or vents to a spray booth or room.

      - Keep self-closing doors, dampers, or louvers of the spraying booth or room fully open to receive makeup air.

      - Require cleaning or replacement of filters of air supply to spray booth and room.

- Responsibility

  - Management

    - Prevents employees from hazards associated with their work environment.

    - Provides personal protective equipment at no cost to the employee.

    - Provides adequate and operational ventilation systems.

    - Follows the suitable design of the location and equipment and is constructed based on regulatory standards.

  - Supervisors

    - Ensures all personnel wear personal protective equipment.

    - Routinely inspects ventilation systems and ensures it is installed and fully operational.

    - Ensures that employees perform proper housekeeping before, during, and after operations.

    - Ensures that employees are trained in the use of ventilation systems and the signs of system failure.

  - Employees

    - Wears proper personal protective equipment.

    - Immediately report incidents of dust leaks to the supervisor, if any.

    - Always remove accumulated dust in the workplace.

    - Attends re-training in case a system failure occurs.

- Corrective Action

  - In the event that a system failure occurs stop the operations and perform preventive maintenance. Do not resume operations until the repair is completed.

  - Re-train personnel when deficiencies in the employee's understanding of the program are detected and reflected in their performance.

- Review - Quality

  - Routinary inspection of ventilation systems.

  - Complete and efficient PPEs are provided.

  - Training for current and new employees is performed.

  - Review annually and update this program whenever operations change and are applicable to the facility.

- History

| Revision No.: | Revision Date: | Description of Change: | Originator / Author Name: | Title / Department: |
| --- | --- | --- | --- | --- |
| 0 | 20220903 | Original | Arnel Ryan | PCQI / Compliance |
| 1 | 20241213 | - Aligned with Program - Changed title part from Lesson to Training Material and Comprehension Quiz - Changed email from etrr@consultareinc.com to rfp@consultareinc.com - Added title to Comprehension Quiz - Added answer key | Arnel Ryan | PCQI / Compliance |

Requirements:

- The trainee has read or received a verbal translation of all or part of the policy, procedure, method, and/or SOP for which they are being trained.

- The trainee has demonstrated the task they are to perform or the procedure for which they are being trained as required.

- The trainee has demonstrated the ability to perform the task with acceptable proficiency and minimal supervision.

| TRAINEE (Print Name): | TRAINEE (Signature): | Date: | Department: |
| --- | --- | --- | --- |
| Trainer / Supervisor (Print Name): | Trainer (Signature): | Date: | Department: |

UPON COMPLETION – EMAIL TO rfp@consultareinc.com

| NAME (Print): | Date: |
| --- | --- |

# Ventilation – General Industry Comprehension Quiz

Direction: Choose the best answer and encircle the letter.

- ___________ is abrasive blasting operations can be loud and expose operators to potentially hazardous noise levels.

  - Dust Hazard

  - Noise Hazard

- Who will ensure that all personnel wear personal protective equipment?

  - Management

  - Employer

  - Supervisors

- __________ support exhaust ductwork adequately throughout its length to sustain its weight, normal accumulation, and negative pressure.

  - Ventilation

  - Exhaust Ventilation System

  - Blast Cleaning Exposure

- Romero’s Food Products Inc. ensures proper operation ventilation standards, including abrasive blasting, grinding, polishing, buffing, and spray finishing promoting safe working conditions and proper environmental control.

  - True

  - False

- Who is the responsible person for providing adequate and operational ventilation systems?

  - Supervisor

  - Management

  - Employee

UPON COMPLETION – EMAIL TO rfp@consultareinc.com