Receipt, Storage, and Handling of Goods Lesson

- Policy

  - The storage plan is documented and implemented.

  - There is a control of stock to manage product safety, shelf life, and codes based on risk, customer specifications, conditions of the product, storage locations, and inventory management.

  - Materials used in the construction of storage rooms comply with Building Materials and light fittings in storage areas requirements.

  - There is an established document and implementation of the stock rotation procedures.

  - Equipment storage rooms that are adjacent to equipment cleaning areas are kept separate to ensure there is no commingling of dirty and cleaned tools, utensils, and equipment.

  - Racks are provided to ensure tools and equipment are not stored on the floor.

  - Where temporary or overflow storage is used, a risk analysis is undertaken to ensure the stored product is not at risk or poses a risk to products, processes, or personnel. The risk analysis is documented and available every time overflow storage is applied.

    - Dry ingredients and packaging materials are held in a dry, clean area that is free from pests.

    - Chemicals are stored in a safe secure area that complies with relevant regulations and does not pose a risk to personnel or other products.

- Procedures

  - Storage Plan

    - Addresses how food products, materials, and recoups do not get lost, damaged, or contaminated in the process and, along with stored equipment and chemicals, are safely identified, utilized, and rotated stored to maintain the safety, and integrity of the item.

    - Storage racks are maintained unobstructed, and delineated, with 18-inch spacing between walls and stored items for cleaning and inspection.

      - Ice

        - Ice rooms and receptacles are constructed of materials as outlined in elements of section 6 and designed to minimize contamination of the ice during storage and distribution.

        - Microbiological analysis of the water and ice supply that is in contact with food or food contact surfaces is conducted to verify the cleanliness of the supply, the monitoring activities, and the effectiveness of the treatment measures implemented.

      - Food Products

        - In warehouses where food products are recouped or exposed, product contact surfaces are constructed of materials that do not contribute to a food safety risk.

        - Food products are stored in storage racks and pallets. Small amounts of excess products are stored in cabinets and recycled but safe containers. Food products are not stored directly on the floor and are easily movable by pallet jacks, forklifts, and other similar devices.

        - All stored food products are shrink-wrapped properly to prevent exposure.

      - Packaging Materials

        - Kept in appropriate containers as required and off the floor.

        - Stored separately from food products.

      - Equipment Storage

        - Rooms are adjacent to equipment cleaning areas but kept separate to ensure there is no commingling of dirty and cleaned tools, utensils, and equipment.

        - All nonfood items including equipment are stored separately from food products.

      - Chemical Storage

        - Hazardous chemicals and toxic substances with the potential for pet food contamination are stored so as not to present a hazard to staff, product, packaging, product handling equipment, or areas in which the product is handled, stored, or transported.

        - Chemicals are stored separately and comply with the storage requirements specified in Material Safety Data Sheet.

  - Inventory Management

    - Stock Control

      - Necessary inventories are established and maintained at reasonable levels, consistent with CFR requirements, applicable laws and regulations, and the following objectives:

        - The maintenance of adequate stock levels through accurate analyses of quantities to determine requirements and stock replenishment so that only minimal obsolescence losses are encountered while ensuring adequate inventory levels to meet program schedules.

        - The protection of materials against misuse, theft, and misappropriation.

        - The maintenance of an efficient operation.

        - The standardization of inventories to the greatest extent practicable.

      - Stock controls are maintained based on stock record accounts of inventories on hand, on order, received, issued, and disposed of, and supported by proper documents in evidence of these transactions. Stock record accounts are available for review and inspection.

    - Stock Rotation

      - Stock is controlled to ensure materials, ingredients, work in progress, and the finished product does not get lost in the process and exceeds their shelf life.

      - The stock rotation procedures are reviewed initially at the desk audit and compliance to this requirement by observation, interview, and review of records at each facility audit.

  - Materials and Light Fittings

    - Materials used in the construction of storage rooms

      - Buildings that contain storage and distribution operations, specifically the floors, partitions, doors, and ceilings are designed and constructed in such a way as to minimize the risk to product safety and in some instances to offer protection to the product. The extent to which these elements are relevant depends on the type of processes housed and whether the product is enclosed or exposed.

    - Lighting and Light Fittings

      - Light fittings in areas where the food product is recouped or exposed are shatterproof, manufactured with a shatterproof covering or fitted with protective covers, and recessed into or fitted flush with the ceiling.

      - Light fixtures are accounted for in the glass and brittle plastic program to prevent contamination.

  - Racks Storage

    - Tools and equipment are not stored on the floor.

    - Pallets used are free from any signs of damage. If wooden pallets are allowed, they are fumigated and with a liner, not directly in contact with the product.

    - Stored items are secured. The regulation does not specifically address racking; rather, it is a general stacking requirement, to keep materials from falling over or collapsing.

    - The height of any storage materials in relation to fire sprinklers. There are 18 inches between the top of the racked materials and the sprinklers.

      - Issues

        - Rack columns not anchored to the floor OSHA recommends that the bottom of all columns be furnished with column base plates and be anchored to the floor with anchor bolts capable of resisting the forces caused by the loads on the rack.

        - Load ratings are not present on racking. OSHA recommends that load ratings be present which provide the maximum permissible unit load and/or maximum uniformly distributed load per level, the average unit load, and the maximum total load per bay. See ANSI/RMI MH16.1 for additional information.

        - Damaged racking OSHA recommends that the employer develop a maintenance and inspection program for storage racks. The program includes keeping aisles clear and providing sufficient clearance for material handling equipment. Also, ensure racks are properly aligned, plumb, and level, per the manufacturer’s instructions. Employees promptly report any damage to racks.

  - Risk Analysis

    - Where temporary or overflow storage is used, a risk analysis is undertaken to ensure the stored product is not at risk or poses a risk to products, processes, or personnel. The risk analysis is documented and available every time overflow storage is applied. In particular:

      - Materials and products are held in a dry, clean area that is free from pests.

      - Chemicals are stored in a safe secure area that complies with relevant regulations and does not pose a risk to personnel or other products.

  - Receiving

    - All incoming materials are from an approved supplier.

    - Pre-unloading Activities: Upon the arrival of the vehicle of material at the unloading area, the following activities are carried out:

      - Ensure that the vehicle is covered properly to protect the material.

      - Instruct the vehicle driver to stop the engine to avoid carbon spillage from the silencer until completion of unloading and transfer of material into the Material Receiving Area.

      - Ensure cleanliness of vehicle. Ensure that the vehicle is free from foul odor and pests.

      - Warehouse and Quality personnel check the consignment to ensure the absence of material other than mentioned in the delivery documents, to avoid mix-ups.

      - Ensure that the vehicle is not carrying any pesticides or oily material and is free from abnormal odors.

      - Ensure the material receiving area is clean before unloading the material.

      - Ensure the cleanliness of the dedicated pallets available at the receiving platform.

      - Ensure the cleanliness and working condition of the lift and trolleys.

      - Ensure the air curtain is in working condition.

      - Ensure the weighing balance is calibrated.

      - The Warehouse Staff confirms that the received goods, as labeled by the supplier conforms with the Purchase order under which the material was ordered. The purchase order number, product name, description, quantity (items are weighed where appropriate), a unit of measure, product expiration, or shelf-life date are verified. The following receiving documents are checked, if required:

        - Purchase Order (PO)

        - Certificate of analysis (COA)

        - Certificate of Guarantee or Letter of Guarantee

        - Product Specifications

        - Vehicle Sanitary Certificate

        - Certificate of Origin (COO)

        - Receiving chemical agents (COA, SDS)

      - If there is a discrepancy between the information listed on the packing slip and the purchase order information, the goods are not received, and the Warehouse Staff notifies the Purchasing Department for corrective action.

      - After verification of the purchase order information, the Warehouse Staff assigns the lot number and logs the materials in the Inventory.

      - Packaging of good quality and contains the required data and source data. Each container is checked for obvious container damage, and evidence of broken seals, contamination, and pest infestation.

      - The sampling of materials for Quality Control checking and inspection is conducted according to Sampling Plan.

      - Any discrepancies are noted, recorded, and reported to the concerned manager and communicated to the supplier based on a discussion with the Purchasing Department.

      - Materials/packages that have signs of damage or contamination are stored separately subject to QC disposition.

      - Chemicals and toxic substances are stored so as not to present a hazard to staff, product, packaging, product handling equipment, or areas in which the product is handled, stored, or transported.

    - Loading and Unloading

      - Considerations for loading and unloading include:

        - Employees responsible for loading and unloading food materials follow company standards for hygiene and sanitation practices.

        - Food products are packaged in suitable containers to prevent product damage.

        - Proper product temperature is maintained during transport, loading, and unloading. Movers are aware of the product temperature requirements.

        - Ensure transport devices such as pallet jacks, forklifts, and other similar devices are clean, sanitized, and in good condition before use.

        - Ensure the transport container van is clean, dry, and thoroughly inspected before loading products for deliveries.

    - Documentation

      - Document specific information when receiving any ingredients/packaging materials. This is important for tracing shipments in case of a recall and includes:

        - Date of receipt

        - Type and Name of product

        - Ingredient and product packaging

        - Labeling

        - Lot Number

        - Supplier

        - Pallet tag, quantity, size, and weight

        - Best before date

      - Stock record logs are maintained and regularly updated.

  - Handling

    - Food products are handled according to Good Manufacturing Practices (GMP).

    - Handling procedures include controlled temperature and humidity (if necessary), personal hygiene, etc. This can protect food from contamination by pests, and microbiological, physical, or chemical hazards, and prevent deterioration or spoilage.

    - Issuance and withdrawal of materials are controlled by the Warehouse and Inventory department.

  - Allergen control precautions for raw materials, purchasing, transportation, and storage.

    - Ensure suppliers have documented and implemented an allergen control plan.

    - Check labels on incoming ingredients to ensure the supplier has not sent the wrong product, a substitute product, or used the wrong label.

    - Ensure vehicles and shipping containers are cleaned before shipping.

    - Clearly label raw materials to indicate they contain food allergens (e. color-coded containers, tags).

- Responsibility

  - All Personnel

    - Shall adhere to this policy.

  - Preventative Control Qualified Individual (PCQI)

    - Visually inspect areas for compliance with this policy.

    - Oversee (Verify) compliance of procedure.

    - Ensure personnel are trained in this policy and procedures.

    - Monitor and take corrective action to ensure compliance with this policy.

  - Supervisors or Department Heads and Team Leaders

    - Ensure (inspect) and verify this policy and procedure is adhered to.

    - Take corrective action to ensure compliance with this policy.

  - Warehouse Personnel

    - Ensure goods are received, stored, and labeled according to this policy.

    - Authorized personnel to receive incoming deliveries of materials and issue materials to the end user.

  - Quality Control Personnel

    - Validate training and adherence to policy and procedure.

    - Monitor and document non-conformance observation also known as discrepancy using Corrective Action Preventive Action-CAPA Report.

  - Quality/Compliance Department

    - Document control, records, and archive.

- Corrective Action

  - In the event of product, materials, or equipment contamination due to Receipt, Storage, and Handling of Goods:

    - Train the key personnel involved in the implementation of the program,

    - Review the procedures for effectiveness and revise if needed.

- Review-Quality Department

  - Annual review of compliance to Receipt, Storage, and Handling of Goods Program requirements by observing the following:

    - Storage plan is documented, implemented, and effective.

    - Review of stock rotation procedure.

    - Review of stock records.

    - Understanding of personnel responsible for inventory management.

    - Visual confirmation of raw material, ingredient, packaging, work in progress, and finished product stock in storage.

    - There is a dedicated storage area for clean tools, utensils, and equipment.

    - The equipment storage area allows access for cleaning.

    - The equipment storage area protects equipment during storage.

    - Alternative storage is being used.

    - Risk analysis has been conducted for alternate storage.

    - Materials or products are not being stored continuously in temporary storage.

    - There is no risk of product contamination from the use of temporary storage.

    - Records validate the safe alternate or temporary storage control measures.

- History

| Revision No.: | Revision Date: | Description of Change: | Originator / Author Name: | Title / Department: |
| --- | --- | --- | --- | --- |
| 0 | YYYYMMDD | Original | ENTER AUTHOR NAME | ENTER TITLE / ENTER DEPARTMENT |

Requirements:

- The trainee (employee/visitor) has read or received a verbal translation of all or part of the policy, procedure, method, and/or SOP for which they are being trained.

- The trainee has observed a demonstration of the task they are to perform or the procedure for which they are being trained as required.

- The trainee has demonstrated the ability to perform the task with acceptable proficiency and minimal supervision.

| TRAINEE (Print Name): | TRAINEE (Signature): | Date: | Department: |
| --- | --- | --- | --- |
| Trainer / Supervisor (Print Name): | Trainer (Signature): | Date: | Department: |

*Should you need assistance in customizing this document please email services@consultareinc.com