Storage of Dry Ingredients, Packaging, and Shelf Stable Packaged Goods Lesson

- Policy

  - Ingredients, raw materials, packaging, and packing materials are stored in designated storage areas which protect the materials from contamination and deterioration. These materials are stored only in dry areas of the processing room when staged for use during processing or packing.

  - Packaging storage areas are adequately protected from the elements, rodents, and other pests.

  - Packaging materials that become food contact surfaces are protected from dust and other contaminants while in storage. Packaging materials are properly covered in the dry goods warehouse and the packaging room.

  - Suppliers are aware of the need to segregate identity-preserved products, particularly materials, and products containing allergens. These materials may require separate, dedicated storage rooms.

  - Materials used to construct storage rooms and light fittings in storage areas must comply with 21 CFR 117.20.

  - The racks provided for the storage of packaging are constructed of impervious materials and designed to be easy to clean.

  - The use of wooden racks for storage of packaging and packing materials is limited to dry areas only.

  - Stands and the lower shelves of stands are at least 150 mm (6 inches), or as required by applicable regulations above floor level, to facilitate proper cleaning.

  - Establish procedures for outside storage warehouse facilities.

- Procedures

  - General Procedure

    - Dry ingredients, packaging, and shelf-stable packaged goods storage area are located away from wet processing areas. It is designed and constructed to adequately protect products, ingredients materials, and packaging materials.

    - The storage area is designed to facilitate stock rotation.

    - Storage racks and shelving are made up of non-corrosive material and are easily cleanable. Racks allow access to floor/wall junctions for cleaning. Plastic pallets in the coolers and freezers are on the floor to prevent floor exposure.

    - Dry ingredients, packaging, and shelf-stable packaged goods storage areas are kept clean and well maintained.

    - The storage area is kept dry, cool, well-lit, ventilated, and vermin-proof.

    - Dry ingredients, packaging, and shelf-stable packaged goods are protected from exposure to moisture and direct sunlight.

    - The dry ingredients, packaging, and shelf-stable packaged goods storage areas are appropriately labeled, and different food groups are stored separately.

    - Dry ingredients, packaging, and shelf-stable packaged goods are stored well away from chemicals and cleaning equipment. Smocks and aprons are not placed on boxes in the warehouse, coolers, or freezers.

    - Dry ingredients, packaging, and shelf-stable packaged goods are stored such that cross-contamination from physical, chemical, or biological contaminants is prevented. Work-in-process materials are covered properly to prevent the chance of contamination.  This includes during chilled storage and transfer between plants.

    - Dry ingredients, packaging, and shelf-stable packaged goods are stored off the floor, shelving, or racks to prevent contamination and allow adequate air circulation around the product. The materials are stored a minimum of 6 inches from the floor. Do not store foods directly on the floor. An 18-inch clear area against all walls is maintained for pest control surveillance and cleaning purposes.

    - Packaged foods that have been opened are stored in clean, non-toxic, food-grade, pest-proof containers with tight-fitting lids or are wrapped in protective packaging (such as foil, plastic food wrap, grease-proof paper, etc.). The container is properly labeled and dated.

    - When receiving dry ingredients or packaged goods, the packaging is checked for integrity (intact and undamaged), and the raw materials are checked to if within their use-by-date. Dry ingredients or packaged goods that are out of date or damaged packaging are returned to the supplier or disposed of properly.

    - Dry ingredients, packaging, and shelf-stable packaged goods are labeled with the date of receipt prior to storage for traceability purposes and then rotated on a first in, first out, or first expiry, first out basis. Labels are faced front and readable for FIFO rotation purposes.

    - Dry ingredients, packaging, and shelf-stable packaged goods are stored according to the manufacturer’s instructions.

    - Food spills are cleaned up promptly to minimize the attraction of pests and to reduce the risk of contamination. Appropriate utensils are used per the cleaning utensil color coding system.

    - Pests are deterred and eradicated as soon as possible. Signs of pest activity include droppings, eggs, webs, feathers, and smells.

    - Staff adhere to good housekeeping practices and visually inspect products for signs of damage, contamination, or spoilage.

    - All dry ingredients, packaging, and shelf-stable packaged goods are properly labeled.

    - Raw materials containing allergens are identified, properly labeled, and segregated.

    - Forklifts, hand forks, and other vehicles used in storage areas are safe to use, and hydrocarbon emissions are controlled and operated in a manner that does not cause damage to products and equipment.

    - Broken or dirty pallets are not used (pallets are cleaned and repaired when needed). Pallets are stored in areas free of dirt, litter, birds, insects, and rodent contamination. Never stand pallets on end or lean them against walls.

  - Outside Storage Warehouse Facilities

    - Outside warehouse storage facilities are either inspected by COMPANY NAME at least annually or provide a record of third-party inspection.

    - Outside storage warehouse facilities comply with all applicable GWP rules.

    - Relative to the rating system used, satisfactory performance is necessary for the outside warehouse facility to be considered approved.

    - The outside warehouse agrees to share its corrective action response with COMPANY NAME upon request.

    - COMPANY NAME reserves the right to audit the facility if problems exist.

    - A list of approved outside storage warehouse facilities is maintained on file by COMPANY NAME as applicable.

- Responsibility

  - All Personnel

    - Shall adhere to this policy.

  - cGMP Team Lead

    - Visually inspect areas for compliance with this policy.

    - Oversee (Verify) compliance of procedure.

    - Ensure personnel are trained in this policy and procedures.

    - Monitor and take corrective action to ensure compliance with this policy.

  - Supervisors or department heads and team leaders are responsible to;

    - Ensure (inspect) and verify this policy and procedure is adhered to and;

    - Take corrective action to ensure compliance with this policy.

  - Quality Control Personnel.

    - Validate training and adherence to policy and procedure.

    - Monitor and document non-conformance observation, also known as discrepancy, using Corrective Action Preventive Action-CAPA Report.

  - Quality / Compliance Department.

    - Document control, records, and archive

- Corrective Action

  - In the event of product, materials, or equipment contamination due to Storage of Dry Ingredients, Packaging, and Shelf Stable Packaged Goods:

    - Train the key personnel involved in the implementation of the program,

    - Review the procedures for effectiveness and revise if needed.

- Review - Quality

  - Annual review of compliance to Storage of Dry Ingredients, Packaging, and Shelf Stable Packaged Goods requirements by observing the following:

    - Product storage rooms are located away from wet processing areas.

    - Storage rooms are adequately designed to protect products, ingredients, materials, or packaging materials.

    - Packaging racks are made of material that is easily cleanable.

    - Packaging racks allow access to floor/wall junctions for cleaning.

    - Vehicles used in food processing, storage, or cold storage areas release hydrocarbon emissions or present a hazard to the food product, ingredients, materials, or packaging materials.

- History

| Revision No.: | Revision Date: | Description of Change: | Originator / Author Name: | Title / Department: |
| --- | --- | --- | --- | --- |
| 0 | YYYYMMDD | Original | ENTER AUTHOR NAME | ENTER TITLE / ENTER DEPARTMENT |

Requirements:

- The trainee (employee/visitor) has read or received a verbal translation of all or part of the policy, procedure, method, and/or SOP for which they are being trained.

- The trainee has observed a demonstration of the task they are to perform or the procedure for which they are being trained as required.

- The trainee has demonstrated the ability to perform the task with acceptable proficiency and minimal supervision.

| TRAINEE (Print Name): | TRAINEE (Signature): | Date: | Department: |
| --- | --- | --- | --- |
| Trainer / Supervisor (Print Name): | Trainer (Signature): | Date: | Department: |

*Should you need assistance in customizing this document please email services@consultareinc.com