Handwashing Lesson

- Policy

  - In all food contact packaging manufacturing areas(s), employees, contractors, and visitors have clean hands upon entering packaging manufacturing and handling areas:

    - after each visit to a toilet

    - after using a handkerchief

    - after smoking, eating, or drinking

    - after handling wash-down hoses

    - after handling dropped product

    - after handling contaminated materials

  - Hand wash stations are correctly equipped and available at convenient locations for use.

- Procedure

  - Hand Wash Basin

    - Handwash basins are provided near pedestrian entry points at each site area, with instructions for all staff, contractors, and visitors to wash their hands immediately before entering the manufacturing area.

    - Additional hand basins are required where hands could become contaminated before working with the product.

    - Potable water at a suitable temperature, liquid soap, single-use paper towels, and a means of disposing of used paper towels are provided at each station.

    - Hands-free operated taps and hand sanitizers are also required for high-risk operations.

    - Hands-free operated taps can include foot, knee, or elbow operated handles, auto-sensing devices, or any other method that does not require the user to touch the handle with their washed hands to turn it off.

    - The following handwashing procedures are followed using a hands-free handwashing station:

      - By using foot, knee, or elbow operated handles or auto-sensing devices, turn on the water tap (cold and hot) to the desired temperature (USDA: At least 110oF or 43oC)

      - Preload or Pre-dispense single-use towel or paper towel.

      - Handwashing Procedure- Wet hands thoroughly, apply enough hand soap to cover all hand surfaces, rub hands palm to palm, rub right palm over left-back of the left hand with interlaced fingers and vice versa, rub back of fingers to opposing palms with fingers interlocked, do a rotational rubbing of left thumb clasped in right palm and vice versa, do rotational rubbing backward and forward with clasped fingers of the right hand in left palm and vice versa.

      - Rinsing procedure – Perform handwashing procedures while applying water to complete the rinse cycle.

      - Hand drying procedure – obtain a preloaded or pre-dispense paper towel or single-use towel and dry hands thoroughly.

      - Dispose of paper towels on the designated waste receptacle.

        - If the water temperature does not meet the minimum requirements of 100°F, apply sanitizer and wear gloves.

    - Hand Washing soaps and scrubs are evaluated to be free of allergens (see also 2.8.1).

    - Where alternative hand-drying methods are preferred (e.g., high-speed air dryers), they must be justified, and their effectiveness validated (refer to 2.4.2.2 or 2.5.1.1).

    - Handwash basins are to be constructed of stainless steel or similar non-corrodible material. Handwash basins constructed of porcelain or similar materials are located at a distance from packaging manufacturing and handling areas.

  - Handwashing Procedure

    - Hands or Gloves are washed or changed before handling any exposed food contact material. The following are some situations:

      - When entering or reentering the production area for any reason.

      - After handling any boxes, boxed products, or pallets.

      - After handling non-food contact materials.

      - After handling pallet jacks, hand trucks, or any unsanitary equipment.

      - Hands are washed before handling gloves.

      - Hand washing always takes place before putting new gloves on.

      - Whenever a hand or finger cut requires a band-aid, plastic/vinyl gloves are worn on the affected hand to prevent food contact material contamination. If the cut bleeds excessively, the employee may be asked to leave work or get out of the production area until the bleeding comes under control.

      - Employees with any bandage also wear a glove in a processing room even if they are not directly handling food contact materials.

      - Gloves are changed or washed after handling trash, boxes, pallets, pallet jacks, hand trucks, or unsanitary equipment.

      - Gloves are changed if torn.

  - Handwash Signage

    - Handwash signage is posted near the station and is translated as needed to reflect the languages spoken by workers.

  - Designation of Handwash Basin

    - Designated and well-stocked handwash basins (hands-free) are located at every regular employee entrance to the processing area,

    - Hand sinks are available inside restroom facilities and are used for that purpose only.

    - Once an employee exits a restroom, they need to re-wash their hands at a designated production area hand-sink before returning to work.

    - Location of the handwash basin is designated where employees can go directly from the hand sink to their workstation without the risk of contamination.

    - The following are the requirements for processing area hand sinks

      - Warm water, liquid soap, and hands-free towels (or other sanitary drying devices).

      - A hands-free waste container.

    - Employees are to wash their hands prior to starting work, after a break, after using the restroom, and any time their hands become contaminated.

- Responsibility

  - All Personnel

    - Shall adhere to this policy.

  - SQF Practitioner

    - Visually inspect areas for compliance with this policy.

    - Oversee (Verify) compliance of procedure.

    - Ensure personnel is trained on this policy and procedures.

    - Monitor and take corrective action to ensure compliance with this policy.

  - Supervisors or department heads and team leaders are responsible to;

    - Ensure (inspect) and verify this policy and procedure is adhered to and;

    - Take corrective action to ensure compliance with this policy.

  - Quality Personnel

    - Validate training and adherence to policy and procedure.

    - Monitor and document non-conformance observations, also known as discrepancy, using the Corrective Action Preventive Action-CAPA Report.

  - Quality / Compliance Department

    - Document control, records, and archive.

- Corrective Action

  - Train all personnel on this policy and reference SSOPs.

  - Non-compliance will require retraining or dismissal from job duties or termination of employment.

- Review - Quality

  - The location and construction of handwash stations and their use by staff, contractors, and visitors shall be reviewed.

  - Handwash basins are available for staff, contractors, and visitors.

  - Handwash basins are located at personnel access points and areas where hands could become contaminated.

  - Handwash basins are constructed of an appropriate material.

  - Handwash basins have potable water supplied at appropriate temperatures.

  - There is liquid soap available at handwash stations.

  - There are paper towels available at handwash stations.

  - There are containers for used paper towels at handwash stations.

  - There is signage near handwash stations instructing people to wash their hands.

  - Personnel in packaging manufacturing and handling areas have clean hands.

  - Personnel washes their hands-on entering manufacturing areas.

  - Personnel washes their hands-on leaving toilet areas.

  - Personnel washes their hands on leaving the lunchroom.

  - Personnel washes their hands after handling products, hoses, or waste.

  - Personnel washes their hands after eating, drinking, or smoking.

  - Personnel who use gloves also follow handwashing requirements.

- History

| Revision No.: | Revision Date: | Description of Change: | Originator / Author Name: | Title / Department: |
| --- | --- | --- | --- | --- |
| 0 | 20230110 | Original | Arnel Ryan | PCQI / Compliance |

Requirements:

- The trainee (employee/visitor) has read or received a verbal translation of all or part of the policy, procedure, method, and/or SOP for which they are being trained.

- The trainee has observed a demonstration of the task they are to perform or the procedure for which they are being trained as required.

- The trainee has demonstrated the ability to perform the task with acceptable proficiency and minimal supervision.