Air and Other Gases Lesson

- Policy

  - The compressed air used in food operations is appropriate, verified, and validated and does not serve as a source of contamination.

  - Preventive maintenance programs are ensured, and an appropriate filtration program is in place at the point of use.

  - Testing is conducted to validate the compressed air-filtration control system’s effectiveness based on the risk to the product.

- Procedure

  - Compressed Air

    - Compressed air can be a source of chemical and microbiological contamination.

    - Potential contaminants can include particulates, including dirt (microorganisms, atmospheric dirt and solid particulates, rust, and pipe scales), water (water vapor, condensed liquid water, and water aerosols), and oil (oil vapor, liquid oil, and oil aerosols).

    - Food-grade oil is used in the air compressor when air comes in contact with the exposed product or direct product contact surfaces.

  - Preventive Maintenance

    - Filters are cleaned or changed at a frequency appropriate to the product and process or following any air supply source or equipment maintenance.

    - Any maintenance is done hygienically.

    - Nozzles and air hoses are in good condition, properly repaired, and maintained hygienically (e.g., cleaned and sanitized).

    - Hoses and nozzles are kept off the ground.

    - Locate the filtration as close as possible (near the “point of use” or the point where air contacts the product) to avoid long lengths of piping/tubing between the microbial removal filter and the air/food contact point.

  - Particulate Controls

    - The following are controls for particulates:

      - Intake filters to remove atmospheric dirt and solid particulates.

      - An effective Preventive Maintenance program should be in place to maintain the integrity of the filter.

      - Water, including vapor, liquid, and condensed.

    - A dryer in the compressed air system provides effective control.

    - The presence of coalescing filters in the compressed air system effectively removes contamination.

  - When compressed air comes in contact with the exposed product or direct product contact surfaces, the air compressor must use food-grade oil.

- Responsibilities

  - Supervisors / Department Heads / Team Leaders

    - Ensure (inspect) and verify this policy and procedure is adhered to and;

    - Take corrective action to ensure compliance with this policy.

  - Quality Personnel

    - Validate training and adherence to policy and procedure.

    - Monitor and document non-conformance observation using the Corrective and Preventive Action Form, also known as a discrepancy.

    - Document control, records, and archive.

  - Maintenance Personnel

    - Ensure compliance with this policy.

  - Performs preventive maintenance of Air and other gases system.

- Corrective Action

  - Review and revise the preventive maintenance procedure.

  - Check and change the filters at a frequency.

  - Maintain cleanliness and properly sanitize nozzles and hoses.

- Review-QA

  - The condition of air compressors and compressed air used to transport product or otherwise to come into contact with exposed product and product contact surfaces;

  - Filters are checked or changed at a frequency based on the air quality program;

  - The maintenance staff has the data specification sheet for the filter housing;

  - Follow up with preventative maintenance and SSOPs;

  - Performance characteristics of the filter in place must match the risks identified in the site’s assessment.

  - Compressed air in contact with food is checked for purity using methods and frequency based on the air quality program and test procedures.

- History

| Revision No.: | Revision Date: | Description of Change: | Originator / Author Name: | Title / Department: |
| --- | --- | --- | --- | --- |
| 0 | 20230111 | Original | Arnel Ryan | PCQI / Compliance |

Requirements:

- The trainee (employee/visitor) has read or received a verbal translation of all or part of the policy, procedure, method, and/or SOP for which they are being trained.

- The trainee has observed a demonstration of the task they are to perform or the procedure for which they are being trained as required.

- The trainee has demonstrated the ability to perform the task with acceptable proficiency and minimal supervision.